T52 BMC Firmware 2.50 Release Note

Content

T52 management board firmware 2.50 release note	2

T52 management board firmware 2.40 release note	3

T52 management board firmware 2.30 release note	4

T52 management board firmware 2.20 release note	5

T52 management board firmware 2.00 release note	7

# T52 management board firmware 2.50 release note

Release Date: 02/19/2003

Firmware Version: 2.50

BMC/B Version Supported: D3C

M/B Version Supported: F, G

BIOS Version Supported: 3F03 or latter

RDM Version Supported: 5.0.2.4

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.50

Fix/Change from v2.40:

- Change the threshold of fan speed to meet hardware request. (rear fan speed is 1620 RPM, front fan speed is 1800 RPM)

- Fixed the bug that event log in RDM show 1970/XX/XX

Known Issues:

  - Do not support PEF/PET function for now.

  - Don’t support CPU reduction function.

  - Only support one session..

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

# T52 management board firmware 2.40 release note

Release Date: 01/28/2003

Firmware Version: 2.40

BMC/B Version Supported: D3C

M/B Version Supported: F, G

BIOS Version Supported: 3F03 or latter

RDM Version Supported: 5.0.2.4

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.10

Fix/Change from v2.30:

- Change front/rear fan lower threshold from 1984/2488 RPM to 1792/1633 RPM

Known Issues:

  - Do not support PEF/PET function for now.

  - Don’t support CPU reduction function.

  - Only support one session..

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

# T52 management board firmware 2.30 release note

Release Date: 12/03/2002

Firmware Version: 2.30

BMC/B Version Supported: D3C

M/B Version Supported: F, G

BIOS Version Supported: 3F03 or latter

RDM Version Supported: 5.0.2.3

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.10

Fix/Change from v2.20:

- Fix the bug that ASM can’t read the chassis asserted event.

- Support “Get Chassis status” command to get chassis state.

- Add event description in the subject of mail.

- Fix the bug that when go into setup utility read event log, the BMC version occasionally disappear if consol redirection is opened(defect id 4206)

- Fix the bug that Watchdog action will be executed at DOS mode(defect id 4197).

- Fix the bug that some wrong message about Voltage Test (defect id 4204)

- Fix the bug that SDR Repository Command test cause BMC can't log EVENT like chassis/Fan..etc. (defect id 4205).

Known Issues:

  - Do not support PEF/PET function for now.

  - Don’t support CPU reduction function.

  - Only support one session..

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

# T52 management board firmware 2.20 release note

Release Date: 11/22/2002

Firmware Version: 2.20

BMC/B Version Supported: D3C

M/B Version Supported: F, G

BIOS Version Supported: 3F03 or latter

RDM Version Supported: 5.0.2.2

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.10

Fix/Change from v2.00:

- Add GetCPUStatus OEM command.

- Fix bug that network broadcast is busy, BMC firmware will reset itself frequency.

- Fix bug that warm boot can’t go out front panel system led when one-shut is happened.

- Fix problem that log event twice when temperature over UNC.

- Change the LCT value(3.225V) of 3.3V and 3.3V standby in SDR

- Fix the bug of Amber LED couldn't turn off if temperature is return to normal

- Turn on HDD LED of front panel direct from the SAFTE low, don’t check HDD LED whether turn on before or not.

- Fix the bug that ICTS test near 100 times the get sdr time will fail twice.

- Fix the bug that KCS interface will fail if heavy scan all sensor event from ASM 5.1 utility.

- Modify function that front panel system led don’t light when chassis open than close during power off, but event log must log.

- Fix the bug that “Get Chassis Status” can not set chassis intrusion activate bit when chassis is opened.

Known Issues:

  - Do not support PEF/PET function for now.

  - Don’t support CPU reduction function.

  - Only support one session..

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

# T52 management board firmware 2.00 release note

Release Date: 10/23/2002

Firmware Version: 2.00

BMC/B Version Supported: D3C

M/B Version Supported: F, G

BIOS Version Supported: 3E01 or latter

RDM Version Supported: 5.0.2.0

BMCIPCFG.EXE Version Support: 1.4

FILET Version: 1.10

Fix/Change from v1.90:

- Front panel system led function is full support ACER requirement as follow:

One-shot item fail led status:

    - CPU IERR --- light.

    - Thermal Trip – light

    - ECC single bit error – blink light

    - ECC multiple bit error – light

    - POST Error (PERR & SERR) – blink light

    - Memory resize – blink light.

    - Intrusion if power off -- light

The front panel system led will go out when system reset, power off, power off.

Return to Normal item : (Fail : light, return to normal: go out)

[1] Back Fan

[2] Front fan 1

[3] Front fan 2

[4] VCC 12V

[5] VCC 5V

[6] VCC 3.3V

[7] VCC 3.3 standby

[8] VCC 2.5

[9] VCC 5V standby

[10] CPU1 Temperature

[11] CPU2 Temperature

[12] System Temperature

[13] Intrusion if power on – blink light ( Intrusion happened)

- Fix bug that DNS ip address is always static ip 192.168.25.5.

- Fix bug that the ip address will wrong in the message of mail.

- Fix bug when AC power is lost, the time is very strange in event log. Change to when AC power is lost and plug in again BMC default time is 1980/01/01 00:00:00

Known Issues:

  - Do not support PEF function for now.

  - Don’t support FRB function.

  - Warm boot can’t go out front panel system led when one-shot is happened.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.