| # BIOS Release Note |  |  |  |
| --- | --- | --- | --- |
| MODEL NO. | T52A | RELEASE DATE | 04/17/2003 |
| BIOS REV. | AMI BIOS 07.00 T52A 3I02 (FILE: T523I02.ROM) | CHECK SUM. | H |
| LAST BIOS REV. | AMI BIOS 07.00 T52A 3H01 (FILE: T523H01.ROM) | LAST RELEASE DATE. | 04/14/2002 |
| *BIOS limit.. 1. BIOS UnLock the Power / Reset button after INT19. 2. RDM console flag need 1 min timeout. T52A 3I01 Release note. (04/17/2003) - D1 stepping Processor Microcode update. Note: This Microcode is a Production Candidate, not a formal Production one. T52A 3I01 Release note. (04/14/2003) - Fix the HCT11.1 “Unreported IO and Memory (Report used resource to OS, thus OS will not use the reported system resource). - Replace “07.00.xx” string in BIOS setup menu with “07.00. - Replace Quanta string with Acer in Bios. - Fix HCT 11.1a --ACPI errors. T52A 3H01 Release note. (01/16/2003) - Fix the APC KVM switch issue. - Change the boot sequence when plugging FlashDisk (Acer Request). - Clear “CTRL-ALT-ESC” string when user press ESC key during Acer Logo is displaying; original Bios blank it only (Acer Request). T52A 3G01 Release note. (12/23/2002) - Fix the boot fail issue from onboard SCSI when plugging LSI SCSI card. - Fix the CIOBX2 issue that may lead Win2K blue screen. T52A 3F05 Release note. (12/13/2002) - Improve the console fail issue when entering Bios setup engine by ESC-key + S-key from client via RDM and BMC. Note: This console issue is still traced by Bios . T52A 3F04 Release note. (12/05/2002) - Add FSB information in Setup Menu (Acer Request). - Additional KB\MS description in System info (Acer Request). - Add “Memory Test” option in Setup menu (Acer Request). - Prevent system shutdown when CPU hot was exerted (The critical temperature value will be handled by BMC). T52A 3F03 Release note. (11/19/2002) - Remove F9 - Barcode Scan Function. T52A 3F02 Release note. (11/14/2002) 1. Read BMC completion code after each BMC command. - Reset century register in super-io chip. T52A 3F01 Release note. (11/11/2002) - Auto-detection of CPU Frequency follow ACER request. - Increase Memory type “Disabled” while Memory reduction active. - CPU reduction for MP system follow ACER request. - Regulate Memory hole size to 32MB/128MB/192MB/256MB granularity. - Show BMC IPMI / ASF version on SetupMenu by BMC report! - Display eventlog time as --:--:-- while BMC time lose! - Write the SVID (1025) / SSID (0026) for ACER request. - Update CPU MAX speed from 2.2G to 2.8G on SMBIOS - Secruity mode unlock PWB/RSB by polling during BIOS POST but not real time check! T52A 3E02 Release note. (10/23/2002) - Fixed MP Table APIC4 didn't use issue. (Linux 7.3 BIOS error message) - Fixed the SMBIOS Type9 (PCI) current usage issue. - Fixed the SMBIOS Type6 / Type17 Memory Bank = Pair (M/B printing) - Support G700 New M/B Dual C1 Stepping work. * Please refresh BIOS by /B and need Load default by SetupMenu. |  |  |  |

| T52A 3E01 Release note. (10/04/2002) - Update the SCSI BIOS to V3.10S1 for 7899 support Ultra320 x 8 - Modify BIOS SetupMenu show product info upto 24Char - Disable Memory Remapping replace by Memory Hole for non-PAE mode - Disable CPU-Reduction support T52A 3D01 Release note. (10/03/2002) 1. Change BIOS Version to 3D01 only. (POST / SMBIOS Type0) T52A 3C06 Release note. (09/18/2002) - Check the AP at socket2 if 0xF27 (C1 stepping) then show notify message. - Update System / BaseBoard information by ACER request. - Update INTEL CPU uCode. (Srv_C_20.pdb – C stepping = 0x28) - Enable the CPU reduction function. T52A 3C05 Release note. (09/03/2002) - Modify Setupmenu GPNV event viewer by ACER require. - Bug fixed for RDM can't pass memory test by press ESC key! - Update SMBIOS information (CPU / Slot) and Clear SYSTEM_VERSION_$ for Acer. - SecruMode notify message by ACER request. - Chang SecruMode hotkey to F10/F11/F12 from ALT-SHIFT-S. - Correct maximum capacity as 6GB in SMBIOS type 16. - Clear BMC RFD flag after System POST end. - Takeout the SecruMode hotkey F10 for ACER spec. - Hidden the CPU reduction function. - Clear BMC RFD flag fixed Win2000 Blue Screen. - Bug fixed for system hang when install 'Free BSD' from CDROM |  |  |  |
| --- | --- | --- | --- |
|  | Akey Tsao | SUPERVISOR | Leo Hsu |