T52A BMC Firmware V1.50 Release Note

Content

T52A management board firmware 1.50 release note	2

T52A management board firmware 1.30 release note	4

T52A management board firmware 1.20 release note	5

T52A management board firmware 1.10 release note	6

T52A management board firmware 1.00 release note	7

T52A management board firmware 0.80 release note	9

T52A management board firmware 0.75 release note	10

T52A management board firmware 0.70 release note	11

# T52A management board firmware 1.50 release note

Release Date: 03/18/2003

Firmware Version: 1.50

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.2.1.7

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.60 (Support G700, G701, and G510)

Change from V1.30:

- Re-initial BMC UART 1 for console redirection at every LPC reset signal asserted. Because LPC reset signal trigger timer active to initial the peripheral of Zircon, this will cause Zircon's UART1 disable and let console redirection failed.

- Modified the threshold of rear fan speed to meet hardware request (Rear fan is 1000 RPM)

- The severity of "Power supply presence detected" in mail show critical. Because the string in firmware is not correct, we fixed it using "information" string.

- Fixed the bug that boot code can't load default password. Because the position of load default password flag in EEPROM is changed, we fixed it using correct position.

- Modify E-mail string " Rear Fan ..." to " Back FAN 1 .."

- The sending time of e-mail from BMC is incorrect. This is BMC send the incorrect time to mail server. Modified the time in BMC firmware can fix it.

- Running console redirect long time will show garbage in RDM. Because BIOS send the wrong video data to BMC and BMC will transfer it to RDM, It cause RDM (Power VT object) to set incorrect character set. We must filter the wrong video data then sent it to RDM.

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

# T52A management board firmware 1.30 release note

Release Date: 02/19/2003

Firmware Version: 1.30

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.1.1.3

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.50 (Support G700, G701, and G510)

Change from V1.20:

- Change the threshold of 2.5V to meet hardware request (2.6V).

- Modified the threshold of rear fan speed and front fan speed to meet hardware request (rear fan is 1500 RPM, front fan is 1700 RPM)

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

# T52A management board firmware 1.20 release note

Release Date: 02/18/2003

Firmware Version: 1.20

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.1.1.3

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.50

Change from V1.10:

- Fixed the bug of date in SEL show 1970/XX/XX.

- If press power on button and enable remote floppy flag in RDM, BT port will hang up when we reset or turn off the host. This version solve this problem.

- Add Get Oem Capability function (function name is GetOemCap()).

- Change the threshold of rear fan to 1620 RPM, and front fan to 1800 RPM.

- Add "IPMI get LAN configuration parameter" function to get max number of mail entry..

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

# T52A management board firmware 1.10 release note

Release Date: 01/29/2003

Firmware Version: 1.10

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.1.1.1

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.11

Change from V1.00:

- The threshold of voltage and fan of SDR is changed to hardware re-define value.

- Fix the bug of redundant power supply led and event log issues(defect ID 4567 & 4577).

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

# T52A management board firmware 1.00 release note

Release Date: 01/16/2003

Firmware Version: 1.00

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 1.00

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.11

Function support:

- Support Redundant power supply fan detect and log, the threshold is the same as front fan.

- Fix the bug that PCI error, RAM error, BIOS post error,BIOS resize event light system led will fail.(defect id 4490)

- Fix the bug that RDM auto-refresh and reset BMC board, Window will show error dialog.(defect id 4493)

- Fix the bug that when fan speed too low and send mail to user, the content of value and threshold is 0.

- The CPU temperature critical and non-recovery threshold change to 95C and 105C.

- The 3.3V low threshold change to 3.15V.

- Fix the bug that front value is the same as front fan 1.

- Fix the bug that when cpu 1 is disabled, the RDM led will show fail.

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

- The release firmware is only one image name FBMCX.XX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCX.XX to the same directory and type “ filet fbmcx.xx” it will flash firmware.

- Voltage color is always orange in RDM if voltage out of range.

# T52A management board firmware 0.80 release note

Release Date: 12/30/2002

Firmware Version: 0.80

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 0.80

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.11

Function support:

- Support CPU disable event can be logged and send by mail.

- Support CPU reduction.

- Fix bug that event log return to normal can not be send by mail(Defect 4407)

- Fix bug that front fan 1 & 2 can’t be read by RDM(Defect 4371).

- Full support system led policy.

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

  - The function of Power Supply fan is not supported.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

- The release firmware is only one image name FBMCX.XX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCX.XX to the same directory and type “ filet fbmcx.xx” it will flash firmware.

- Voltage color is always orange in RDM if voltage out of range.

# T52A management board firmware 0.75 release note

Release Date: 12/23/2002

Firmware Version: 0.75

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 0.75

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.11

Function support:

- Support CPU reduction and FRB function.

- Support system led policy.

- Support Power supply event log.

- Support send mail if power supply, IERR and thermal trip event happened.

- IPMB support.

- Support graceful shutdown by ACPI button.

Known Issues:

  - Do not support PEF/PET function for now.

  - Don’t support CPU reduction function.

  - Only support one session.

  - CPU disable can support but event log don’t support now.

  - The function of Power Supply fan is not work now.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

- The release firmware is only one image name FBMCX.XX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCX.XX to the same directory and type “ filet fbmcx.xx” it will flash firmware.

- Voltage color is always orange in RDM if voltage out of range.

# T52A management board firmware 0.70 release note

Release Date: 12/16/2002

Firmware Version: 0.70

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.0.0.7

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.11

Function support:

- IPMI 1.5 support.

- Console redirection support.

- Remote floppy boot support.

- Support Voltage, temperature, chassis intrusion, thermal trip,

CPU IERR, memory single & double etc. monitoring.

Known Issues:

  - Do not support PEF/PET function for now.

  - Don’t support CPU reduction function.

  - Only support one session..

  - Send mail is not ready.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

- The release firmware is only one image name FBMCX.XX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCX.XX to the same directory and type “ filet fbmcx.xx” it will flash firmware.