#### Flash Utility Filet V1.70 Release Note

Release Date: 05/13/2003

### Filet Version: V1.70

Fix/Change from V1.60:

- Fix the message typo on screen issue.

Release Date: 05/07/2003

### Filet Version : V1.60

Fix/Change from V1.50:

- Fixed the bug that filet150.exe show the incorrect version on screen.

- This version can support G700 and G701.

### Filet Version : V1.50

Fix/Change from V1.10:

- This version will check the BMC ID and binary file (G700 binary file size is 944KB, G701 binary file size is 1024KB)

- This version can support G700 and G701.

### Filet Version : V1.10

Fix/Change from V1.00:

  - Flash Utility FILET Bug issue :

The flash ROM erase need 14-16 seconds, in the V1.00 (filet.exe) wait for BMC erase time is 15 seconds, so some flash ROM erase is doing and host wait for 15 seconds is timeout, so flash is failure.

Because of the flash flag on EEPROM is not cleared, so when BMC board is reboot boot code will go into update mode and wait for host data.

On version 1.10, the timeout of erase is set to 90 seconds, and release on 08/26/2002. If user use new version may not have this issue.

Flash BMC firmware utility version history:

| Utility Name | Version | Use in |
| --- | --- | --- |
| Filet.exe | V 1.00 | # Only for G700 |
| Filet110.exe | V 1.10 | # Only for G700 Fixed the bug that flash BMC failure issue |
| Filet150.exe | V 1.50 | # Can use in G700, G701 Check the BMC ID and binary file between G700 and G701 |
| Filet160.exe | V 1.60 | # Can use in G700, G701 Fixed the bug that filet150.exe show the incorrect version on screen |
| Filet170.exe | V 1.70 | # Can use in G700, G701 # Fix the message typo on screen issue |