T52A BMC Firmware V2.40 Release Note

Content

T52A management board firmware 2.40 release note	2

T52A management board firmware 2.35 release note	3

T52A management board firmware 2.30 release note	4

T52A management board firmware 2.20 release note	5

T52A management board firmware 2.10 release note	7

T52A management board firmware 2.00 release note	9

T52A management board firmware 1.90 release note	10

T52A management board firmware 1.85 release note	12

T52A management board firmware 1.70 release note	14

T52A management board firmware 1.60 release note	15

T52A management board firmware 1.50 release note	16

T52A management board firmware 1.30 release note	18

T52A management board firmware 1.20 release note	19

T52A management board firmware 1.10 release note	20

T52A management board firmware 1.00 release note	21

T52A management board firmware 0.80 release note	23

T52A management board firmware 0.75 release note	24

T52A management board firmware 0.70 release note	25

# T52A management board firmware 2.40 release note

Release Date: 11/13/2003

Firmware Version: 2.40

BMC/B Version Supported: B3B

M/B Version Supported: B3E

BIOS Version Supported: 3D01

RDM Version Supported: 5.3.1.4

BMCIPCFG.EXE Version Support: 1.7

FILET Version: 1.70 ( Support G700, G701, and G510 , Please reference Flash Release Note V1.70)

Fixed/Change from V2.35:

- Change the version from V2.35 to V2.40

Known Issues:

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Multi-Session allow multiple RDM client to connect BMC at the same time. But, only the first login user that run the “Remote Floppy” or “Console Redirection” function can perform “Power ON/OFF/RESET” to server. If the second user want to “Power ON/OFF/RESET” server, the RDM Version 5.3.1.1 or later will show “Can’t shutdown/reset server, maybe other user using console redirection now” dialog box to notify user. The older RDM version (5.3.1.0 and before) will show “Node Busy” dialog box.

- Update the flash utility from FILET V1.60 to FILET V1.70, please reference the Flash Release Note V1.70.

# T52A management board firmware 2.35 release note

Release Date: 10/20/2003

Firmware Version: 2.35

BMC/B Version Supported: B3B

M/B Version Supported: B3E

BIOS Version Supported: 3C06

RDM Version Supported: 5.3.1.3

BMCIPCFG.EXE Version Support: 1.7

FILET Version: 1.70 ( Support G700, G701, and G510 , Please reference Flash Release Note V1.70)

Fixed/Change from V2.30:

- Fixed the bug that AC lost deassertion show un-know severity

- Fixed PET/PEF and LAN command function to pass ICTS testing

Known Issues:

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Multi-Session allow multiple RDM client to connect BMC at the same time. But, only the first login user that run the “Remote Floppy” or “Console Redirection” function can perform “Power ON/OFF/RESET” to server. If the second user want to “Power ON/OFF/RESET” server, the RDM Version 5.3.1.1 or later will show “Can’t shutdown/reset server, maybe other user using console redirection now” dialog box to notify user. The older RDM version (5.3.1.0 and before) will show “Node Busy” dialog box.

- Update the flash utility from FILET V1.60 to FILET V1.70, please reference the Flash Release Note V1.70.

# T52A management board firmware 2.30 release note

Release Date: 10/13/2003

Firmware Version: 2.30

BMC/B Version Supported: B3B

M/B Version Supported: B3E

BIOS Version Supported: 3C06

RDM Version Supported: 5.3.1.3

BMCIPCFG.EXE Version Support: 1.7

FILET Version: 1.70 ( Support G700, G701, and G510 , Please reference Flash Release Note V1.70)

Fixed/Change from V2.20:

- Modified PET/PEF command to pass ICTS test

- Modified LAN command to pass ICTS test

- The "Server IP address" item in PET/PEF trap doesn't show server IP address. This version fixed it.

Known Issues:

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Multi-Session allow multiple RDM client to connect BMC at the same time. But, only the first login user that run the “Remote Floppy” or “Console Redirection” function can perform “Power ON/OFF/RESET” to server. If the second user want to “Power ON/OFF/RESET” server, the RDM Version 5.3.1.1 or later will show “Can’t shutdown/reset server, maybe other user using console redirection now” dialog box to notify user. The older RDM version (5.3.1.0 and before) will show “Node Busy” dialog box.

- Update the flash utility from FILET V1.60 to FILET V1.70, please reference the Flash Release Note V1.70.

# T52A management board firmware 2.20 release note

Release Date: 08/22/2003

Firmware Version: 2.20

BMC/B Version Supported: B3B

M/B Version Supported: B3E

BIOS Version Supported: 3C06

RDM Version Supported: 5.3.1.3

BMCIPCFG.EXE Version Support: 1.7

FILET Version: 1.70 ( Support G700, G701, and G510 , Please reference Flash Release Note V1.70)

Fixed/Change from V2.10:

- Running the weekly day schedule power on/off once, the fixed day schedule power on/off function will failure. Fixed it in this version.

- Modified the OEM's IANA and ASF IANA in RMCP/ASF pong message packet.

- Modified the CPU1 and CPU2's temperature threshold to meet ACER request (Critical=80, Non-recoverable=85)

- Fixed the bug that BMC will log VCC12V event if system run DC on/off test program

- The description of event severity of single-bit ERROR is wrong, fixed it in this version.

- The description of event severity of power supply AC recovery is wrong, fixed it in this version.

Known Issues:

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Multi-Session allow multiple RDM client to connect BMC at the same time. But, only the first login user that run the “Remote Floppy” or “Console Redirection” function can perform “Power ON/OFF/RESET” to server. If the second user want to “Power ON/OFF/RESET” server, the RDM Version 5.3.1.1 or later will show “Can’t shutdown/reset server, maybe other user using console redirection now” dialog box to notify user. The older RDM version (5.3.1.0 and before) will show “Node Busy” dialog box.

- Update the flash utility from FILET V1.60 to FILET V1.70, please reference the Flash Release Note V1.70.

# T52A management board firmware 2.10 release note

Release Date: 07/03/2003

Firmware Version: 2.10

BMC/B Version Supported: B3B

M/B Version Supported: B3E

BIOS Version Supported: 3C06

RDM Version Supported: 5.3.1.3

BMCIPCFG.EXE Version Support: 1.7

FILET Version: 1.70 ( Support G700, G701, and G510 , Please reference Flash Release Note V1.70)

Fixed/Change from V2.00:

- Add PET/PEF function

- Add schedule power on/off function

- Description of "CPU FAN" in e-mail alert is wrong. This version fixed it.

- When CPU temperature return to normal, BMC doesn't send e-mail alert. Fixed it in this version

- When Power Unit on/off, BMC doesn't send e-mail alert. Fixed in it in this version.

Known Issues:

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Multi-Session allow multiple RDM client to connect BMC at the same time. But, only the first login user that run the “Remote Floppy” or “Console Redirection” function can perform “Power ON/OFF/RESET” to server. If the second user want to “Power ON/OFF/RESET” server, the RDM Version 5.3.1.1 or later will show “Can’t shutdown/reset server, maybe other user using console redirection now” dialog box to notify user. The older RDM version (5.3.1.0 and before) will show “Node Busy” dialog box.

- Update the flash utility from FILET V1.60 to FILET V1.70, please reference the Flash Release Note V1.70.

# T52A management board firmware 2.00 release note

Release Date: 06/23/2003

Firmware Version: 2.00

BMC/B Version Supported: B3B

M/B Version Supported: B3E

BIOS Version Supported: 3C06

RDM Version Supported: 5.3.1.2

BMCIPCFG.EXE Version Support: 1.7

FILET Version: 1.70 ( Support G700, G701, and G510 , Please reference Flash Release Note V1.70)

Fixed/Change from V1.90:

- The "Get Session Challenge" and "Activate Session" command use numeric to indicate the authentication type that application (e.g ICTS) supported. In BMC firmware, we use bit mask to check the supported authentication type. We modified it to meet the IPMI spec.

Known Issues:

  - Do not support PEF/PET function for now.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Multi-Session allow multiple RDM client to connect BMC at the same time. But, only the first login user that run the “Remote Floppy” or “Console Redirection” function can perform “Power ON/OFF/RESET” to server. If the second user want to “Power ON/OFF/RESET” server, the RDM Version 5.3.1.1 or later will show “Can’t shutdown/reset server, maybe other user using console redirection now” dialog box to notify user. The older RDM version (5.3.1.0 and before) will show “Node Busy” dialog box.

- Update the flash utility from FILET V1.60 to FILET V1.70, please reference the Flash Release Note V1.70.

# T52A management board firmware 1.90 release note

Release Date: 06/09/2003

Firmware Version: 1.90

BMC/B Version Supported: B3B

M/B Version Supported: B3E

BIOS Version Supported: 3C06

RDM Version Supported: 5.3.1.2

BMCIPCFG.EXE Version Support: 1.7

FILET Version: 1.70 ( Support G700, G701, and G510 , Please reference Flash Release Note V1.70)

Fixed/Change from V1.85:

- The Lower Non Recovery threshold of FAN change to 0.

- The Upper Non Critical threshold of temperature change to 0

- Change the "Rear Fan 1" string to "Rear Fan" in SDR

- Fixed the bug that Vcc_IMB voltage event will be recorded when system run DC on/off test program.

- To do console redirection enable/disable continually between two RDM client many times will cause console redirection function failed. This version fixed it.

- Change the Sensor Maximum Reading value to 0xFE ( Original value is 0xFF ), because G2 will use it to check the reading range of FAN speed.

Known Issues:

  - Do not support PEF/PET function for now.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Multi-Session allow multiple RDM client to connect BMC at the same time. But, only the first login user that run the “Remote Floppy” or “Console Redirection” function can perform “Power ON/OFF/RESET” to server. If the second user want to “Power ON/OFF/RESET” server, the RDM Version 5.3.1.1 or later will show “Can’t shutdown/reset server, maybe other user using console redirection now” dialog box to notify user. The older RDM version (5.3.1.0 and before) will show “Node Busy” dialog box.

- Update the flash utility from FILET V1.60 to FILET V1.70, please reference the Flash Release Note V1.70.

# T52A management board firmware 1.85 release note

Release Date: 05/14/2003

Firmware Version: 1.85

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.3.1.1

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.70 ( Support G700, G701, and G510 , Please reference Flash Release Note V1.70)

Fixed/Change from V1.70:

- Support Multi-Session function

- Modified the function to synchronize the BMC, BIOS and RDM SEL string

- Modified the sensor maximum reading and sensor minimum reading (Range from 0x7F to 0x80) in SDR for G2 to get temperature correctly

- Add the solution of BMC lost characters when BIOS send video data to BMC

- When FRB1, FRB2 and FRB3 occurs, the event LED will be lightened

- Provide the more voltage level monitor

Known Issues:

  - Do not support PEF/PET function for now.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Multi-Session allow multiple RDM client to connect BMC at the same time. But, only the first login user that run the “Remote Floppy” or “Console Redirection” function can perform “Power ON/OFF/RESET” to server. If the second user want to “Power ON/OFF/RESET” server, the RDM Version 5.3.1.1 or later will show “Can’t shutdown/reset server, maybe other user using console redirection now” dialog box to notify user. The older RDM version (5.3.1.0 and before) will show “Node Busy” dialog box.

- Update the flash utility from FILET V1.60 to FILET V1.70, please reference the Flash Release Note V1.70.

# T52A management board firmware 1.70 release note

Release Date: 04/17/2003

Firmware Version: 1.70

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.2.1.7

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.60 (Support G700, G701, and G510)

Fixed/Change from V1.60:

- Fixed the bug that RDM show CPU 2 temperature over 127 degree C always

2. Fixed the bug that the critical event of voltage and CPU temperature should show blinking LED

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

# T52A management board firmware 1.60 release note

Release Date: 04/03/2003

Firmware Version: 1.60

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.2.1.7

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.60 (Support G700, G701, and G510)

Change from V1.50:

1. To read back fan sensor after 45 seconds when host power on

2. Fixed the bug that RDM show CPU 1 temperature over 127 degree

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

# T52A management board firmware 1.50 release note

Release Date: 03/18/2003

Firmware Version: 1.50

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.2.1.7

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.60 (Support G700, G701, and G510)

Change from V1.30:

- Re-initial BMC UART 1 for console redirection at every LPC reset signal asserted. Because LPC reset signal trigger timer active to initial the peripheral of Zircon, this will cause Zircon's UART1 disable and let console redirection failed.

- Modified the threshold of rear fan speed to meet hardware request (Rear fan is 1000 RPM)

- The severity of "Power supply presence detected" in mail show critical. Because the string in firmware is not correct, we fixed it using "information" string.

- Fixed the bug that boot code can't load default password. Because the position of load default password flag in EEPROM is changed, we fixed it using correct position.

- Modify E-mail string " Rear Fan ..." to " Back FAN 1 .."

- The sending time of e-mail from BMC is incorrect. This is BMC send the incorrect time to mail server. Modified the time in BMC firmware can fix it.

- Running console redirect long time will show garbage in RDM. Because BIOS send the wrong video data to BMC and BMC will transfer it to RDM, It cause RDM (Power VT object) to set incorrect character set. We must filter the wrong video data then sent it to RDM.

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

# T52A management board firmware 1.30 release note

Release Date: 02/19/2003

Firmware Version: 1.30

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.1.1.3

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.50 (Support G700, G701, and G510)

Change from V1.20:

- Change the threshold of 2.5V to meet hardware request (2.6V).

- Modified the threshold of rear fan speed and front fan speed to meet hardware request (rear fan is 1500 RPM, front fan is 1700 RPM)

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

# T52A management board firmware 1.20 release note

Release Date: 02/18/2003

Firmware Version: 1.20

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.1.1.3

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.50

Change from V1.10:

- Fixed the bug of date in SEL show 1970/XX/XX.

- If press power on button and enable remote floppy flag in RDM, BT port will hang up when we reset or turn off the host. This version solve this problem.

- Add Get Oem Capability function (function name is GetOemCap()).

- Change the threshold of rear fan to 1620 RPM, and front fan to 1800 RPM.

- Add "IPMI get LAN configuration parameter" function to get max number of mail entry..

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

# T52A management board firmware 1.10 release note

Release Date: 01/29/2003

Firmware Version: 1.10

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.1.1.1

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.11

Change from V1.00:

- The threshold of voltage and fan of SDR is changed to hardware re-define value.

- Fix the bug of redundant power supply led and event log issues(defect ID 4567 & 4577).

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

# T52A management board firmware 1.00 release note

Release Date: 01/16/2003

Firmware Version: 1.00

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 1.00

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.11

Function support:

- Support Redundant power supply fan detect and log, the threshold is the same as front fan.

- Fix the bug that PCI error, RAM error, BIOS post error,BIOS resize event light system led will fail.(defect id 4490)

- Fix the bug that RDM auto-refresh and reset BMC board, Window will show error dialog.(defect id 4493)

- Fix the bug that when fan speed too low and send mail to user, the content of value and threshold is 0.

- The CPU temperature critical and non-recovery threshold change to 95C and 105C.

- The 3.3V low threshold change to 3.15V.

- Fix the bug that front value is the same as front fan 1.

- Fix the bug that when cpu 1 is disabled, the RDM led will show fail.

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

- The release firmware is only one image name FBMCX.XX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCX.XX to the same directory and type “ filet fbmcx.xx” it will flash firmware.

- Voltage color is always orange in RDM if voltage out of range.

# T52A management board firmware 0.80 release note

Release Date: 12/30/2002

Firmware Version: 0.80

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 0.80

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.11

Function support:

- Support CPU disable event can be logged and send by mail.

- Support CPU reduction.

- Fix bug that event log return to normal can not be send by mail(Defect 4407)

- Fix bug that front fan 1 & 2 can’t be read by RDM(Defect 4371).

- Full support system led policy.

Known Issues:

  - Do not support PEF/PET function for now.

  - Only support one session.

  - The function of Power Supply fan is not supported.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

- The release firmware is only one image name FBMCX.XX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCX.XX to the same directory and type “ filet fbmcx.xx” it will flash firmware.

- Voltage color is always orange in RDM if voltage out of range.

# T52A management board firmware 0.75 release note

Release Date: 12/23/2002

Firmware Version: 0.75

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 0.75

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.11

Function support:

- Support CPU reduction and FRB function.

- Support system led policy.

- Support Power supply event log.

- Support send mail if power supply, IERR and thermal trip event happened.

- IPMB support.

- Support graceful shutdown by ACPI button.

Known Issues:

  - Do not support PEF/PET function for now.

  - Don’t support CPU reduction function.

  - Only support one session.

  - CPU disable can support but event log don’t support now.

  - The function of Power Supply fan is not work now.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

- The release firmware is only one image name FBMCX.XX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCX.XX to the same directory and type “ filet fbmcx.xx” it will flash firmware.

- Voltage color is always orange in RDM if voltage out of range.

# T52A management board firmware 0.70 release note

Release Date: 12/16/2002

Firmware Version: 0.70

BMC/B Version Supported:

M/B Version Supported:

BIOS Version Supported:

RDM Version Supported: 5.0.0.7

BMCIPCFG.EXE Version Support: 1.5

FILET Version: 1.11

Function support:

- IPMI 1.5 support.

- Console redirection support.

- Remote floppy boot support.

- Support Voltage, temperature, chassis intrusion, thermal trip,

CPU IERR, memory single & double etc. monitoring.

Known Issues:

  - Do not support PEF/PET function for now.

  - Don’t support CPU reduction function.

  - Only support one session..

  - Send mail is not ready.

Notice:

- If DHCP server exist, IP is assigned by DHCP server, else will use

the IP that user previous set in BMC board instead. BMC firmware default is 192.168.224.222

- By default, user password is ADMIN@ACER

- Clear event log don’t go out the system led of front panel.

- Version description: VM.BC, M: major number, B: first minor number is release for ACER version, C: second minor number is for beta version

- The release firmware is only one image name FBMCX.XX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCX.XX to the same directory and type “ filet fbmcx.xx” it will flash firmware.