S54 BMC Firmware V2.05 Release Note

#### CONTENT

S54 management board firmware 2.05 release note	2

S54 management board firmware 2.00 release note	4

S54 management board firmware 1.90 release note	6

S54 management board firmware 1.80 release note	8

S54 management board firmware 1.70 release note	10

S54 management board firmware 1.70 release note	12

S54 management board firmware 1.50 release note	14

S54 management board firmware 1.40 release note	16

S54 management board firmware 1.30 release note	18

S54 management board firmware 1.20 release note	20

S54 management board firmware 1.10 release note	22

S54 management board firmware 1.00 release note	24

S54 management board firmware 0.98 release note	26

S54 management board firmware 0.50 release note	28

S54 management board firmware 0.40 release note	30

S54 management board firmware 0.30 release note	32

S54 management board firmware 0.25 release note	34

S54 management board firmware 0.24 release note	36

S54 management board firmware 0.23 release note	37

S54 management board firmware 0.22 release note	38

S54 management board firmware 0.21 release note	39

S54 management board firmware 0.20 release note	40

S54 management board firmware 0.19 release note	41

S54 management board firmware 0.18 release note	43

S54 management board firmware 0.17 release note	44

# S54 management board firmware 2.05 release note

Release Date: 07/05/2005

Firmware Version: 2.05

M/B Version Supported: D3C

BIOS Version Supported: 3B03

ASM Version Supported: 6.00.334 or later

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V2.00:

- Remove Platform Alert SDR.

Known Issues/Workarounds:

  - Don’t support DHCP client

  - BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.50 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 2.00 release note

Release Date: 12/29/2004

Firmware Version: 2.00

M/B Version Supported: D3C

BIOS Version Supported: 3B03

ASM Version Supported: 6.00.334 or later

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V1.90:

- Remove FRB framework from ACER’s request.

- Fix Defect 12018 and 12019.

Known Issues/Workarounds:

  - Don’t support DHCP client

  - BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.50 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 1.90 release note

Release Date: 12/06/2004

Firmware Version: 1.90

M/B Version Supported: D3C

BIOS Version Supported: 3B02

ASM Version Supported: 6.00.334 or later

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V1.80:

- AC ON/OFF test fail while insert ARMC card issue.

- New FRB framework from ACER’s request.

- New ECC request while mirror is enabled.

- FRB3 timeout timer is changed to 20 sec from 8 sec.

Known Issues/Workarounds:

  - Don’t support DHCP client

  - BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.50 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 1.80 release note

Release Date: 10/21/2004

Firmware Version: 1.80

M/B Version Supported: D3B

BIOS Version Supported: 3B01

ASM Version Supported: 6.00.334 or later

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V1.70:

1	Change SDR entity instance of Memory Resize from 1 to 0 in order to work around issue of ASM PET always show Memory module 1.

2.	Fixed FRB3 timeout if battery removed and AC Off then On.

- FRB3 timeout timer is changed to 8 sec from 4 sec.

- BIOS can't show event log issue when AC off then on.

Known Issues/Workarounds:

  - Don’t support DHCP client

  - BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

  - By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

  - The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

  - Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

  - The release firmware is only one image name BMCXXX.bin ( We released this version V1.50 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

  - This release includes BroadCom 5721 image and flash tool.

  - For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

  - The BroadCOM boot code version is 3.29a, ASF version is 8.04

  - Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

  - Execute the FlashNIC.bat to flash the NIC image

  - In this version, we also release new version of BMCIPCFG.exe Ver 1.8

  - BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

  - BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 1.70 release note

Release Date: 10/20/2004

Firmware Version: 1.70

M/B Version Supported: D3B

BIOS Version Supported: 3B01

ASM Version Supported: 6.00.334 or later

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V1.60:

- Enhance IPMB port performance to prevent IPMB port lock issue.

Known Issues/Workarounds:

  - Don’t support DHCP client

  - BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.50 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from version 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 1.70 release note

Release Date: 10/19/2004

Firmware Version: 1.60

BMC/B Version Supported:

M/B Version Supported: D3B

BIOS Version Supported: 3A13

ASM Version Supported: 6.00.334 or later

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V1.50:

- Change SDR entity instance of Memory from 1 to 0 in order to work around issue of ASM PET always show Dimm 1.

- Change SDR entity instance of Critical Interrupt from 1 to 0 in order to work around issue of ASM PET always show System Bus 1.

Known Issues/Workarounds:

- Don’t support DHCP client

- BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.50 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 1.50 release note

Release Date: 10/19/2004

Firmware Version: 1.50

BMC/B Version Supported:

M/B Version Supported: D3B

BIOS Version Supported: 3A13

ASM Version Supported: 6.00.334 or later

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V1.40:

- Change SDR entity instance of Memory from 1 to 0 in order to work around issue of ASM PET always show Dimm 1.

- Change SDR entity instance of Critical Interrupt from 1 to 0 in order to work around issue of ASM PET always show System Bus 1.

- Initial IPMB data structure before open this port. Prevent receiving interrupt before data structure initial will cause system hang.

Known Issues/Workarounds:

- Don’t support DHCP client

- BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.50 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 1.40 release note

Release Date: 10/04/2004

Firmware Version: 1.40

BMC/B Version Supported:

M/B Version Supported: D3B

BIOS Version Supported: 3A11

ASM Version Supported: 6.00.306 or later

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V1.30:

- Fixed the bug that single ECC event will be logged when DIMM was disabled

- Watchdog will be disabled when system power off or reset.

Known Issues/Workarounds:

- Don’t support DHCP client

- BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.40 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 1.30 release note

Release Date: 09/23/2004

Firmware Version: 1.30

BMC/B Version Supported:

M/B Version Supported: D3B

BIOS Version Supported: 3A10

ASM Version Supported: 6.00.306 or later

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V1.20:

- Fixed the bug that power on system will hang up at 0xA2 issue

- Add OEM command to stop reading the SATA status when it is in update firmware state.

Known Issues/Workarounds:

- Don’t support DHCP client

- BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Known Issues/Workarounds:

- Don’t support DHCP client

- BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.30 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 1.20 release note

Release Date: 09/21/2004

Firmware Version: 1.20

BMC/B Version Supported:

M/B Version Supported: D3B

BIOS Version Supported: 3A09

ASM Version Supported: 6.00.306 or later

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V1.10:

- Add voltage sensor for Cage 2 back plane

- Fixed the bug that sometime the SMBus will hang up when BMC read the HDD status from SATA BP.

Known Issues/Workarounds:

- Don’t support DHCP client

- BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.20 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 1.10 release note

Release Date: 09/01/2004

Firmware Version: 1.10

BMC/B Version Supported:

M/B Version Supported: D3A

BIOS Version Supported: 3A07

ASM Version Supported: 6.00.306

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V1.00:

- Add T-control function

- Add SATA HDD monitor function

- Add SATA H/W sensor monitor function

- Fixed the bug memory ECC still log event when DIMM disable

- Reset the single ECC error count if the timer is timeout (10 times/hr)

- Change HDD string from 0~7 to 0~3

- Support VRD hot protected function

- Fixed the bug that running DC ON/OFF will cause BMC reset if ARMC is installed

Known Issues/Workarounds:

- Don’t support DHCP client

- BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.10 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 1.00 release note

Release Date: 07/27/2004

Firmware Version: 1.00

BMC/B Version Supported:

M/B Version Supported: D3A

BIOS Version Supported: 3A07

ASM Version Supported: 6.00.306

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.98:

- Change the firmware version V0.98 to V1.00

- Change the “SCSI 2 temp” string to “SCSI 1 temp” string in SDR

Known Issues/Workarounds:

- Don’t support DHCP client

- BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V1.00 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 0.98 release note

Release Date: 07/23/2004

Firmware Version: 0.98

BMC/B Version Supported:

M/B Version Supported: D3A

BIOS Version Supported: 3A04

ASM Version Supported: 6.00.300

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.50:

- Fixed defect 10261, The fault LED can’t return to normal (no blinking) when SCSI temp return to normal event occurs.

- Fixed defect 10084, BMC will log error DIMM slot number when ECC event occurs.

- Fixed defect 10608, BMC don’t set the processor thermal assertion bit in SDR, we fixed it in version 0.98.

- Remove PCI PSER and SERR function

- Fixed defect 10706, If plug in the two RJ45 connectors to LAN port, remote software can’t login to BMC.

Known Issues/Workarounds:

- Don’t support DHCP client

- BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name BMCXXX.bin ( We released this version V0.98 is runtime code ), the flash utility is filet.exe. Copy filet.exe and BMCXXX to the same directory and type “ filet bmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

- BMC support dedicate MAC address and static IP other than OS. User has to set a static IP other than OS when configuring BMC's IP address. Otherwise, BMC OOB does not work.

- BMC from vesion 040, it supports "Fail-Over-LAN" solution. BMC can be connected from OOB via PORT 2 or PORT1. And application/user has no needs to re-establish the session/connection when one of the LAN-Leash is occurred.

# S54 management board firmware 0.50 release note

Release Date: 07/07/2004

Firmware Version: 0.50

BMC/B Version Supported:

M/B Version Supported: D3A

BIOS Version Supported: 3A02

ASM Version Supported: 6.00.299

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.40:

- Fixed defect 10516, ICTS test fail item “Get Chassis Capabilities”

- Fixed defect 10448, Processor Disabled assert LED status is different with spec ( spec define LED status is Amber blinking )

- Fixed defect 10344, fill the event severity, sensor device and entity instance in PET trap to meet spec definition.

- Fixed defect 10260, Vcore error assert LED status is different with spec ( spec define LED status is Amber on )

- Fixed the VTT, VTT_GTL, V1.8 and VGA voltage out of range issue. (H/W must be PVT M/B, firmware must be V0.50)

- Modified SDR “sensor reading conversion formula” for –12V

- The field in PET trap “server IP address” must be the server’s IP (BMC IP).

- Fixed the PCI SERR can’t be generated issue.

- Switch the COM1 owner to host when system is power on and to BMC when system is power off

Known Issues:

- Don’t support DHCP client

- BMC support sensor type “Event Logging Disabled” event ( Offset 02h Log Area Reset/Cleared ). Note : Keep in mind log this event will cause ICTS test fail.

Notice:

- By default, user password is ADMIN@ACER ( Login from ASM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A03

- The BroadCOM boot code version is 3.29a, ASF version is 8.04

- Please make a boot disk and load high memory device ( in config.sys use “DEVICE=HIMEM.SYS” ) to make sure flash share NIC successful.

- Execute the FlashNIC.bat to flash the NIC image

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

# S54 management board firmware 0.40 release note

Release Date: 06/24/2004

Firmware Version: 0.40

BMC/B Version Supported:

M/B Version Supported: C2C

BIOS Version Supported: 2A05

GRW Version Supported:

BMCIPCFG.EXE Version Support: V 1.8 (Release data 2004/6/23)

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.30:

- Support LAN leash fail over.

- Support memory device disable function.

- Change the IANA and password to meet customer requirement

- Fixed wrong maximum and minimum reading value in SDR for -12V sensor

- Implement PET severity to meet customer requirement

- Filter the NULL character from CR port

- To fix status LED can’t return to normal when SCSI temperature is OK

- Change the Processor Hot, Vcore, SCSI fuse and USB over current status LED to meet customer requirement.

Known Issues:

- Don’t support DHCP client

Notice:

- By default, user password is ADMIN@ACER ( Login from ADM, RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A02

- The BroadCOM boot code version is 3.28a, ASF version is 8.04

- Please make a Windows Millennium boot disk and copy those in the same directory.

- Execute the FlashNIC.bat to flash the NIC image

- NOTE: Windows Millennium OS can flash NIC image successful. But other OS like DOS can’t be guarantee.

- In this version, we also release new version of BMCIPCFG.exe Ver 1.8

# S54 management board firmware 0.30 release note

Release Date: 06/09/2004

Firmware Version: 0.30

BMC/B Version Supported:

M/B Version Supported: C2B

BIOS Version Supported: 2A03

GRW Version Supported:

BMCIPCFG.EXE Version Support:

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.25:

- Support SCSI BP voltage, temperature and SAFE-TE board sensor function

- Fixed the LAN command can’t pass ICTS test issue

- Fixed the bug that BMC will log rear fan too low event when system power cycle.

- Modified the share NIC SMBus to 100K

- Support OEM command (0xF6) to change the active LAN port for remote manager software (e.g. GRW, ASM..).

- Modified the FAN speed control table for thermal team request

- Add the LNCT and UNCT threshold value for voltage

Known Issues:

- Don’t support LAN fail-over function.

- Don’t support DIMM disable function

Notice:

- By default, user password is quanta ( Login from RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A01

- Please make a Windows Millennium boot disk and copy those in the same directory.

- Execute the FlashNIC.bat to flash the NIC image

- NOTE: Windows Millennium OS can flash NIC image successful. But other OS like DOS can’t be guarantee.

# S54 management board firmware 0.25 release note

Release Date: 05/28/2004

Firmware Version: 0.25

BMC/B Version Supported:

M/B Version Supported: C2B

BIOS Version Supported: 2A03

GRW Version Supported:

BMCIPCFG.EXE Version Support:

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.24:

- Fixed defect 9884, AC Power lost/resume test for Dos failed

- Fixed defect 9991, All of Voltage sensor Threshold about UNCT,LCT still un-define on SDR

- Fixed defect 9668, The event log show VCC -12V out of range and Rear fan out of range when run AC on/off

- Add OEM command to enable/disable memory ECC, AC lost, intrusion and PCI PERR, SERR event log.

- Fixed invalid SEL time issue

- Support S52 SCSI BP H/W sensor ( SCSI1 and SCSI2 BP temperature detection )

- Support power supply voltage detection

- When BIOS post state, don’t access MCH until BIOS sent command to BMC.

Known Issues:

- BMC out-band only support LANB.

- Don’t support LAN fail-over function.

Notice:

- By default, user password is quanta ( Login from RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

- This release includes BroadCom 5721 image and flash tool.

- For NIC image and flash tool include following files : FlashNIC.bat, B57DIAG.EXE, CPU05.BIN, FLSHDG05.BIN and S54ASF.A01

- Please make a Windows Millennium boot disk and copy those in the same directory.

- Execute the FlashNIC.bat to flash the NIC image

- NOTE: Windows Millennium OS can flash NIC image successful. But other OS like DOS can’t be guarantee.

# S54 management board firmware 0.24 release note

Release Date: 05/12/2004

Firmware Version: 0.24

BMC/B Version Supported:

M/B Version Supported: C2B

BIOS Version Supported: 2A01

GRW Version Supported:

BMCIPCFG.EXE Version Support:

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.23:

- Fixed defect 9257, Running SDR check utility have error message

- Fixed defect 9795, -12V is to high if plug out power supply 2 AC

- Fixed defect 9587, Running ICTS5.06 out-band test will show “BMC did not respond packet

- Modified PET/PEF to pass ICTS testing

- Fixed chassis intrusion issue. If chassis is close, BMC will log chassis open event.

Known Issues:

- Share NIC auto-discovery from LAN is not supported in this version.

- Don’t support Remote floppy boot

Notice:

- By default, user password is quanta ( Login from RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

# S54 management board firmware 0.23 release note

Release Date: 04/21/2004

Firmware Version: 0.23

BMC/B Version Supported:

M/B Version Supported: B2A

BIOS Version Supported: 1A10

GRW Version Supported:

BMCIPCFG.EXE Version Support:

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.22:

- Fixed defect 9486, the Hardware Monitor Over Current status show ok when SCSI and USB asserted in CLI mode.

- Fixed defect 9143 and 9146, PET/PEF configuration parameter ICTS test fail issue.

- Fixed defect 9144, GUID not define in PET package.

- Monitor the SCSI cage to detect HDD inserted, device faulty and HDD rebuilding event.

Known Issues:

- This firmware version of Share NIC only support master mode.

- Share NIC auto-discovery from LAN is not supported in this version.

- Don’t support Remote floppy boot

Notice:

- By default, user password is quanta ( Login from RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

# S54 management board firmware 0.22 release note

Release Date: 04/15/2004

Firmware Version: 0.22

BMC/B Version Supported:

M/B Version Supported: B2A

BIOS Version Supported: 1A10

GRW Version Supported:

BMCIPCFG.EXE Version Support:

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.21:

- Fixed PET/PEF ICTS test fail issue

- Fixed event log LED can’t work normally issue

- Fixed Console redirection can’t work issue

Known Issues:

- This firmware version of Share NIC only support master mode.

- Share NIC auto-discovery from LAN is not supported in this version.

- Don’t support Remote floppy boot

Notice:

- By default, user password is quanta ( Login from RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

# S54 management board firmware 0.21 release note

Release Date: 04/07/2004

Firmware Version: 0.21

BMC/B Version Supported:

M/B Version Supported: B2A

BIOS Version Supported: 1A10

GRW Version Supported:

BMCIPCFG.EXE Version Support:

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.20:

- Fixed defect 9190, FAN speed don’t match BMC spec definition

- Fixed defect 9267, Power supply FAN fail can’t be log problem

- Fixed defect 9261, PCI SERR and PERR can’t be log problem

- Fixed defect 9019, PCI SERR and PERR can’t be log problem

Known Issues:

- This firmware version of Share NIC only support master mode.

- Share NIC auto-discovery from LAN is not supported in this version.

- Don’t support Remote floppy boot

Notice:

- By default, user password is quanta ( Login from RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

# S54 management board firmware 0.20 release note

Release Date: 03/18/2004

Firmware Version: 0.20

BMC/B Version Supported:

M/B Version Supported: B2A

BIOS Version Supported: 1A05

GRW Version Supported:

BMCIPCFG.EXE Version Support:

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.19:

- Support remote access to BMC from Share NIC (LAN A)

- Fix the defect 8995

- Fix the defect 8994

Known Issues:

- Don’t support Share NIC (LAN B) in this version

- Don’t support Remote floppy boot

Notice:

- By default, user password is quanta ( Login from RDM or GRW )

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

# S54 management board firmware 0.19 release note

Release Date: 03/04/2004

Firmware Version: 0.19

BMC/B Version Supported:

M/B Version Supported: B2A

BIOS Version Supported: 1A05

GRW Version Supported:

BMCIPCFG.EXE Version Support:

FILET Version: V1.5.4 (Release date 2003/9/1)

Change from V0.18:

- Fixed the defect 8362. Write FRU inventory data test fail in ICTS 5.05

- Fixed the defect 8399. Enable message channel receive test result not implemented

- Fixed the defect 8450. Voltage sensor display error in CLI mode

- Fixed the bug that BMC will reset when system is power on or reset.

- Fixed the defect 8942, 8944. Redundant power supply AC lost event show incorrect string.

- Fixed the defect 8958. Remove rear fan, front fan1, front 2, the event log don’t be record

Known Issues:

- Don’t support Share NIC in this version (BROADCOM 5721 issue)

- Don’t support Remote floppy boot

Notice:

- By default, user password is quanta (In CLI mode)

- Because BroadCOM share NIC is not ready, we release two versions for SIT test. The first one is BMC019_1 which enable COM1 for BMC, the other is BMC019_2 which enable COM1 for HOST.

- The FRUREAD.exe and FRUWRITE.exe is the utility tool to read and to write MB FRU data.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

# S54 management board firmware 0.18 release note

Release Date: 02/19/2004

Firmware Version: 0.18

BMC/B Version Supported:

M/B Version Supported: B2A

BIOS Version Supported:

GRW Version Supported:

BMCIPCFG.EXE Version Support:

FILET Version:

Change from V0.17:

- Change the CPU disable pin setting from output pin to input pin. This change is to fix current leakage from 3.3V.

Known Issues:

- Don’t support Share NIC in this version (BROADCOM 5721 issue)

- Don’t support Remote floppy boot

Notice:

- By default, user password is quanta (In CLI mode)

- BMC018_1 enable COM1 for BMC, BMC018_2 enable COM1 for HOST.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware

# S54 management board firmware 0.17 release note

Release Date: 02/16/2004

Firmware Version: 0.17

BMC/B Version Supported:

M/B Version Supported: B2A

BIOS Version Supported:

GRW Version Supported:

BMCIPCFG.EXE Version Support:

FILET Version:

Change from V0.16:

- Add CPU VRD hot function

- Add CPU Vcore function

- Add USB and SCSI fuse over current function

- Add power supply present, failure function

- Test ICTS SEL command failure is fixed in this version

Known Issues:

- Don’t support Share NIC in this version (BROADCOM 5721 issue)

- Don’t support Remote floppy boot

Notice:

- By default, user password is quanta (In CLI mode)

- BMC017_1 enable COM1 for BMC, BMC017_2 enable COM1 for HOST.

- Version description: VM.BC, M: major number, B: first minor number, C: second minor number is for beta version

- The release firmware is only one image name FBMCXXX.bin, the flash utility is filet.exe. Copy filet.exe and FBMCXXX to the same directory and type “ filet fbmcxxx.bin” it will flash firmware