# NVDATA Release Notes

# 53C1020, 53C1030, 53C1030T, 53C1035

Release Notes and Restrictions for 1.05.02.00:

VERSION 1.05.02.00

3/05/2004

Major Changes From Version 1.05.01.00:

General Changes

- Rolled version number.

- Changed SEEPROM load start address for the 1020A/1030T.

#### Defect fixes

- The SEEPROM load start address was changed for the 1020A and 1030T.  This is only used when concatenating images used for firmware download boot.  This version extracts the SEEPROM load start address out of the firmware image and places it in the extended image header.

VERSION 1.05.01.00

1/27/2004

Major Changes From Version 1.05.00.00:

General Changes

- Rolled version number.

- Changed size of Manufacturing Page 3.

#### Defect fixes

- The size of Manufacturing Page 3 was increased from 6 to 7 dwords.  The incorrect size was causing the firmware to see an incorrect checksum value, and cause it to write default values to Manufacturing Page 3.  The GPIO pin settings and NVSRAM/FLASH timings are stored in this page.

VERSION 1.05.00.00

Major Changes From Version 1.04.03.00:

General Changes

- Rolled version number.

#### Functionality

- This version of NVDATA adds support for version 0x0C of the NVDATA structure.

- NVDATA.EXE should be used for version 0x0B of the NVDATA structure.

- NVDATA_C.EXE should be used for version 0x0C of the NVDATA structure.

#### Defect fixes

- None

# Version 1.04.03.00

# 6/16/2003

Release Notes and Restrictions for 1.04.03.00:

- This version of NVDATA is to be used with version 0x0B of the NVDATA structure.

- The /VPD option cannot be used in conjunction with the /CONV option.

VERSION 1.04.03.00

Major Changes From Version 1.04.02.01:

General Changes

- Rolled version number.

Functionality

- None.

Defect fixes

- None

VERSION 1.04.03.00

General Changes

Functionality

- Add /PRE command line option to allow user to define a serial number prefix that must be matched when scanning bar codes on the board during VPD data programming.

Defect fixes

- None

VERSION 1.04.02.00

General Changes

Functionality

- Updated to get VPD data fully supported via command line.  New command line option, /VPD, added.  Can be used when entering a .DAT file for downloading or as the only operation.

Defect fixes

- None

VERSION 1.04.01.00

Major Changes From Version 1.04.00.00:

General Changes

Functionality

- **This version is for Maufacturing only **

- Updated tracer # input functions to require a specific           length of 14 characters (length per manufacturing).

- If tracer scan fails after 3 attempts, a fail message will be displayed/logged and no programming will occur.

Defect fixes

- None

VERSION 1.04.00.00

Major Changes From Version 1.03.03.00:

General Changes

Functionality

- Added  support for the 53C1035. This version now supports the 53C1020, 53C1030, and 53C1035.

Defect fixes

- None

VERSION 1.03.03.00

Major Changes From Version 1.03.02.00:

General Changes

Functionality

- Added /MATCH command line option to force NVDATA to check that the SSID/SSVID in the current adapter’s SEEPROM matches up with the SSID/SSVID entries in the user specified file.  In the case of a mismatch, the current adapter will be skipped.

Defect fixes

- None

VERSION 1.03.02.00

Major Changes From Version 1.03.01.00:

General Changes

Functionality

- Basic 1035 support.

Defect fixes

- None

VERSION 1.03.01.00

Major Changes From Version 1.03.00.00:

General Changes

Functionality

- Added ability to scan (or type) in a Board Tracer Number and have it stored in the Manufacturing Page 0 BoardTracerNumber field.

Defect fixes

- None

VERSION 1.03.00.00

Major Changes From Version 1.01.01.00:

General Changes

Functionality

- Added error messages if the SEEPROM bus can not be cleared up and is not operating correctly.

Defect fixes

- Clear up the Two-Wire-Serial Bus each time DiagMode is entered to put the SEEPROM in a good state.

VERSION 1.01.01.00

Major Changes From Version 1.01.00.00:

General Changes

Functionality

- Enabled for use with configurations that consist of up to 32 channels.

Defect fixes

- Fixed issue with Single Channel devices causing the adapter # to be off by 1.

VERSION 1.01.00.00

Major Changes From Version 1.00.10.00:

General Changes

Functionality

- Removed "Hit any key to continue..." messages and required keystroke, when not in menu mode.

- Removed separate logging of pattern tests in separate files for each adapter (these are now all logged in "LOG.TXT").

- Added persistent logging to "LOG.TXT" when downloading a ".DAT" or ".IMG" file to serial EEPROM.

- All errors are displayed on the screen and the log file when downloading a ".DAT" or ".IMG" file to serial EEPROM.

- Added Date/Time stamping to the log file.

- Added Serial EEPROM verification after downloading to the Serial EEPROM - the Serial EEPROM is now read back after writting to verify the data was written correctly.

- The word "PASSED" is now output only to denote successful status of downloading a  ".DAT" or ".IMG" file to serial EEPROM.  This is for automated detection of successful downloading via the log file.

- The message "Error: No PCI SCSI chips found in system." was changed to  "Error: No MPT SCSI chips found in system."

Defect fixes

- None

##### VERSION 1.00.10.00

Major Changes From Version 01.00.08.00:

General Changes

Functionality

- Modified command line options and usage.

- Added autodetection of .IMG or .DAT file input.

- No command line switches are now needed to download a .IMG or .DAT file.

- The .DAT file may have any filename (it used to have to be named NVDATA.DAT).

- The input or output files may be located in any directory if the path is give.

- Command line parsing algorithms were rewritten.

###### Defect fixes

- None.