## R300 BIOS REVISION

| # Date | # BIOS Version | ### Description | CheckSum |
| --- | --- | --- | --- |
| 01/20/03 | R1.00 (R01-A0 A0) | 1. Change the hot key that enter setup to be 'Ctrl_Alt_Esc'. 2. Exchange logo. 3. Set the default setting of 'ASF', 'WOL', and 'Boot fail count' to be disable and remove these three Setup selections. 4. Modify 2-level page of Setup 'Product Data', 'System Security', and 'Error Logs' to be 1-level. 5. Change Setup background color. 6. Exchange SSID and SVID of VGA controller. 7. Remove IBM copyright string. 8. Modify all error code string. 9. Build static DMI table type 26, 27, and 28. | 9E00H |
| 01/27/03 | R01-A0 A1 | Update items: 1. Remove ASF and ASR. 2. Update SMBIOS type 26, 27, and 28 detection code. 3. Update eventlog code block. 4. Modify the style of BIOS version number. 5. Modify company mark to be 'Acer'. Working items: 1. Eventlog style modification. 2. Setup eventlog list interface modification. 3. Resolve that LSI SCSI can not initial. | 3800H |
| 01/28/03 | R01-A0 A2 | Update items: 1. Resolve that LSI SCSI can not be initialized. 2. Update eventlog code block and eventlog interface of Setup. Working items: 1. Event log style modification. 2. Setup event log interface modification. 3. Setup item strings and message strings modification. | 1200H |
| 01/29/03 | R01-A0 A3 | Update items: 1. Modify Setup items, help messages, and event log viewer. Working items: 1. Event log function. 2. Setup product information table. 3. Post error F1 key function. | 2600H |
| 01/30/03 | R01-A0 A4 | Update items: 1. Modify Setup ‘Product Information’ table. Working items: 1. Event log function. | 0200H |
| 02/07/03 | R01-A0 A5 | Update items: 1. Update event log module. 2. Support 'F9' key toggle screen mode. 3. Resolve that DMI table does not update fan status when fan status changes from critical to ok. 4. Resolve that LSI SCSI can not be initialized. Working Items: 1. Update event log code. 2. Resolve that DMI table does not update voltage status when voltage status changes 3. Resolve that BIOS version title of text mode screen is damaged. | AE00H |
| 02/12/03 | R01-A1 A0 | Update items: 1. Support DMI Fan, Voltage, and Temperature flash. 2. Resolve that Celeron CPU will be showed to be 'Pentium 4'. 3. Update event log format. 4. Support system health LED blink function when event issues. 5. Support memory module speed. 6. Modify CPU L1 cache to be 8 KB. Working Items: 1. System will hang up when W2K warm reboot. 2. Support Fan speed control. | 6500H |
| 02/14/03 | R01-A1 A1 | Update items: 1. Support 5 Level fan speed control. 2. (PTS bug 18) Support PNP function 54 sub-function 0 to clear event logs. 3. Modify Setup Console Redirection page. 4. Modify DMI table type 27 OEM definition. 5. (PTS bug 10) Include the solution that F10 key function affect memory size display function. 6. Resolve that TAB toggle function will destroy screen. 7. (PTS bug 1 and 21) Resolve that system always set LAN PXE to be first boot up device. 8. Update LAN PXE firmware. 9. Resolve that BIOS will show unknown string on text mode screen. 10. (PTS bug 15) Include the solution that system hangs up when W2K warm reboot. Working items: 1. Event log function. | AE00H |
| 02/18/03 | R01-A1 A2 | Update items: 1. Resolve that Setup 'Event Logging' can not be disabled. 2. (PTS bug 29) Resolve that Setup 'L1 cache' shows 20 KB. 3. (PTS bug 28) Resolve that the text mode screen is not correct at the quiet boot mode after press TAB key. 4. (PTS bug 26) Correct the wrong string, 'Onboard serial port A IRQ conflict(s).'. 5. (PTS bug 25) Correct the wrong string, 'PS/2 pointing device error or not correct'. 6. Resolve W2K HCT ACPI system test fail. Working items: 1. Event log function. | 0900H |
| 02/19/03 | R01-A1 A3 | Update items: 1. (PTS bug 42) Modify that L1 cache is showed to be 20 KB on configuration table. 2. (PTS bug 39) Resolve that BIOS version information on W2K is not correct. 3. (PTS bug 35) Remove 'Power-on Passwrod corrupted' event. 4. (PTS bug 34) Resolve that system BIOS can not detect 'AC power has been restored.' event. 5. (PTS bug 33) Resolve that system BIOS can not detect 'CMOS battery failure.' event. 6. (PTS bug 30) Disable system resource of LPT port. 7. (PTS bug 28) Resolve that BIOS can not show 'Entering Setup' message after press Ctrl_Alt_Esc hot key after toggle screen mode from graphic mode to text mode. 8. (PTS bug 27) Support PCI PERR and SERR event detection. 9. (PTS bug 22) Rsolve that BIOS event log is different from ASM SMBIOS SEL. 10. (PTS bug 19) Resolve that system fan definition is wrong on event log function. 11. (PTS bug 7) Resolve that memory size is not correct after toggle screen mode from graphic mode to text mode. 12. (PTS bug 5) Resolve that the Bus Speed of memory infomation is unknown. 13. (PTS bug 3) Resolve that system will hang up at Check Point A7 under system archecture. 14. (PTS bug 43) Resolve that USB controller can not be disabled. 15. Modify CPU frequency 2.52 to 2.53. Working items: 1. Run-time voltage, fan, and temperature event log function. | ED00H |
| 2003/02/24 | R01-A1 A4 | Update items: 1. Include GCSL errata. 2. Remove the PCI SERR PERR, ECC single-bit multi-bit, and AC power recovery debug code. 3. (PTS bug 40) Resolve that F12 key can not break pause status. 4. (PTS bug 9) Resolve that system logs point device error when break memory test. 5. (PTS bug 41) Resolve that BIOS can not log Fan, Voltage, and Temperautre event at POST. 6. (PTS bug 43) Resolve that BIOS can not log Fan, Voltage, and Temperature event at Windows. 7. Modify system fan 4 speed control to full time full speed. 8. Correct that memory module bank of DMI memory module information always is 'single bank'. 9. Support to engage memory test at POST automatically when multi-bit ECC error at run-time. 10. Resolve system can not issue NMI normally after any system event issued. | D600H |
| 2003/03/07 | R01-A1 A5 | 1. Remove CIOBX2 register offset 7B setting. 2. Resolve that system temperature can not be logged. 3. Modify system temperature alert and shutdown threshold. 4. Modify ECC log limitation to be 35 times/2 hrs. 5. Resolve that system temperature threshold follow CPU temperature to change. 6. Modify fan event log to be only one time. 7. Modify motherboard ID to be 'M51SL-1M'. 8. Modify Vcore's high limit and low limit to follow W83791D register 2Bh and 2Ch. 9. Modify ACPI_enable. 10. Change the stack of PCI device initialization at POST. | 4900H |
| 2003/03/10 | R01-A2 A0 | Update items: 1. Update Vcore high limit and low limit code block. 2. Modify ECC log to be 10 times/ 1 hour. 3. Modify boot startup sequence to let floppy to be first one. | 4800H |
| 2003/03/11 | R01-A2 A1 | Update items: 1. Correct BIOS date and Windows BIOS version number. 2. Update boot startup sequence. | 3C00H |
| 2003/03/24 | R01-A2 A2 | Update items: 1. Correct BIOS date and Windows BIOS version number. 2. Update boot startup sequence. 3. (PTS bug 51) Support >137GB HDD. 4. (PTS bug 55) Resolve BIOS loads CMOS default setting when multi-bit ECC error issues at run time. 5. (PTS bug 54) Remove Wistron BIOS version string. 6. Fix system problem when USB disable. 7. Modify system voltage high limit and low limit to be +-0.2/1.5. 8. (PTS bug 53) Remove setup hot key function of boot menu. 9. (PTS bug 52) Correct the Celeron L2 cache information of configuration table. 10. Fix the failure of PQA's PS/2 mouse test. 11. (PTS bug 56) Fix the initial problem of LSI 1600 ELITE on Windows OS. 12. (PTS bug 57) Remove the incorrect event log, 'PCI memory conflict'. | 1200H |
| 2003/03/31 | R01-A2 A3 | Update items: 1. Support hardware system reset. 2. (PTS bug 58) Modify IDE event strings. 3. (PTS bug 59) Modify Setup IDE channel information strings. 4. Modify configuration table IDE string. 5. Modify north bridge function 0 register 4Eh, 4Fh, and 54h by following M51SG. 6. Move CIOBX2 function 0 register F0-F3 code block to the front side of CP B0. 7. Fill 1 to date when the date is invalid zero. | 5300H |
| 2003/04/09 | R01-A2 A4 | Update items: 1. Add the solution of Setup system information delays to show. 2. Modify the memory reduction code block of CP 1E. 3. Include the solution of windows 2003 FoxFire PME problem. 4. Add power button notify when system wakes up from S1. | D100H |
| 2003/06/05 | R01-A2 A5 | Update items: 1. (PTS bug 38) Resolve system hangs up after exit Setup when reduce slot 3 DIMM. 2. Modify IDE event to show ATA error when there is error on IDE drive and to show IDE change when just change IDE drive. 3. Correct the problem that system can not log ECC event when ECC error issues at the highest bank. 4. Implement in new bu-code, M04F2911. 5. Skip I2C bus access if AMI G2 present on system. 6. Change USB code model. 7. Resolve that the virtual floppy function can not work properly. 8. Move the event log code block to front side of USB initialization. 9. (PTS bug 81) Slow down fan speed more early. | CF00H |
|  | R01-A3 A0 | Update items: 1. (PTS bug 81) Resolve that hard size data is destroyed by boot sequence data. 2. Add CPU 24 to 30 ratio. 3. Add the solution of S4 power button issue. 4. To shut down system when hyper-threading technology CPU is present at system. 5. Modify the behavior of I2C error when AMI G2 is present at system. 6. Resolve that system delays to boot when AMI G2 is present at system. 7. Modify the solution for system fail to pass warm reboot test. 8. Remove the BIOS code block that forces to enable USB legacy function when AMI G2 present at system. 9. Resolve that system hangs up at configuration table when plug USB CDROM. 10. Force fan speed to be full speed. |  |
| 2003/07/01 | R01-A3 A1 | Update items: 1. Fill 83791 with one set of initial table after memory initialization. 2. Modify the high limitation and low limitation of voltage probe. | 1200H |
| 2003/07/03 | R01-A3 A2 | Update items: 1. Add 2 seconds delay before polling hardware monitor. | 0B00H |