Contents:

##### Product Introduction

Compatible Hardware Type ------------------------------------------------------- 2

Minimal System Requirements for Playing DVD Movies --------------------- 2

Operation environment ------------------------------------------------------------ 2

Compatible Disc Types ------------------------------------------------------------ 2

Special features of the product --------------------------------------------------- 3

Accessories ------------------------------------------------------------------------- 4

Names and Functions of Product Components -------------------------------- 5

##### Applications in Windows 95/98

Before you start --------------------------------------------------------------------- 6

Confirm the Card Bus Driver in the Notebook PC ------------------------------- 6

Confirm the Version of the Windows 95 ------------------------------------------ 6

#### Requirements for Using the Card Bus 32bit Mode ------------------------------- 6

Conditions for Using PCMCIA 16bit Mode As the Only Option ----------------- 6

### Procedure for Installing Device Driver ---------------------------------------- 7

Note ------------------------------------------------------------------------------- 7

Installing Device Driver Under the Windows 95/98 System ---------------------- 7

Product Installation Procedure --------------------------------------------------10

Figure for Product Connections -------------------------------------------------12

###### Unplugging or Inserting the Interface cards During the Use of

###### Windows 95/98 -------------------------------------------------------------------- 13

#### Procedure for properly terminating system operation -------------------------- 13

Unplugging or Inserting the Interface Cards During the Use of Windows

95/98------------------------------------------------------------------------------13

Procedure of Supplying Power from the PC Card Slot Only --------------- 14

UN-INSTALLATION Procedure ------------------------------------------------ 15

Applications in Windows 2000

Installation device drive under Windows 2000 ------------------------------- 17

Confirm devices under Windows 2000 ----------------------------------------- 20

Unplugging the Interface Card During the Use of Windows 2000 -------- 22

Specifications ----------------------------------------------------------------- 24

Appendix: DVD region code ---------------------------------------- 25

## External Slim DVD User Manual

Product Introduction

Compatible Hardware Type:

Notebook computers equipped with Card Bus or PCMCIA Type II, III slots.  The computer should have 3.5-inch disk drive or external 3.5-inch disk drive for installation of driver software from the diskette.

Minimal System Requirements for Playing DVD Movies:

CPU: Celeron 366Mhz or above, or Pentium II 300Mhz or above, or AMD-K6-2 400Mhz or above.

MEMORY: 32MB or above.

HDD: At least 20MB Spare Space

Operation environment:

Windows 95 (OSR2 or later version)

Windows 98

Windows 2000

Windows ME

Compatible Disc Types:

- DVD-ROM (one-sided/two sided), DVD-Video*, DVD-R (3.95GB/4.7GB)

- CD-ROM (Mode 1), CD-ROM XA (Mode 2 Form 1, Form 2)

- CD-DA (Music CD), Video CD, CD-R, CD-RW*

- Photo CD (corresponding to Multi Session), CD-Extra, CD-Text

(DVD-RAM not usable)

*1. DVD playing software included with the product needs to be installed for playing DVD-Video.  Depending on the type of computer you use, not all images can be entirely displayed.

*2. When using the CD-RW, it sometimes can not be executed to its full capacity due to the differences in disc properties.

【Note】

- OSR2 (Ver 4.00 950B) or newer version is required for executing the “Card Bus Mode” under Windows 95.

- PCMCIA Type II Card Bus Slot (32bit) needs to be installed before the “Card Bus Mode” can be used.

- This DVD machines is not able to provide the sound effects of your DVD-ROM/CD-ROM games. Your PC must have a sound-generating function.

- You will not be able to hear the sound of music CD on your computer if the machine and the computer are only connected through the Card Bus Cable.  Earphone sets or power speakers need to be plugged into the earphone slot in order to generate sound.

Special features of the product:

- High-speed information transmission: DVD-ROM is 8x speed maximum, and CD-ROM is 32x maximum.

- The machine utilizes the ATA Interface Card which is compatible for both the Card Bus Slot (32bit) and PC Card Type II Slot (16bit).

- Under Card Bus Mode: High speed transmission corresponding to the Bus Master transmission protocol；32 bit Databus；33Mhz Clock.

- In 16 bit Mode: achieving high-speed transmission based on the Ultra SMIT transmission protocol.

- Slim, small, light, and very portable.  It also comes with a small and light AC Adapter.

- The machine can be turned on only by power supply from the Notebook PC’s PC Card Slot (only under 16 bit Mode).

- DVD playing software is included in the product package. (Please refer to the instruction in the software to procedure of executing such program).

Accessories:

- Card Bus/PCMCIA interface card X 1

- Interface card Cable X 1

- AC Adapter X 1

- 3.5-inch Device Driver Diskette X 1

- DVD Playing Software Disc X 1

- Instruction Manual X 1

※If you encounter any technical failure while using External Slim DVD and need repair service.  Please return the drive along with all the accessories to the nearest retailer or service site.

※Please refer to Acer CM Website :  http://www.acercm.com/

While you have any question in using External Slim DVD.

Or you could leave your valuable comments in “Question/Suggestion Feedback”.

Names and Functions of Product Components:

- AC Power Switch.

- AC Power Jack.

- Interface Connector.

- Head Phone Jack.

- Volume Control Knob

- Power Indicator

- Disc Tray

- Eject Button

- Forced Eject hole

- Busy Indicator

Applications in Windows 95/98

Before you start:

Confirm the Card Bus Driver in the Notebook PC:

- From “My Computer” select “Control Panel”, then choose “System”.

- Select “Device Manager”. Make sure that the name of the Controller in the PCMCIA slot category is Card Bus.

Confirm the Version of the Windows 95:

If the operation system in use is Windows 95, please follow the following procedure for version confirmation.

- From “My Computer” select “Control Panel”, then choose “System”.

- The number shown in “General” is the version of your Windows 95.

- If the version shown is “4.00.950B” or “4.00. 950C”, then the system belongs to a newer version than OSR2.  This means that the product can be used under the Card Bus 32-bit mode.

Requirements for Using the Card Bus 32bit Mode:

- The operation system belongs to a newer version than Windows 95 OSR2, or Windows 98, Windows 2000.

- Notebook PC with Card Bus Slots.

- Card Bus Driver is already installed on the computer.

Conditions for Using PCMCIA 16bit Mode As the Only Option:

- The operating system is Windows 95 “Ver.4.00.950”, “Ver.4.00.950a”.

- The computer does not, or is not able to, install the Card Bus Driver.

- No AC Adapter is used, the only source of power supply comes from the PC Card slot.

If the conditions occur, please change the Mode Switch on the Card Bus/PCMCIA interface card to 16 bit Mode.  Please make the switch by using a hard pointed tool.

Mode Switch

16 bit Mode                 Card Bus Mode  (Original setting)

Procedure for Installing Device Driver:

Note:

Under Windows 95/98, please do not insert the Card Bus/PCMCIA interface card into the Notebook PC slot before the Device Driver is completely installed.

Installing Device Driver Under the Windows 95/98 System:

- Start Windows 95/98.

- Insert the “Device Driver 3.5” disc into the disc drive.

- Select “My Computer”  “3.5-inch drive (A:)” “Win9X” “Ddsetup.exe”

- Click on “Ddsetup.exe”, select “Install”, and press “OK”.

- Make sure that your Notebook PC can support Card Bus 32 bit Mode (please refer the instruction for confirmation procedure). Select “Card Bus Mode” and press “OK”.

※If your Notebook PC can only support 16 bit Mode, please select the “16Bit PIO Mode”.

- Press “Enter” after installation is complete.

- After the installation of the Device Driver is done, next install the product.  Please refer the next section for procedure.

Product Installation Procedure:

Once the Device Driver is installed, please connect the machine with the accessories according to the following procedure:

- Turn on the Notebook PC, and start Windows.

- Connect the AC Adapter to the machine, and plug the other end to the power slot.

- Make sure that the power switch of the machine is at the “DC” position.

DC              PC

- Connect the Card Bus/PCMCIA interface card with Cable line.  (Please pay close attention to the direction of connection: The labeled side of the interface card should be facing upward, so should the marked side of the cable line, before the connection is made.

- Connect the other end of the Cable line to the machin’s Connector. （Pay attention to the direction of connection）.

- Insert the interface card into the Notebook PC’s PC Card slot. （At this time there will be sound coming from the Notebook PC, indicating the installation of a new hardware）.

- On some of the Notebook PC, the Wizard program for hardware addition will be executed at this time.  Please insert the Device Driver 3.5” disc once again, and follow the step-wise setting instruction provided by the Wizard.

- After completing the above connections and settings, please enter the “My Computer” selection and make sure that the DVD machine exists.

If you want to make sure that the interface card is in normal status, please double click “PCMCIA card display” in the status bar.

After the “slot status” is confirmed:

- At this point you may insert the CD-ROM disc or the DVD movie into the machine. (The accessory DVD playing software has to be installed before viewing DVD films.)

Figure for Product Connections:

Unplugging or Inserting the Interface Cards During the Use of  Windows 95/98:

Procedure for properly terminating system operation:

- Turn off the computer’s operating system, and shut down the power supply to the computer.

- Turn off the DVD machine’s power supply.

- Remove the interface card from the computer’s PC Card slot.

Unplugging or Inserting the Interface Cards During the Use of Windows 95/98:

The interface card can be unplugged from or inserted into the computer even when the power supply is not off and the Windows 95/98 is still in use.  Please refer to the following procedure for such operation.  If the interface card is removed without following the proper procedure, unexpected system errors might occur.

- Move the mouse cursor to the “PCMCIA card” icon in the status menu, click on the left button.

- The following message will then appear.  Click on this message with the left button once again.

Stop CI-iCN Series DuoATA Card (CardBus Mode)

3. This will appear next:

You may remove the interface card after pressing “enter”.

※Another procedure for unplugging the interface card: “My computer”  “ Control Panel” ”PC Card (PCMCIA)”.

Press “Stop”, then remove the interface card according to the instruction.

Procedure of Supplying Power from the PC Card Slot Only:

- The DVD machine does not use the AC Adapter.  It can be turned on by the power supply from the Notebook PC.

- Please confirm that the Mode Switch on the Card Bus/PCMCIA interface card has been set to 16 bit Mode.

Please use hard pointed tool to make the switch.

Mode Switch

16 bit Mode                  Card Bus Mode  (original setting)

No execution if it’s not in the “Card Bus Mode”.

- Transmission speed will be hampered due to the source of power supply (Notebook PC).

DVD-ROM:  2,700 KB/sec maximum （2X）

CD-ROM: 1,350 KB/sec maximum（9X）

- Connection procedure:

After completing the setting on the Notebook PC, connect the interface card to the cable line.  One end should be connected to the Connector, while the other end is plugged into the PC Card slot.

- On some of the Notebook PC, the Wizard program for hardware addition will be executed at this time.  Please insert the Device Driver 3.5” disc once again, and choose the folder in 「A:\WIN9X\16BIT」then step-wise setting instruction provided by the Wizard.

Under “My Computer”, make sure that the DVD machine has already been recognized by the computer.

UN-INSTALLATION Procedure:

Follow the instruction below to completely un-install the Card Bus driver already installed on the Notebook PC:

1. Start Windows 95/98.

2. Insert the Device Driver 3.5” disc into the 3.5” disc drive.

- Select “My Computer”  “3.5-inch drive (A:)” “Win9X” “Ddsetup.exe”

- Select “Uninstall”, and press “OK”.

- The following screen will appear, press “OK” to continue.

- After Uninstall is complete, press “OK”.

- The system will then request a re-start.  Press “Yes”.

Applications in Windows 2000

Installation device driver under Windows 2000:

      - Start Windows 2000 OS.

      - Insert the “Device Driver 3.5” disc into the disc drive.

      - Connect all product devices, including DVD drive, interface Card & Cable and AC Adapter.

      - Switch on DC Power.

      - Plug PC Card into PCMCIA slot of Notebook PC.

      - Then the hardware wizard will be initialed, please press “Next”.

      - In the next screen, please select “Search for a suitable driver for my device”, then press “Next”.

      - In the “Locate driver files” screen, please select “Specify a location”, then press “Next”.

      - Make sure the device driver disk is in the floppy drive. Click “Browse” and choose file location at “A:\WIN2000”, then press “OK”.

      - The wizard will find driver for this device at “a:\win2000\duo32w2k.inf”, then click “Next”.

      - In the next screen, please press “Yes” to continue the installation.

      - After Windows finish the installation, please press “Finish” to close the wizard.

      - Remove the device driver disk from floppy drive.

      - The system will then request a re-start.  Press “Yes”.

Confirm devices under Windows 2000:

            - Start Windows 2000.

            - Move the mouse cursor to the “PCMCIA card” icon in the status menu, and then double click the icon.

            - Then the screen will show:

Make sure the hardware device shows “CI-iCN Series DuoATA

Card”.

            - Press “Close” to finish this window.

            - After above steps, please enter the “My Computer” selection and make sure that the DVD machine exists.

Unplugging the Interface Card During the Use of Windows 2000:

Method 1:

- Move the mouse cursor to the “PCMCIA card” icon in the status menu, click on the left button.

- The following message will then appear.  Click on this message with the left button once again.

Stop CI-iCN Series DuoATA Card (CardBus Mode)

- This will appear next:

You may remove the interface card after pressing “enter”.

Method 2:

- Double click the “PCMCIA card” icon in the status menu.

- This will appear next:

- Choose “CI-iCN Series DuoATA Card (CardBus Mode), then press “Stop”.

- Then this screen will appear next, and press “OK”.

- You may remove the interface card after pressing “enter”.

Specifications:

| # Data Transfer Rate | DVD-ROM: 4,725~10,800 KB/sec (8X Max)* CD-ROM: 2,070~4,800KB/sec (32X Max)* |
| --- | --- |
| Access time | DVD-ROM: 150 msec typical CD-ROM: 110 msec typical |
| Seek time | DVD-ROM: 130 msec typical CD-ROM: 100 msec typical |
| Disc Formats | DVD-ROM (Single/Dual layer) DVD-Video DVD-R (3.95GB/4.7GB) CD-ROM (Mode 1) CD-ROM x A (Mode 2 Form1, Form2) CD-DA (Music CD) Video CD CD-R/RW Photo CD CD-Extra CD-TEXT |
| Interface | CARDBUS/PCMCIA |
| Physical Characteristics | Dimension: W150mm x D138mm x H20mm Net Weight: 480g |
| Buffer size | 512 Kbytes |
| Power Supply | Input : DC 5V (From AC Adapter) Or power supplied only through CardBus/PCMCIA interface from Notebook Computer (Only in 16bit mode) |
| Accessories | - Card Bus/PCMCIA Interface Card - Interface Cable - AC Adapter - DVD Software Player - 3.5” Device Driver Disk - User Manual |
| OS Compatibility | Window 95/98/2000, Windows ME |

Appendix: DVD region code:

The world according to DVD titles is set up into regions. There are 6 regions around the world. (Please see map below).

This DVD drive can be changed to play a DVD from any region in the world but can only be changed 5 times. This restriction makes the drive adaptable for use in any region while still maintaining compliance with RPC-II regulations.

The ability to switch from one region to another is not an ongoing feature.  Please set the drive to your local region only or you may risk setting the drive permanently for a region that is not compliant with the media available in your part of the world.

| Region Code | Area |
| --- | --- |
| 1 | North America, Canada |
| 2 | Europe, Middle East, Japan, South .Africa |
| 3 | South Eastern Asia, Taiwan, Hong Kong, Korea |
| 4 | Australia, New Zealand, Latin America |
| 5 | Africa, Russia, India, Pakistan |