# Upgrade FW Utility Quick Using Guide

- Install PCMCIA SW and insert PC Card.

- Un-zip file “FW136”, and “WinUpdate-0-5-1-0

- Setup “winupdate-0-5-1-0” in your desktop PC of Notebook.

- Execute “WinUpdate” from windows “Start -> Program -> PRISM Test utilities”.

- Add files -> “37300p04.pda”, “pk010007.hex”, “sf010306.hex”.

- Click “Update”.

- Click “Update” again.

- It will start process the flash update.

- Finally you will get a message “Update Successful”.