Copyright

Copyright © 2013 YEALINK NETWORK TECHNOLOGY

Copyright © 2013 Yealink Network Technology CO., LTD. All rights reserved. No parts of this publication may be reproduced or transmitted in any form or by any means, electronic or mechanical, photocopying, recording, or otherwise, for any purpose, without the express written permission of Yealink Network Technology CO., LTD. Under the law, reproducing includes translating into another language or format.

When this publication is made available on media, Yealink Network Technology CO., LTD. gives its consent to downloading and printing copies of the content provided in this file only for private use and not for redistribution. No parts of this publication may be subject to alteration, modification or commercial use. Yealink Network Technology CO., LTD. will not be liable for any damages arising from use of an illegally modified or altered publication.

Warranty

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS GUIDE ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS GUIDE ARE BELIEVED TO BE ACCURATE BUT ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF PRODUCTS.

YEALINK NETWORK TECHNOLOGY CO., LTD. MAKES NO WARRANTY OF ANY KIND WITH REGARD TO THIS GUIDE, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. Yealink Network Technology CO., LTD. shall not be liable for errors contained herein nor for incidental or consequential damages in connection with the furnishing, performance, or use of this guide.

Declaration of Conformity

|  | Hereby, Yealink Network Technology CO., LTD. declares that this phone is in conformity with the essential requirements and other relevant provisions of the CE, FCC. |
| --- | --- |

CE Mark Warning

This device is marked with the CE mark in compliance with EC Directives 2006/95/EC and 2004/108/EC.

Part 15 FCC Rules

This device is compliant with Part 15 of the FCC Rules. Operation is subject to the following two conditions:

1.  This device may not cause harmful interference, and

2.  This device must accept any interference received, including interference that may cause undesired

operation.

Class B Digital Device or Peripheral

Note: This device is tested and complies with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses, and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

1.  Reorient or relocate the receiving antenna.

2.  Increase the separation between the equipment and receiver.

3.  Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

4.  Consult the dealer or an experience radio/TV technician for help.

WEEE Warning

|  | To avoid the potential effects on the environment and human health as a result of the presence of hazardous substances in electrical and electronic equipment, end users of electrical and electronic equipment should understand the meaning of the crossed-out wheeled bin symbol. Do not dispose of WEEE as unsorted municipal waste and have to collect such WEEE separately. |
| --- | --- |

GNU GPL INFORMATION

Yealink SIP-T26P firmware contains third-party software under the GNU General Public License (GPL). Yealink uses software under the specific terms of the GPL. Please refer to the GPL for the exact terms and conditions of the license.

The original GPL license, source code of components licensed under GPL and used in Yealink products can be downloaded from Yealink web site: http://www.yealink.com/GPLOpenSource.aspx?BaseInfoCateId=293&NewsCateId=293&CateId=293.

About This Guide

Thank you for choosing the SIP-T26P IP phone, an exquisitely designed SIP IP phone. This unit provides business telephony features such as Call Hold, Call Transfer, Busy Lamp Field, Multicast Paging and Conference over an IP network.

This guide provides everything you need to quickly use your new phone. Be sure to verify with your system administrator that your network is prepared for configuring your IP phone. As well, be sure to read the Packaging Contents and Regulatory Notices sections in this guide before you set up and use the SIP-T26P IP phone.

If this is your first time using the SIP-T26P IP phone, we recommend that you first refer to the Quick Installation Guide and Quick Reference Guide, which are available at: http://www.yealink.com/DocumentDownload.aspx?CateId=142&flag=142.

The Shared Line, Busy Lamp Field List, Network Directory and Network Call Log features are designed for BroadWorks environment. Please refer to BroadWorks documents or contact your system administrator for more information.

Note

In This Guide

The topics provided in this guide include:

- Chapter 1  Overview

- Chapter 2  Getting Started

- Chapter 3  Customizing Your Phone

- Chapter 4  Basic Call Features

- Chapter 5  Advanced Phone Features

Summary of Changes

This section describes the changes to this guide for each release and guide version.

Changes for Release 71, Guide Version 71.110

The following sections are new:

- Recent Call In Dialing on page 69

Major updates have occurred to the following sections:

- Basic Network Settings on page 14

- Keypad Lock on page 22

- Contact Management on page 28

- DSS Keys on page 46

Changes for Release 70, Guide Version 70

Major updates have occurred to the following sections:

- Keypad Lock on page 22

- Volume on page 25

- Ring Tones on page 26

- Call Completion on page 72

- DSS Keys on page 46

- Do Not Disturb (DND) on page 75

- Call Forward on page 77

- Call Pickup on page 89

- Busy Lamp Field (BLF) on page 95

Table of Contents

About This Guide	v

In This Guide	v

Summary of Changes	v

Changes for Release 71, Guide Version 71.110	v

Changes for Release 70, Guide Version 70	vi

Table of Contents	vii

Overview	1

Hardware Component Instructions	1

Icon Instructions	3

LED Instructions	5

User Interfaces	5

Phone User Interface	6

Web User Interface	6

Documentations	8

Getting Started	9

Packaging Contents	9

Phone Installation	11

Phone Initialization	13

Phone Status	13

Basic Network Settings	14

Registration	15

Idle Screen	16

Customizing Your Phone	17

General Settings	17

Backlight	17

Language	18

Time & Date	19

Administrator Password	21

Key as Send	22

Keypad Lock	22

Audio Settings	25

Volume	25

Ring Tones	26

Contact Management	28

Directory	28

Local Directory	29

Blacklist	39

Remote Phone Book	39

Call History Management	42

System Customizations	44

Logo Customization	44

Headset Use	44

DSS Keys	46

Account Management	55

Dial Plan	56

Emergency Number	62

Live Dialpad	63

Hot Line	64

Basic Call Features	65

Placing Calls	65

Answering Calls	67

Ending Calls	68

Redialing Numbers	69

Recent Call In Dialing	69

Auto Answer	70

Auto Redial	71

Call Completion	72

Call Return	73

Call Mute	73

Call Hold/Resume	74

Do Not Disturb (DND)	75

Call Forward	77

Call Transfer	83

Call Waiting	84

Conference	85

Local Conference	85

Network Conference	86

Call Park	88

Call Pickup	89

Anonymous Call	93

Anonymous Call Rejection	94

Advanced Phone Features	95

Busy Lamp Field (BLF)	95

Call Recording	98

Hot Desking	100

Intercom	101

Outgoing Intercom Calls	102

Incoming Intercom Calls	102

Multicast Paging	104

Sending RTP Stream	104

Receiving RTP Stream	106

Music on Hold	107

Automatic Call Distribution (ACD)	108

Messages	110

Short Message Service (SMS)	110

Voice Mail	112

Message Waiting Indicator (MWI)	114

Troubleshooting	117

Regulatory Notices	127

Service Agreements	127

Limitations of Liability	127

Safety Instructions	127

Appendix A - Time Zones	129

Index	131

Overview

This chapter provides the overview of the SIP-T26P IP phone. The topics include:

- Hardware Component Instructions

- Icon Instructions

- LED Instructions

- User Interfaces

- Documentations

If you require additional information or assistance with your new phone, contact your system administrator.

Hardware Component Instructions

The main hardware components of the SIP-T26P IP phone are the LCD screen and the keypad.

Hardware component instructions of the SIP-T26P IP phone are:

|  | Item | Description |
| --- | --- | --- |
| ① | LCD Screen | Shows information about calls, messages, soft keys, time, date and other relevant data: • Call information — caller ID, call duration • Icons (for example, ) • Missed call text or second incoming caller information • Prompt text (for example, “Saving config files!”) • Time and date |
| ② | Power Indicator LED | Indicates phone power status and phone status. |
| ③ | Line Keys | Use these keys to activate up to three accounts and assign various features. |
| ④ | Memory Keys | Use these keys to assign various features. |
| ⑤ | MESSAGE Key | Indicates and accesses voice mails. |
| ⑥ | HEADSET Key | Toggles and indicates the headset mode. |
| ⑦ | HOLD Key | Places a call on hold or resumes a held call. |
| ⑧ | TRAN Key | Transfers a call to another party. |
| ⑨ | MUTE Key | Mutes or un-mutes an active call. |
| ⑩ | Speakerphone Key | Toggles the hands-free speakerphone mode. |
| ⑪ | RD Key | Redials a previously dialed number. |
| ⑫ | CONF Key | Conducts a conference call with multiple other parties. |
| ⑬ | Volume Key | Adjusts the volume of the handset, headset, speaker, and ringer. |
| ⑭ | Keypad | Provides the digits, letters, and special characters in context-sensitive applications. |
| ⑮ |  | Scroll through the displayed information. |
|  |  | Confirms actions or answers incoming calls. |
|  |  | Cancels actions or rejects incoming calls. |
| ⑯ | Soft Keys | Label automatically to identity their context-sensitive features. |

Icon Instructions

Icons appear on the phone LCD screen are described in the following table:

| Icon | Description |
| --- | --- |
|  | Network is unavailable |
|  | Private line registers successfully |
|  | Registration failed |
|  | Registering |
|  | Hands-free speakerphone mode |
|  | Handset mode |
|  | Headset mode |
|  | Multi-lingual lowercase letters input mode |
|  | Multi-lingual uppercase letters input mode |
|  | Alphanumeric input mode |
|  | Numeric input mode |
|  | Voice Mail |
|  | Text Message |
|  | Auto Answer |
|  | Do Not Disturb |
|  | Call Forward/Forward Calls |
|  | Call Hold |
|  | Call Mute |
|  | Ringer volume is 0 |
|  | Keypad Lock |
|  | Received Calls |
|  | Placed Calls |
|  | Missed Calls |
|  | Recording box is full |
|  | A call cannot be recorded |
|  | Recording starts successfully |
|  | Recording cannot be started |
|  | Recording cannot be stopped |

LED Instructions

Power Indicator LED

| LED Status | Description |
| --- | --- |
| Solid green | The phone is powered on. |
| Fast Flashing green | The phone is ringing (Hardware version must be 4.0.0.2 or higher). |
| Slow flashing green | The phone receives a voice mail (Hardware version must be 4.0.0.2 or higher). |
| Off | The phone is powered off. |

Line key LED

| LED Status | Description |
| --- | --- |
| Solid green | The line is in conversation. The line is seized. |
| Fast flashing green | The line receives an incoming call. |
| Slow flashing green | The call is placed on hold. |
| Off | The line is inactive. |

Line key LED (configured as BLF key)

| LED Status | Description |
| --- | --- |
| Solid green | The monitored user is idle. |
| Fast flashing green | The monitored user receives an incoming call. |
| Slow flashing green | The monitored user is busy. |
| Off | The monitored user does not exist. |

Memory key LED (configured as BLF key)

| LED Status | Description |
| --- | --- |
| Solid green | The monitored user is idle. |
| Flashing red | The monitored user receives an incoming call. |
| Solid red | The monitored user is busy. |
| Off | The monitored user does not exist. |

User Interfaces

There are two ways to customize specific configurations on your SIP-T26P IP phone:

- Using the user interface on the IP phone.

- Using the user interface in an internet browser window from your PC.

The hardware components keypad and LCD screen constitute the phone user interface, which allows the user to execute all call operation tasks and basic configuration changes directly on the phone. In addition, you can use the web user interface to access all configuration settings. In many instances, it is possible to use both the phone user interface and web user interface to operate the phone and change settings. However, in some instances, it is only possible to use the phone user interface or the web user interface.

Phone User Interface

You can customize your phone by pressing the Menu soft key to access the phone user interface. The Advanced Settings options are only accessible to an administrator, and the default administrator password is admin (case-sensitive). For more information on customizing your phone using the available options from the phone user interface, refer to Customizing Your Phone on page 17.

Web User Interface

In addition to the phone user interface, you can also customize your phone via web user interface. In order to access the web user interface, you need to know the IP address of your new phone. To obtain the IP address, press the OK key on the phone. Enter the IP address (e.g., http://192.168.0.10 or 192.168.0.10) in the address bar of web browser on your PC. The default administrator user name and password are both admin (case-sensitive).

The options you can use to customize the IP phone via phone user interface and/or via web user interface are listed in the following table:

| Options | Phone User Interface | Web User Interface |
| --- | --- | --- |
| Status | √ | √ |
| --IPv4 |  |  |
| --MAC |  |  |
| --Firmware |  |  |
| --Network |  |  |
| --Phone |  |  |
| --Accounts |  |  |
| Basic Call Features |  | √ |
| --Auto Answer | √ |  |
| --Auto Redial | √ |  |
| --Call Completion | √ |  |
| --Call Forward | √ |  |
| --Call Waiting | √ |  |
| --Call Park | √ |  |
| --Call Pickup | √ |  |
| --Call Return | √ |  |
| --Conference |  |  |
| --Anonymous Call | √ |  |
| --Anonymous Call Rejection | √ |  |
| --DND | √ |  |
| Basic Phone Settings |  |  |
| --Language | √ | √ |
| --Time & Date | √ | √ |
| --Administrator Password | √ | √ |
| --Ring Tones | √ | √ |
| --Backlight |  | √ |
| --Keypad Lock | √ | √ |
| --Phone Volume | √ |  |
| --Logo Customization |  | √ |
| --Call History Management | √ | √ |
| --Contact Management |  |  |
| --Directory |  | √ |
| --Local Directory | √ | √ |
| --Blacklist | √ | √ |
| --Remote Phone Book |  | √ |
| --Dial Plan |  | √ |
| --DSS Keys | √ | √ |
| --Key as Send | √ | √ |
| --Hot Line | √ | √ |
| --Live Dialpad |  | √ |
| --Emergency Call |  | √ |
| Advanced Phone Features |  | √ |
| --BLF | √ |  |
| --Call Recording | √ |  |
| --Hot Desking | √ |  |
| --Intercom | √ |  |
| --Multicast Paging |  |  |
| --Music on Hold |  |  |
| --ACD | √ |  |
| --Messages | √ |  |
| SIP Account |  | √ |
| --User Options |  |  |
| --Register Status | √ |  |
| --Activation | √ |  |
| --Label | √ |  |
| --Display Name | √ |  |
| --Register Name | √ |  |
| --User Name | √ |  |
| --Password | √ |  |
| --SIP Server1/2 | √ |  |
| --Server Option |  |  |
| --Registrar Port |  |  |
| --Outbound Status | √ |  |
| --Outbound Proxy | √ |  |
| --NAT Traversal |  |  |
| --STUN Status | √ |  |
| --STUN Server | √ |  |

The table above lists most of the feature options. Please refer to the relevant sections for more information.

Note

Documentations

The following table shows documentations available for the SIP-T26P IP phone.

| Name | Contents | Where found | Format/Language |
| --- | --- | --- | --- |
| Quick Installation Guide | Basic set up of the phone | In the package | PDF/English |
| Quick Reference Guide | Basic call features and phone customizations | In the package | PDF/English |
| User Guide | Phone/ Web user interface settings Basic call features and advanced phone features | CD attached in the package | PDF/English |

You can also download the latest documentations from website:

http://www.yealink.com/DocumentDownload.aspx?CateId=142&flag=142.

Note

Getting Started

This chapter provides basic installation instructions and information for obtaining the best performance with the SIP-T26P IP phone. The topics include:

- Packaging Contents

- Phone Installation

- Phone Initialization

- Phone Status

- Basic Network Settings

- Registration

- Idle Screen

If you require additional information or assistance with your new phone, contact your system administrator.

Packaging Contents

The following components are included in your SIP-T26P IP phone package:

- SIP-T26P IP phone

- Phone Stand

- Power Adapter

- Handset & Handset Cord

- Ethernet Cable

- Quick Installation Guide & Quick Reference Guide

- CD-ROM

Check the list before installation. If you find anything missing, contact your system administrator.

Phone Installation

If your phone is already installed, proceed to Phone Initialization on page 13.

This section introduces how to install the phone with the components in the packaging contents:

- Attach the stand

- Connect the handset and optional headset

- Connect the network and power

1)   Attach the stand

2)   Connect the handset and optional headset

A headset is not provided in the packaging contents. Contact your system administrator for more information.

Note

3)   Connect the network and power

You have two options for power and network connections. Your system administrator will advise you which one to use.

- AC power

- Power over Ethernet (PoE)

AC Power

To connect the AC power:

- Connect the DC plug on the power adapter to the DC5V port on the phone and connect the other end of the power adapter into an electrical power outlet.

- Connect the supplied Ethernet cable between the Internet port on the phone and the Internet port in your network or switch/hub device port.

Power over Ethernet

Using a regular Ethernet cable, the SIP-T26P IP phone can be powered from a PoE (IEEE 802.3af) compliant switch or hub.

To connect the PoE:

- Connect the Ethernet cable between the Internet port on the phone and an available port on the in-line power switch/hub.

If in-line power is provided, you don’t need to connect the AC adapter. Make sure the Ethernet cable and switch/hub is PoE compliant.

The phone can also share the network with other network devices such as a PC (personal computer). It is an optional connection.

Important! Do not unplug or remove power to the phone while it is updating firmware and configurations.

Note

Phone Initialization

After your phone has been powered on, the system boots up and performs the following steps:

Automatic Phone Initialization

The phone finishes the initialization process by loading the saved configuration. The phone LCD screen will display “Initializing, Please wait…” during the initialization.

DHCP (Dynamic Host Configuration Protocol)

By default the phone attempts to contact a DHCP server in your network to obtain valid IPv4 and IPv6 network settings, e.g., IP address, subnet mask, default gateway address and DNS address.

If your network does not use DHCP, proceed to Basic Network Settings on page 14.

Note

Phone Status

You can view the system status of your phone via phone user interface or web user interface.

Available information of phone status includes:

- Network status including IPv4 status, IPv6 status, IP mode, MAC address, LAN type, LAN IP and LAN mask.

- Phone status including product name, hardware version, firmware version, product ID, MAC address and device certificate status (including factory, installed and not installed).

- Account status indicating the register status of SIP accounts.

To view the phone status via phone user interface:

- Press     , or press Menu->Status.

- Press     or     to scroll through the list and view the specific information.

To view the phone status via web user interface:

- Open the web browser of your computer.

- Enter the IP address in the browser’s address bar, and then press Enter.

- Enter the user name (admin) and password (admin) in the login page.

- Click Confirm to log in.

- The phone status displays on the first page of the web user interface.

Basic Network Settings

If your phone cannot contact a DHCP server for any reason, you need to configure network settings manually. IP phones support to use the IPv4 address only, the IPv6 address only or both IPv4 and IPv6 addresses.

To configure the IP address mode via phone user interface:

- Press Menu->Settings->Advanced Settings (password: admin) ->Network->WAN Port.

- Press     or     to select IPv4, IPv6 or IPv4 & IPv6 from the IP Mode field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

To configure a static IPv4 address via phone user interface:

- Press Menu->Settings->Advanced Settings (password: admin) ->Network->WAN Port->IPv4->Static IP Client.

- Enter the desired values in the IPv4, Subnet Mask, Default Gateway, Pri DNS and Sec DNS fields respectively.

- Press the Save soft key to accept the change or the Back soft key to cancel.

To configure a static IPv6 address via phone user interface:

- Press Menu->Settings->Advanced Settings (password: admin) ->Network->WAN Port->IPv6->Static IPv6 Client.

- Enter the desired values in the IPv6 IP, IPv6 IP Prefix, IPv6 Gateway, IPv6 Pri.DNS and IPv6 Sec.DNS fields respectively.

- Press the Save soft key to accept the change or the Back soft key to cancel.

If you are using an xDSL modem, you can connect your phone to the Internet via PPPoE mode. You can set the WAN port to be a PPPoE port. The PPPoE port will perform a PPP negotiation to obtain the IP address. Contact your system administrator for the PPPoE user name and password.

To configure PPPoE via phone user interface:

- Press Menu->Settings->Advanced Settings (password: admin) ->Network->WAN Port->IPv4->PPPoE IP Client.

- Enter the user name and password in the corresponding fields.

- Press the Save soft key to accept the change or the Back soft key to cancel.

Using the wrong network parameters may result in inaccessibility of your phone and may also have an impact on your network performance. For more information on these parameters, contact your system administrator.

Note

Registration

Generally, your phone will be deployed with multiple other phones. In this case, your system administrator will configure the phone parameters beforehand, so that after you start up your phone, the phone will be registered and ready to use. If your phone is not registered, you may have to register it. For more information on how to register your phone, refer to Account Management on page 55.

Idle Screen

If the phone has successfully started up, the idle screen is shown as below:

The idle screen shows the label of current account, time and date, and four soft keys.

Customizing Your Phone

You can customize your SIP-T26P IP phone by configuring the backlight, time & date, ring tones and so on. You can add contacts to the phone’s local directory manually or from call history. You can handle incoming calls from different contacts in different ways.

This chapter provides basic operating instructions for customizing your phone. The topics include:

- General Settings

- Audio Settings

- Contact Management

- Call History Management

- System Customizations

If you require additional information or assistance with your new phone, contact your system administrator.

General Settings

Backlight

You can configure the backlight status on the LCD screen as the following types:

- Always Off: Backlight is turned off permanently.

- Always On: Backlight is on permanently.

- 15, 30, 60, 120: Backlight is turned off when the phone is inactive after the specified time (in seconds).

To configure the backlight via web user interface:

- Click on Settings->Preference.

- Select the desired type from the pull-down list of Backlight Time (seconds).

- Click Confirm to accept the change.

You can configure the backlight via web user interface only.

Note

Language

The default language of the phone user interface is English. The phone will detect and use the same language as which of your internet browser for the web user interface. If the language of your internet browser is not supported by the phone, the web user interface will use English. You can change the language for the phone user interface and the web user interface respectively.

Not all of the support languages are available for selection. The available languages depend on the language packs currently loaded to the IP phone. Please contact your system administrator for more information on the available languages of your new phone.

Note

To change the language for the phone user interface:

- 1.  Press Menu->Settings->Basic Settings->Language.

- 2.  Press     or     to select the desired language.

- 3.  Press the Save soft key to accept the change.

Text appearing on the LCD screen will use the selected language.

To change the language for the web user interface:

- Click on Settings->Preference.

- Select the desired language from the pull-down list of Language.

- Click Confirm to accept the change.

- Text displaying on the web user interface will use the selected language.

Time & Date

The time and date display on the LCD screen when the phone is idle. If the phone cannot obtain the time and date from the Simple Network Time Protocol (SNTP) server, you need to configure the time and date manually. For more information on the SNTP server, contact your system administrator.

To configure the SNTP settings via phone user interface:

- Press Menu->Settings->Basic Settings->Time & Date->SNTP Settings.

- Press     or     , or the Switch soft key to select the time zone that applies to your area from the Time Zone field.

- The default time zone is "+8 China(Beijing)".

- Enter the domain names or IP addresses in the NTP Server1 and NTP Server2 fields respectively.

- Press     or     , or the Switch soft key to select the desired value from the Daylight Saving field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

Please refer to Appendix A - Time Zones for the list of available time zones on the IP phone.

Note

To configure the time and date manually via phone user interface:

- Press Menu->Settings->Basic Settings->Time & Date->Manual Settings.

- Enter the specific date in the Date field.

- Enter the specific time in the Time field.

- Press the Save soft key to accept the change.

- The date and time displaying on the LCD screen will change accordingly.

To configure the time and date format via phone user interface:

- Press Menu->Settings->Basic Settings->Time & Date->Time & Date Format.

- Press     or     , or the Switch soft key to select the desired time format from the Clock field.

- Press     or     , or the Switch soft key to select the desired date format from the Date Format field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

There are 7 available date formats. For example, the date format is “WWW DD MMM”, “WWW” represents the abbreviation of the week, “DD” represents the two-digit day, and “MMM” represents the first three letters of the month.

The date formats you need to know:

| Date Format | Example(2013-1-16) |
| --- | --- |
| WWW MMM DD | Wed Jan 16 |
| DD-MMM-YY | 16-Jan-13 |
| YYYY-MM-DD | 2013-01-16 |
| DD/MM/YYYY | 16/01/2013 |
| MM/DD/YY | 01/16/13 |
| DD MMM YYYY | 16 Jan 2013 |
| WWW DD MMM | Wed 16 Jan |

You can also configure the time and date via web user interface at the path Settings->Time & Date.

Administrator Password

The Advanced Settings option is only accessible to an administrator. The default administrator password is admin. For security reasons, you should change the default administrator password as soon as possible.

To change the administrator password via phone user interface:

- Press Menu->Settings->Advanced Settings (password: admin) ->Set Password.

- Enter the old password in the Current PWD field.

- Enter the new password in the New PWD field.

- Enter the new password again in the Confirm PWD field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure the administrator password via web user interface at the path Security->Password.

Key as Send

You can set the “#” or “*” to perform as a send key while dialing.

To configure the key as send feature via phone user interface:

- Press Menu->Features->Key as Send.

- Press     or     , or the Switch soft key to select # or * from the Key as Send field, or select Disable to disable this feature.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure key as send via web user interface at the path Features->General Information.

Keypad Lock

You can lock the keypad of your phone temporarily when you are not using it. This feature helps you to protect your phone from unauthorized use.

Keypad Lock consists of the following:

Menu Key:     The Menu soft key and MESSAGE key are locked. You cannot access the menu of the phone until unlocked.

Function Keys:  The function keys are locked. You cannot use the MESSAGE, OK, HOLD, MUTE, TRAN, RD, CONF, X, navigation keys, line keys, memory keys and the soft keys until unlocked.

All Keys:        All keys are locked except the Volume key. You are only allowed to dial emergency numbers, reject incoming calls by pressing the X key, answer incoming calls by lifting the handset, pressing the Speakerphone key, the HEADSET key or the OK key, place an active call on hold by pressing the Hold soft key or the HOLD key and end the call by pressing the X key.

The emergency number setting, if desired, must be made before lock activation. For more information, refer to Emergency Number on page 62.

Note

To activate the keypad lock via phone user interface:

- Press Menu->Settings->Advanced Settings (password: admin) ->Phone Settings->Keypad Lock.

- Press     or     , or the Switch soft key to select the desired type from the Keypad Lock field.

- Press the Save soft key to accept the change.

- Long press      to lock the keypad immediately when the phone is idle.

- The LCD screen prompts “Keypad Locked!” and displays the icon    .

You can specify the interval (in seconds) to automatically lock the keypad instead of long pressing       .

To configure the interval of automatic keypad lock via web user interface:

- Click on Features->Phone Lock.

- Enter the desired interval in the Phone Lock Time Out (0~3600s) field.

- Click Confirm to accept the change.

The default interval is 0 second, that is, you can long press       to lock the keypad only.

You can configure the interval of automatic keypad lock via web user interface only.

Note

To unlock the keypad, you must know the keypad unlock PIN of the phone. The default keypad unlock PIN is 123.

To change the keypad unlock PIN via phone user interface:

- Press Menu->Settings->Basic Settings->Phone Unlock PIN.

- Enter the old PIN in the Current PIN field.

- Enter the new PIN in the New PIN field.

- Enter the new PIN again in the Confirm PIN field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

The unlock PIN length must be within 15 characters.

Note

To unlock the keypad via phone user interface:

- Press any locked key, the LCD screen prompts “Please Enter PIN”.

- Enter the PIN in the PIN field.

- Press the Confirm soft key to unlock the keypad.

The    icon disappears from the LCD screen.

You can long press       or wait for a period of time (if configured) to lock the keypad again.

You can also unlock the keypad by administrator password. When you enter the administrator password to unlock the keypad, the phone will then turn to the Reset Phone PIN interface.

Note

To deactivate the keypad lock via phone user interface:

- Press Menu->Settings->Advanced Settings (password: admin) ->Phone Settings->Keypad Lock.

- Press     or     , or the Switch soft key to select Disable from the Keypad Lock field.

- Press the Save soft key to accept the change.

You can also activate or deactivate the keypad lock via web user interface at the path Features->Phone Lock.

Audio Settings

Volume

You can press the Volume key to adjust the ringer volume when the phone is idle. You can also press the Volume key to adjust the receiver volume of currently used audio devices (handset, speakerphone or headset) when the phone is during a call.

To adjust the volume when the phone is idle:

- Press               to adjust the ringer volume.

To adjust the volume when the phone is during a call:

- Press               to adjust the volume of currently used audio device (handset, speakerphone or headset).

If the ringer volume is adjusted to minimum, the    icon will appear on the LCD screen.

Note

Ring Tones

Ring tones are used to indicate incoming calls. You can select different ring tones to distinguish the different accounts registered on your phone, or to distinguish your phone from your neighbor’s.

To select a ring tone for the phone via phone user interface:

- Press Menu->Settings->Basic Settings->Ring Tones.

- Press     or     to select the desired ring tone.

- Press the Save soft key to accept the change or the Back soft key to cancel.

To select a ring tone for the account via web user interface:

- Click on Account.

- Select the desired account from the pull-down list of Account.

- Click on Basic.

- Select the desired ring tone from the pull-down list of Ring Type.

- If Common is selected, this account will use the ring tone selected for the phone at the path of the web user interface Settings->Preference. Refer to the above instruction.

- Click Confirm to accept the change.

You can select a ring tone for the account via web user interface only.

Note

To upload custom ring tones for your phone via web user interface:

- Click on Settings->Preference.

- Click Browse to locate a ring tone (file format must be *.wav) file from your local system.

- Click Upload to upload the file.

The ring tone for an incoming call on the phone may be different. For example, when the phone receives an incoming call from a contact stored in the local directory, it will play the ring tone assigned to the contact in the contact directory (refer to Adding Contacts in the Contact Management section). Otherwise, the phone will play the ring tone assigned to the account. If both the contact ring tone and the account ring tone are not assigned, then the phone will play the ring tone assigned to the phone.

All custom ring tone files must be within 100KB. You can upload custom ring tones for your phone via web user interface only.

Note

Contact Management

This section provides the operating instructions for managing contacts. The topics include:

- Directory

- Local Directory

- Blacklist

- Remote Phone Book

Directory

Directory provides easy access to the lists you use frequently. The lists may contain Local Directory, History, Remote Phone Book and LDAP.

To configure the directory via web user interface:

- Click on Directory->Setting.

- In the Directory block, select the desired list from the Disabled column and then click     .

- The selected list appears in the Enabled column.

- Repeat the step 2 to add more lists to the Enabled column.

- To remove a list from the Enabled column, select the desired list and then click     .

- To adjust the display order of list, select the desired list and then click     or     .

- The phone LCD screen displays the list(s) in the adjusted order.

- Click Confirm to accept the change.

You can configure the directory via web user interface only.

Note

To check the directory via phone user interface:

- Press the Dir soft key when the phone is idle.

- The phone LCD screen displays the list(s) in the directory.

- If there is only one list in the directory, press the Dir soft key to enter this list directly.

Local Directory

The built-in phone directory stores the names and phone numbers of your contacts. You can store up to 1000 contacts and 7 groups (including the default All Contacts and Blacklist) in your phone's local directory. You can add new groups and add new contacts to different groups in the local directory. You can edit, delete or search for a contact in the local directory. You can also dial a contact from the local directory.

Adding Groups

To add a group to the local directory:

- Press the Dir soft key.

- The IP phone enters the local directory directly as there is only Local Directory in the directory by default.

- If Local Directory is removed from the directory, press Menu->Directory->Local Directory to enter the local directory.

- Press the AddGroup soft key.

- Enter the desired group name in the Name field.

- Press     or     to select the desired group ring tone from the Ring Tones field.

- Press the Add soft key to accept the change or the Back soft key to cancel.

Editing Groups

To edit a group in the local directory:

- Press the Dir soft key.

- The IP phone enters the local directory directly as there is only Local Directory in the directory by default.

- If Local Directory is removed from the directory, press Menu->Directory->Local Directory to enter the local directory.

- Select the desired group (e.g., Test).

- Press the Option soft key, and then select Detail from the prompt list.

- Press     or     to highlight the group information and then edit.

- Press the Save soft key to accept change or the Back soft key to cancel.

Deleting Groups

To delete a group from the local directory:

- Press the Dir soft key.

- The IP phone enters the local directory directly as there is only Local Directory in the directory by default.

- If Local Directory is removed from the directory, press Menu->Directory->Local Directory to enter the local directory.

- Select the desired group (e.g., Test).

- Press the Option soft key, and then select Delete from the prompt list.

The LCD screen prompts “Delete Selected Group?”.

- Press the OK soft key to confirm the deleting or the Cancel soft key to cancel.

You can also delete all groups by pressing the Option soft key and then select Del All. For more information, refer to the above steps.

Note

Adding Contacts

You can add contacts to your local directory in the following ways:

- Manually

- From call history

- From remote phone book

Adding Contacts Manually

To add a contact to the local directory manually:

- Press the Dir soft key.

- The IP phone enters the local directory directly as there is only Local Directory in the directory by default.

- If Local Directory is removed from the directory, press Menu->Directory->Local Directory to enter the local directory.

- Press the Enter soft key.

- If the groups have been added to the local directory, select the desired group and then press the Enter soft key.

- Press the Add soft key.

- Enter the name and the office, mobile or other numbers in the corresponding fields.

- Press     or     , or the Switch soft key to select the desired account from the Account field.

- If Auto is selected, the phone will use the first available account when placing calls to the contact from the local directory.

- Press     or     , or the Switch soft key to select the desired ring tone from the Ring Tones field.

- Press the Add soft key to accept the change or the Back soft key to cancel.

If the contact has existed in the directory, the LCD screen will prompt “ContactName existed!”.

Note

Adding Contacts from Call History

To add a contact to the local directory from the call history:

- Press the History soft key.

- Press     or     to highlight the desired entry.

- Press the Option soft key, and then select Add to Contacts from the prompt list.

- Enter the contact name.

- Press the Save soft key to accept the change.

- The entry is successfully saved to the local directory.

Adding Contacts from Remote Phone Book

To add a contact to the local directory from the remote phone book:

- Press Menu->Directory->Remote Phone Book.

- If Remote Phone Book is added to the directory, press Dir->Remote Phone Book to enter the remote phone book.

- Press     or     to highlight the desired contact.

- Press the Option soft key, and then select Add to Contacts from the prompt list.

- Press the Save soft key to save the contact in the local directory.

If the contact has already existed in the local directory, the LCD screen will prompt “Overwrite the original contact?”. Press the OK soft key to overwrite the original contact in local directory or the Cancel soft key to cancel.

For more information on remote phone book operating, refer to Remote Phone Book on page 39.

Editing Contacts

To edit a contact in the local directory:

- Press the Dir soft key.

- The IP phone enters the local directory directly as there is only Local Directory in the directory by default.

- If Local Directory is removed from the directory, press Menu->Directory->Local Directory to enter the local directory.

- Press the Enter soft key.

- If the groups have been added to the local directory, select the desired group and then press the Enter soft key.

- Press     or     to highlight the desired contact.

- Press the Option soft key, and then select Detail from the prompt list.

- Press     or     to switch between different items and then edit.

- Press the Save soft key to accept change or the Back soft key to cancel.

Deleting Contacts

To delete a contact from the local directory:

- Press the Dir soft key.

- The IP phone enters the local directory directly as there is only Local Directory in the directory by default.

- If Local Directory is removed from the directory, press Menu->Directory->Local Directory to enter the local directory.

- Press the Enter soft key.

- If the groups have been added to the local directory, select the desired group and then press the Enter soft key.

- Press     or     to highlight the desired contact.

- Press the Option soft key, and then select Delete from the prompt list.

The LCD screen prompts “Delete Selected Item?”.

- Press the OK soft key to confirm the deleting or the Cancel soft key to cancel.

Placing Calls to Contacts

To place a call to a contact from the local directory:

- Press the Dir soft key.

- The IP phone enters the local directory directly as there is only Local Directory in the directory by default.

- If Local Directory is removed from the directory, press Menu->Directory->Local Directory to enter the local directory.

- Press the Enter soft key.

- If the groups have been added to the local directory, select the desired group and then press the Enter soft key.

- Press     or     to highlight the desired contact.

- Do one of the following:

- If only one number of the contact is stored in the local directory, press the Dial soft key to dial out the number.

- If multiple numbers of the contact are stored in the local directory, press the Dial soft key to display a list of numbers.

Press     or     to highlight the desired number.

Press the Dial soft key to dial out the number.

Searching for Contacts

To search for a contact in the local directory:

- Press the Dir soft key.

- The IP phone enters the local directory directly as there is only Local Directory in the directory by default.

- If Local Directory is removed from the directory, press Menu->Directory->Local Directory to enter the local directory.

- Press     or     to highlight the All Contacts field.

- Press the Search soft key.

- Enter a few continuous characters of the contact name or the contact phone number using the keypad.

The contacts whose name or phone number matches the characters entered will appear on the LCD screen. You can dial from the result list.

Search Source List in Dialing

You can search for a contact in your desired list when the phone is in the dialing interface. The lists may contain Local Directory, History, Remote Phone Book and LDAP.

To configure search source list in dialing via web user interface:

- Click on Directory->Setting.

- In the Search Source List In Dialing block, select the desired list from the Disabled column and then click     .

- The selected list appears in the Enabled column.

- Repeat the step 2 to add more lists to the Enabled column.

- To remove a list from the Enabled column, select the desired list and then click     .

- To adjust the display order of search results, click     or     .

- The phone LCD screen displays the search results in the adjusted order.

- Click Confirm to accept the change.

You can configure search source list in dialing via web user interface only.

Note

To search for a contact in the enabled search source lists:

- Pick up the handset, press the speakerphone or press the line key.

- Enter a few continuous characters of the contact name or the contact phone number using the keypad.

- The contacts in the enabled search source lists whose name or phone number matches the characters entered will appear on the LCD screen. You can press     or     to scroll to the desired contact and then place a call to the contact.

Importing/Exporting Contact Lists

You can manage your phone’s local directory via phone or web user interface. But you can only import or export the contact list via web user interface.

To import an XML file of contact list via web user interface:

- Click on Directory->Local Directory.

- Click Browse to locate a contact list file (file format must be *.xml) from your local system.

- Click Import XML to import the contact list.

The web user interface prompts “The original contact will be covered, Continue?”.

- Click OK to complete importing the contact list.

To import a CSV file of contact list via web user interface:

- Click on Directory->Local Directory.

- Click Browse to locate a contact list file (file format must be *.csv) from your local system.

- (Optional.) Check the Show Title checkbox.

- It will prevent importing the title of the contact information which is located in the first line of the CSV file.

- Click Import CSV to import the contact list.

- (Optional.) Mark the On radio box in the Delete Old Contacts field.

- It will delete all existing contacts while importing the contact list.

- (Optional.) Select the contact information you want to import into the local directory from the pull down list of Index.

- Click Import to complete importing the contact list.

To export contact list via web user interface:

- Click on Directory->Local Directory.

- Click Export XML (or Export CSV).

- Click Save to save the contact list to your local system.

You can import/export contact lists via web user interface only.

Note

Blacklist

The built-in phone directory stores the names and phone numbers of the blacklist. You can store up to 30 contacts in your phone’s blacklist directory. You can add, edit, delete or search for a contact in the blacklist directory. You can also dial a contact in the blacklist directory, but an incoming call from the blacklist directory will be rejected automatically.

Operating instructions of adding blacklists, editing blacklists, deleting blacklists, placing call to blacklists and searching for a contact in the blacklist, refer to the operating instructions of Local Directory on page 29.

Remote Phone Book

You can access your corporate directory through your phone. You can add local contacts from the corporate directory. You can also dial a contact from the corporate directory.

You can configure your new phone to access up to 5 corporate directories. The phone user interface can display at most 1000 entries of remote phonebooks. Contact your system administrator for the access URL of the corporate directory.

To configure an access URL for remote phone book via web user interface:

- Click on Directory->Remote Phone Book.

- Enter the access URL in the Remote URL field.

- Enter the name in the Display Name field.

- Click Confirm to accept the change.

You can configure an access URL for remote phone book via web user interface only.

Note

To access your corporate directory via phone user interface:

- Press Menu->Directory->Remote Phone Book.

- If Remote Phone Book is added to the directory, press Dir->Remote Phone Book to enter the remote phone book.

- Press     or     to select the desired remote group, and then press the Enter soft key.

- The phone connects to load the corporate directory, and then displays the corporate contact list on the LCD screen.

- Press the Back soft key to back to the previous interface.

To search for a contact in the corporate directory:

- Press Menu->Directory->Remote Phone Book.

- If Remote Phone Book is added to the directory, press Dir->Remote Phone Book to enter the remote phone book.

- Select the desired remote group, and then press the Enter soft key to load the corporate directory.

- Press the Search soft key.

- Enter a few continuous characters of the contact name or continuous numbers of the contact phone number using the keypad.

The contacts whose name or phone number matches the characters entered will appear on the LCD screen. You can dial from the result list.

To place a call from the corporate directory:

- Press Menu->Directory->Remote Phone Book.

- If Remote Phone Book is added to the directory, press Dir->Remote Phone Book to enter the remote phone book.

- Press     or     to select the desired remote group, and then press the Enter soft key to load the corporate directory.

- Select the desired contact in the corporate directory.

- Press the Dial soft key.

In addition, you can enable the phone to present the caller identity stored in the corporate directory when receiving a call.

To enable the presentation of caller identity stored in the remote phone book:

- Click on Directory->Remote Phone Book.

- Select Enabled from the pull-down list of Sremote Name.

- Enter the refresh period of corporate directory in the Sremote Name Flash Time (Seconds) field.

- The default value is 3600 seconds.

- Click Confirm to accept the change.

Call History Management

The SIP-T26P IP phone maintains call history lists of Placed Calls, Received Calls, Missed Calls and Forwarded Calls. The call history lists support to store 100 entries in all. You can check the call history, dial a call, add a contact or delete an entry from the call history list. You should enable the history record feature in advance.

To enable the history record feature via phone user interface:

- Press Menu->Features->History Setting.

- Press     or     , or the Switch soft key to select Enable from the History Record field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

To check the call history:

- Press the History soft key.

The LCD screen displays All Calls list.

- Press     or     to switch between All Calls, Placed Calls, Received Calls, Missed Calls and Forwarded Calls.

- Press     or     to select the desired entry.

- Press the Option soft key, and then select Detail from the prompt list.

The detailed information of the entry appears on the LCD screen.

To dial a call from the call history list:

- Press the History soft key.

- Press     or     to switch between All Calls, Placed Calls, Received Calls, Missed Calls and Forwarded Calls.

- Press     or     to select the desired entry.

- Press the Send soft key.

To add a contact to the local directory or blacklist from the call history list:

- Press the History soft key.

- Press     or     to switch between All Calls, Placed Calls, Received Calls, Missed Calls and Forwarded Calls.

- Press     or     to select the desired entry.

- Press the Option soft key, and then select Add to Contacts (or Add to Blacklist) from the prompt list.

- Enter the desired values in the corresponding fields, and then press the Save soft key.

For more information, refer to Contact Management on page 28.

To delete an entry from the call history list:

- Press the History soft key.

- Press     or     to switch between All Calls, Placed Calls, Received Calls, Missed Calls and Forwarded Calls.

- Press     or     to select the desired entry.

- Press the Delete soft key.

To delete all entries from the call history list:

- Press the History soft key.

- Press     or     to switch between All Calls, Placed Calls, Received Calls, Missed Calls and Forwarded Calls

- Press the Option soft key, and then select Del All from the prompt list.

- Press the OK soft key.

- The LCD screen prompts "Delete all records?".

- Press the OK soft key to confirm the deleting or the Cancel soft key to cancel.

System Customizations

Logo Customization

You can upload your custom logo which will be shown on the idle screen.

To upload a custom logo via web user interface:

- Click on Features->General Information.

- Select Custom Logo from the pull-down list of Use Logo.

- Click Browse to locate the logo file from your local system.

- Click Upload to upload the file.

- Click Confirm to accept the change.

The logo file format must be *.dob, contact your system administrator for more information.

You can configure logo customization via web user interface only.

Note

Headset Use

Physically connect your headset and activate the headset mode for use. For more information on physically connecting a headset, refer to Phone Installation on page 11.

Headset Mode Activation/Deactivation

To activate the headset:

- Press      on the phone.

- The HEADSET key LED illuminates solid green when the headset mode is activated.

- Press the line key or the Answer soft key to answer a call, the call will connect to your headset automatically. Enter the desired number and then press the Send soft key, the phone will use the headset to place a call automatically. For more information on using the headset to place a call, refer to Placing Calls on page 65.

To deactivate the headset:

- Press      again on the phone.

The HEADSET key LED turns off when the headset mode is deactivated.

Headset Prior

You can use headset in priority when enabling the headset prior feature. This feature is especially useful for permanent or full-time headset users.

To enable headset prior via web user interface:

- Click on Features->General Information.

- Select Enabled from the pull-down list of Headset Prior.

- Click Confirm to accept the change.

To activate the headset mode:

- Physically connect the headset.

- Press      to activate the headset mode.

If the headset prior feature is enabled, the headset mode will not be deactivated until you press the HEADSET key again.

You can configure headset prior via web user interface only.

Note

Dual Headset

You can use two headsets when enabling the dual headset feature. To use this feature, you must physically connect headsets to the headset jack and handset jack respectively. Once the phone joins in a call, people with the headset connected to the headset jack has a full-duplex conversation, another people with the headset connected to the handset jack is only allowed to listen to.

To enable dual headset via web user interface:

- Click on Features->General Information.

- Select Enabled from the pull-down list of Dual-Headset.

- Click Confirm to accept the change.

You can configure dual headset via web user interface only.

Note

DSS Keys

There are three types of DSS keys: Memory Keys, Line Keys and Programmable Keys. The details will be introduced in the following. The use of line keys and memory keys are almost the same. The SIP-T26P IP phone supports 10 memory keys and 3 line keys.

Memory Keys

You can assign predefined functionalities to the memory keys located on the right of the phone. Memory keys allow you to have quick access to features such as call return and voice mail. The memory key LEDs will indicate the monitoring status when the keys are assigned specific features, such as BLF. The default key type of each memory key is N/A, which indicates that this memory key provides no functionality.

To configure the memory key via phone user interface:

- Press Menu->Features->DSS Keys->Memory Keys.

- Select the desired memory key, and then press the Enter soft key.

- Select the desired key type from the Type field.

- (Optional.) Select the desired key event from the Key Type field.

- (Optional.) Select the desired line from the Account ID field.

- (Optional.) Enter the corresponding value in the Value field.

- (Optional.) Enter the corresponding value in the Extension field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure the memory key via web user interface at the path DSSKey->Memory Key.

When the phone is idle, you can also long press the memory key to configure it directly on the phone.

Note

The memory key features are explained in the following subchapters in detail:

- Line

- Speed Dial

- Voice Mail

- Directed Pickup

- Group Pickup

- DTMF

- Prefix

- Local Group

- XML Group

- XML Brower

- LDAP

- Conference

- Forward

- Transfer

- Hold

- DND

- SMS

- Group Listening

- Zero Touch

- URL

- Keypad Lock

- Directory

For more information, contact your system administrator.

Line

You can use this key feature to accept the incoming calls, place active calls on hold or resume a held call. It performs the same as a hard line key.

Dependencies: Type (Line)

Account ID (the account this feature will be applied to)

Usage: When receiving an incoming call, the DSS key LED flashes green:

- Press the DSS key to accept the incoming call.

- Press the DSS key to place an active call on hold.

- Press the DSS key again to resume the held call.

Speed Dial

You can use this key feature to speed up dialing numbers often used or hard to remember.

Dependencies: Type (Speed Dial)

Account ID (the account this feature will be applied to)

Value (the number you want to dial out)

Usage: Press the DSS key to dial out the number specified in the Value field, using the account selected from the Account ID field.

Voice Mail

You can use this key feature to connect voice mail quickly. For more information, refer to Voice Mail on page 112.

Dependencies: Type (Key Event)

Key Type (Voice Mail)

Account ID (the account this feature will be applied to)

Value (the voice mail access code)

Usage: Press the DSS key to dial out the voice mail access code, you can follow the voice prompt to listen to the voice mails.

Directed Pickup

You can use this key feature to answer someone else’s incoming call on the phone.

Dependencies: Type (Key Event)

Key Type (Direct Pickup)

Account ID (the account this feature will be applied to)

Value (the directed pickup code followed by the specific phone number)

Usage: Press the DSS key on your phone when the specific phone number receives an incoming call. The call is answered on your phone.

Group Pickup

You can use this key feature to answer incoming calls in a group that is associated with their own group.

Dependencies: Type (Key Event)

Key Type (Group Pickup)

Account ID (the account this feature will be applied to)

Value (the group pickup feature code)

Usage: Press the DSS key on your phone when a phone number in the group receives an incoming call. The call is answered on your phone.

DTMF

You can use this key feature to send the specification of arbitrary key sequences via DTMF.

Dependencies: Type (Key Event)

Key Type (DTMF)

Value (DTMF sequence)

DTMF sequence only contains "0-9", "*", "#" and "A-D".

Note

Usage: Press the DSS key during an active call to send the key sequence specified in the Value field.

Prefix

You can use this key feature to add a specified prefix number before the dialed number.

Dependencies: Type (Key Event)

Key Type (Prefix)

Value (the prefix number)

Usage: Press the DSS key when the phone is idle, the phone enters into the dialing interface and displays the prefix number which you specified in the Value field, enter other digits and then dial out.

Local Group

You can use this key feature to access the group in the local directory quickly. For more information, refer to Local Directory on page 29.

Dependencies: Type (Key Event)

Key Type (Local Group)

Local Group (the group you want to access)

Usage: Press the DSS key to access the group specified in the Local Group field.

XML Group

You can use this key feature to access the remote group in your corporate directory quickly. You should configure the remote phone book in advance. For more information, refer to Remote Phone Book on page 39.

Dependencies: Type (Key Event)

Key Type (XML Group)

PhoneBook Name (the remote group you want to access if the remote phone book is configured)

Usage: Press the DSS key to access the remote group specified in the PhoneBook Name field.

XML Browser

You can use this key feature to access the XML browser quickly. The XML browser allows you to create custom services which meet your functional requirements on the server. You can customize practical applications, such as weather report, stock information, Google search, etc.

Dependencies: Type (XML Browser)

Value (the access URL for xml browser)

Usage: Press the DSS key to access the XML browser specified in the Value field.

LDAP

You can use this key feature to access the LDAP search interface quickly.

Dependencies: Type (Key Event)

Key Type (LDAP)

Usage:

- Press the DSS key to access the LDAP search interface.

- Enter a few continuous characters of the contact name or continuous numbers of the contact phone number using the keypad.

The contacts whose name or phone number matches the characters entered will appear on the LCD screen.

Conference

You can use this key feature to set up a conference call. For more information, refer to Conference on page 85.

Dependencies: Type (Key Event)

Key Type (CONF)

Value (the number you want to add to the conference)

Usage: Press the DSS key during an active call to set up a conference with the number specified in the Value field.

When leaving the Value field blank, the DSS key performs the same function as the CONF key or the Conf soft key during a call.

Note

Forward

You can use this key feature to forward an incoming call to someone else. For more information, refer to Call Forward on page 77.

Dependencies: Type (Key Event)

Key Type (Forward)

Value (the number you want to forward to)

Usage: Press the DSS key to forward an incoming call to the number specified in the Value field.

When leaving the Value field blank, the DSS key performs the same function as the Fwd soft key when receiving an incoming call.

Note

Transfer

You can use this key feature to handle the call differently depending on the transfer mode on DSS key when there is an active call on the phone.

Dependencies: Type (Key Event)

Key Type (Tran)

Value (the number you want to transfer to)

Usage:

- When the transfer mode on DSS key is Blind Transfer, press the DSS key to complete the blind transfer to the number specified in the Value field.

- When the transfer mode on DSS key is Attended Transfer, press the DSS key to dial out the number specified in the Value field and then you can perform the attended or semi-attended transfer.

- When the transfer mode on DSS key is New Call, press the DSS key to place a new call to the number specified in the Value field.

You can configure the transfer mode on DSS key via web user interface at the path Features->Transfer. For more information on how to configure the transfer mode on DSS key, refer to Busy Lamp Field (BLF) on page 95.

When leaving the Value field blank, the DSS key performs the same function as the TRAN key or the Tran soft key during a call. For more information, refer to Call Transfer on page 83.

Note

Hold

You can use this key feature to place an active call on hold or retrieve a held call.

Dependencies: Type (Key Event)

Key Type (Hold)

Usage:

- Press the DSS key during an active call to place the call on hold.

- Press the DSS key again to retrieve the held call.

DND

You can use this key feature to activate or deactivate the DND feature. You can also use this key feature to access the custom DND interface. For more information, refer to Do Not Disturb (DND) on page 75.

Dependencies: Type (Key Event)

Key Type (DND)

Usage:

When the phone is in phone mode:

- Press the DSS key to activate the DND feature.

- Press the DSS key again to deactivate the DND feature.

When the phone is in custom mode:

- Press the DSS key to access the custom DND interface.

- Activate or deactivate the DND feature for an account or all accounts.

When the DND feature is activated, the incoming calls will be rejected automatically.

Note

SMS

You can use this key feature to quick access the text message feature. For more information, refer to Short Message Service (SMS) on page 110.

Dependencies: Type (Key Event)

Key Type (SMS)

Usage: Press the DSS key when the phone is idle to access the text message feature.

Group Listening

You can use this key feature to activate the Speakerphone and Handset/Headset mode at the same time. It is suitable for the group conversation which has more than one person at one side. You are able to speak and listen through the handset/headset, meanwhile the others nearby can only listen through the speaker.

Dependencies: Type (Key Event)

Key Type (Group Listening)

Usage:

- During a call, press the DSS key to activate the group listening mode.

- You can then speak and listen through the handset/headset, other persons at your side can listen through the speaker at the same time.

- Press the DSS key again to deactivate the group listening mode.

Zero Touch

You can use this key feature to configure auto provision and network parameters quickly.

Dependencies: Type (Key Event)

Key Type (Zero Touch)

Usage:

- Press the DSS key to access the zero touch screen.

- Press the OK soft key within a few seconds.

- Configure the network parameters in the corresponding fields.

- Press the Next soft key.

- Configure the auto provision parameters in the corresponding fields.

- Press the OK soft key.

- The phone will reboot to update configurations.

URL

You can use this key feature to trigger the phone to send a HTTP GET request contained a specific URL.

Dependencies: Type (URL)

Value (the URL contained in the HTTP GET request)

Usage: Press the DSS key to trigger the phone to send a HTTP GET request contained the URL specified in the Value field.

Keypad Lock

You can use this key feature to immediately lock the keypad of your phone instead of long pressing      . For more information, refer to Keypad Lock on page 22.

Dependencies: Type (Keypad Lock)

Usage: Press the DSS key to immediately lock the keypad of your phone instead of long pressing      .

Directory

You can use this key feature to easy access to the lists you use frequently. For more information, refer to Directory on page 28.

Dependencies: Type (Dir)

Usage: Press the DSS key to immediately access to the lists you use frequently.

The DSS key performs the same function as the Dir soft key when the phone is idle.

Note

Line Keys

You can assign predefined functionalities to the line keys as memory keys. The default key type of each line key is Line.

To configure the line key via phone user interface:

- Press Menu->Features->DSS Keys->Line Keys.

- Select the desired line key, and then press the Enter soft key.

- Select the desired key type from the Type field.

- (Optional.) Select the desired key event from the Key Type field.

- (Optional.) Select the desired line from the Account ID field.

- (Optional.) Enter the corresponding value in the Value field.

- (Optional.) Enter the corresponding value in the Extension field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure the line key via web user interface at the path DSSKey->Line Key.

For more information on using the line keys, refer to Memory Keys introduced above on page 47.

Programmable Keys

You can customize the soft keys, navigation keys and function keys on the keypad.

To customize the programmable keys via web user interface:

- Click on DSSKey->Programmable Key.

- Customize specific features for these keys.

- Click Confirm to accept the change.

You can configure the programmable keys via web user interface only.

Note

Account Management

You can register one or multiple accounts on the SIP-T26P IP phone. You can also configure each line key associated with an account or configure multiple line keys associated with an account.

Account Registration

To register an account via phone user interface:

- Press Menu->Settings->Advanced Settings (password: admin) ->Accounts.

- Select the desired account and then press the Enter soft key.

- Select Enable from the Activation field.

- Enter the desired values in the Label, Display Name, Register Name, User Name, Password and SIP Server1 fields respectively. Contact your system administrator for more information.

- Press the Save soft key to accept the change or the Back soft key to cancel.

- Repeat steps 2 to 5 until you have registered all accounts.

To disable an account via phone user interface:

- Press Menu->Settings->Advanced Settings (password: admin) ->Accounts.

- Select the desired account and then press the Enter soft key.

- Select Disable from the Activation field to disable the account.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also register or disable an account via web user interface at the path Account->Register.

Multiple Line Keys per Account

You can configure multiple line keys associated with an account. This enhances call visualization and simplifies call handling.

Incoming calls to this line will be distributed evenly among the available line keys. Similarly, outgoing calls will be distributed.

Your phone can be configured to have a combination of lines with a single line key and lines with multiple line keys.

Dial Plan

Dial plan is a string of characters that governs the way your SIP-T26P IP phone processes the inputs received from your phone keypad. The SIP-T26P IP phone supports the following dial plan features:

- Replace Rule

- Dial-now

- Area Code

- Block Out

The basic expression syntax you need to know:

| . | The dot ”.” can be used as a placeholder or multiple placeholders for any character. Example: “12.” would match “123”, “1234”, “12345”, "12abc", etc. |
| --- | --- |
| x | An “x” can be used as a placeholder for any character. Example: “12x” would match “121”, “122”, “123”, "12a", etc. |
| [] | The square brackets “[]”can be used as a placeholder for a single character which matches any of a set of characters. Example: “91[5-7]1234” would match “9151234”, “9161234”, “9171234”. |
| () | The parentheses “( ) “can be used to group together patterns, for instance, to logically combine two or more patterns. Example: “91([5-7])1(x)” would match “91511”, “91618”, “91715”, etc. |
| $ | The “$” should be followed by the sequence number of a parenthesis. The “$” plus the sequence number means the whole character or characters placed in the parenthesis. The number directs to the right parenthesis when there are more than one. Example: A replace rule configuration: Prefix: “9([5-7]) (.)”, Replace: “5$2”. When you enter “96123” to dial out on your phone, the number will be replaced as “5123” and then dialed out. “$2” means the characters in the second parenthesis, that is, “123”. |

Replace Rule

You can configure one or more replace rules to remove the specified string and replace it with another string. You can configure a pattern with wildcards (refer to the expression syntax in the table above), so that any string that matches the pattern will be replaced. This feature is convenient for you to dial out a long number. For example, a replace rule is configured as “Prefix: 1” and “Replace: 1234567”, when you try to dial out the number “1234567”, you just need to enter “1” on the phone and then press the Send soft key.

To add a replace rule via web user interface:

- Click on Settings->Dial Plan->Replace Rule.

- Enter the string (e.g., 1) in the Prefix field.

- Enter the string (e.g., 1234) in the Replace field.

- Enter the desired line ID in the Account field or leave it blank.

- Click Add to add the replace rule.

When you enter the number “1” using the keypad and then press the Send soft key, the phone will dial out “1234” instead.

The valid values of Account field can be one or more digits among 1, 2, and 3. Each digit must be separated by a “,”. For example, when you enter the value “1, 2” in the Account field, this replace rule will apply to account1 and account2.

If you leave the Account field blank or enter 0, the replace rule will apply to all accounts.

Note

To edit a replace rule via web user interface:

- Click on Settings->Dial Plan->Replace Rule.

- Select the desired replace rule by checking the check box.

- Edit the values in the Prefix and Replace fields.

- Enter the desired line ID in the Account field or leave it blank.

- Click Edit to accept the change.

To delete one or more replace rules via web user interface:

- Click on Settings->Dial Plan->Replace Rule.

- Select one or more replace rules by checking the check box(es).

- Click Del to delete the replace rule(s).

You can configure the replace rule via web user interface only.

Note

Dial-now

You can configure one or more dial-now rules on your phone. When the dialed out number matches the dial-now string, the number will be dialed out automatically. For example, a dial-now rule is configured as “2xx”, then entering any three-digit string begins with 2 will be dialed out automatically on the phone.

To add a dial-now rule via web user interface:

- Click on Settings->Dial Plan->Dial-now.

- Enter the desired value (e.g., 1234) in the Rule field.

- Enter the desired line ID in the Account field or leave it blank.

- For more information on the valid values of Account field, refer to Replace Rule on page 57.

- Click Add to add the dial-now rule.

- When you enter the number “1234” using the keypad, the phone will dial out “1234” automatically without pressing any key.

You can also edit or delete the dial-now rule, refer to Replace Rule on page 57 for more information.

You can configure the dial-now rule via web user interface only.

Note

Delay Time for Dial-now Rule

You can configure the delay time for dial-now rules. That is, you can configure your phone to dial out the phone number automatically, which matches a dial-now rule, after the specified delay time.

To configure the delay time for dial-now rule via web user interface:

- Click on Features->General Information.

- Enter the time within 1-14 (in seconds) in the Time-Out For Dial-Now Rule field.

- Click Confirm to accept the change.

You can configure the delay time for dial-now rule via web user interface only.

Note

Area Code

Area codes are also known as Numbering Plan Areas (NPAs). They usually indicate geographical areas in a country. This feature is necessary only when dialing the number outside the code area. For example, area code is configured as “Code: 0592, Min Length: 4, Max Length: 11”, then when you dial out the number “56789”, which has the digits between 4 to 11, the phone will add the area code and dial out the number “059256789”.

To configure the area code via web user interface:

- Click on Settings->Dial Plan->Area Code.

- Enter the desired values in the Code, Min Length (1-15) and Max Length (1-15) fields.

- Enter the desired line ID in the Account field or leave it blank.

- For more information on the valid values of Account field, refer to Replace Rule on page 57.

- Click Confirm to accept the change.

The default values of minimum and maximum length are 1 and 15 respectively.

You can configure the area code via web user interface only.

Note

Block Out

You can block some specific numbers from being dialed on your phone. When you dial a block out number on your phone, the dialing will fail and the LCD screen will prompt “Forbidden Number”.

To add a block out number via web user interface:

- Click on Settings->Dial Plan->Block Out.

- Enter the desired value in the BlockOut Number field.

- Enter the desired line ID in the Account field or leave it blank.

- For more information on the valid values of Account field, refer to Replace Rule on page 57.

- Click Confirm to add the block out number.

You can configure a block out number via web user interface only.

Note

Emergency Number

Public telephone networks in countries around the world have a single emergency telephone number (emergency services number), that allows a caller to contact local emergency services for assistance when required. The emergency telephone number may differ from country to country. It is typically a three-digit number so that it can be easily remembered and dialed quickly. Some countries have different emergency numbers for different emergency services.

You can specify the emergency telephone number on the IP phone for contacting the emergency services in an emergency situation.

Contact your local phone service provider for available emergency numbers in your area.

Note

To specify emergency numbers via web user interface:

- Click on Features->Phone Lock.

- Enter the emergency services number in the Emergency field.

- For multiple numbers, enter a “,” between each emergency number. The default emergency numbers are 112, 911 and 110.

- Click Confirm to accept the change.

You can configure emergency number via web user interface only.

Note

Live Dialpad

You can enable the live dialpad feature on the SIP-T26P IP phone, which enables the IP phone to automatically dial out the phone number without pressing any other key. You can also configure a period of delay time to dial out the phone number. The phone will dial out the phone number automatically after the specified delay time.

To enable the live dialpad feature via web user interface:

- Click on Settings->Preference.

- Select Enabled from the pull-down list of Live Dialpad.

- Enter the desired delay time in the Inter Digit Time (1~14s) field.

- Click Confirm to accept the change.

The default delay time of live dialpad is 4s.

You can configure live dialpad via web user interface only.

Note

Hot Line

You can dial a hotline number immediately once you lift the handset, press the Speakerphone key or press a line key. You can also configure a period of delay time to dial out the hotline number. The phone will dial out the hotline number automatically after the specified delay time.

To configure the hot line number via phone user interface:

- Press Menu->Features->Hot Line.

- Enter the desired number in the Hot Number field.

- Enter the delay time (in seconds) in the HotLine Delay field.

- The valid values range from 0 to 10.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure hot line via web user interface at the path Features->General Information.

Basic Call Features

The SIP-T26P IP phone is designed to be easily used like a regular phone on a public switched telephone network (PSTN). You can place calls, answer calls, transfer a call to someone else, or conduct a conference call.

This chapter provides basic operating instructions for the SIP-T26P IP phone. The topics include:

- Placing Calls

- Answering Calls

- Ending Calls

- Redialing Numbers

- Recent Call In Dialing

- Auto Answer

- Auto Redial

- Call Completion

- Call Return

- Call Mute

- Call Hold/Resume

- Do Not Disturb (DND)

- Call Forward

- Call Transfer

- Call Waiting

- Conference

- Call Park

- Call Pickup

- Anonymous Call

- Anonymous Call Rejection

If you require additional information or assistance with your new phone, contact your system administrator.

Placing Calls

You can place a call in three ways using your SIP-T26P IP phone:

- Using the handset

- Using the speakerphone

- Using the headset

You can also dial the number first, and then choose the way you want to speak to the other party.

You can search the call history, the local contact directory or the remote phone book and dial from the search results. For more information, refer to Contact Management on page 28 and Call History Management on page 42.

During a call, you can alternate between Speakerphone, Headset, or Handset modes by pressing the Speakerphone key, the HEADSET key, or picking up the handset.

The call duration of active call is visible on the LCD screen. In the figure below, the call to Kame has lasted 1 minute and 8 seconds.

To place a call using the handset:

- Pick up the handset.

- Enter the desired number using the keypad.

- Press     ,      , or the Send soft key.

By default, the # key is configured as a send key. You can also set the * key as a send key or set neither of them as send keys. For more information, refer to Key as Send on page 22.

You can also dial using the SIP URI or IP address. To obtain the IP address of the phone, press the OK key. The maximum SIP URI or IP address length is 32 characters. For example, SIP URI: 2210@sip.com, IP: 192.168.1.15.

Your phone may not support direct IP dialing. Contact your system administrator for more information.

Note

To place a call using the hands-free speakerphone mode:

Do one of the following:

- With the handset on-hook, press      or the line key to obtain a dial tone.

- Enter the desired number using the keypad.

- Press     ,      , or the Send soft key.

- With the handset on-hook, enter the desired number using the keypad.

- Press      ,      ,     , or the Send soft key.

To place a call using the headset:

Do one of the following:

- With the optional headset connected, press      to activate the headset mode.

- Press the line key to obtain a dial tone.

Enter the desired number using the keypad.

- Press     ,      , or the Send soft key.

- With the optional headset connected, press      to activate the headset mode.

Enter the desired number using the keypad.

Press     ,      , or the Send soft key.

To permanently enable your headset, refer to Headset Prior on page 45.

Note

To place multiple calls:

You can have more than one call on your SIP-T26P IP phone. To place a new call during an active call, do one of the following:

- Press the line key. The active call is placed on hold.

Enter the desired number using the keypad.

Press     ,      , or the Send soft key.

- Press      or the Hold soft key to place the original call on hold.

Press the NewCall soft key.

Enter the desired number using the keypad.

Press     ,      , or the Send soft key.

You can press     or     to switch between the calls, and then press the Resume soft key to retrieve the desired call.

Answering Calls

When you are not in another call, you can answer a call in three ways:

- Using the handset

- Using the speakerphone

- Using the headset

You can ignore incoming calls by pressing the X key, the Reject soft key or the Silence soft key. You can also activate the Do Not Disturb mode to ignore all incoming calls without ring on your phone. For more information, refer to Do Not Disturb (DND) on page 75.

You can forward incoming calls to someone else by pressing the Fwd soft key. For more information, refer to Call Forward on page 77.

Note

Answering When Not in Another Call

In all cases, the active call will appear on the LCD screen showing call duration and destination.

To answer a call using the handset:

- Pick up the handset.

To answer a call using the hands-free speakerphone mode:

Do one of the following:

- Press      .

- With the handset on-hook and headset mode deactivated, press the Answer soft key.

- With the handset on-hook and headset mode deactivated, press the line key (the line key LED flashes green).

To answer a call using the headset:

Do one of the following:

- Press      .

- With the headset mode activated, press the Answer soft key.

- With the headset mode activated, press the line key (the line key LED flashes green).

Answering When in Another Call

If you have an active call, and an incoming call arrives on the phone, do one of the following:

- Press the Answer soft key.

The incoming call is answered and the original call is placed on hold.

- Press     to access the new call.

- Press     or the Answer soft key.

The incoming call is answered and the original call is placed on hold.

Ending Calls

To end a call:

Do one of the following:

- If you are using the handset, press the Cancel soft key or hang up the handset.

- If you are using the headset, press the Cancel soft key.

- If you are using the speakerphone, press      or the Cancel soft key.

To end a call placed on hold, you can press the Cancel soft key to end the call directly, or press the Resume soft key to resume the call before ending it.

Note

Redialing Numbers

To redial the last dialed number from your phone:

- Press      twice.

- A call to your last dialed number is attempted.

To redial a previously dialed number from your phone:

- Press      when the phone is idle.

- Press     or     to select the desired entry from the placed call list, and then

- press      or the Send soft key.

Recent Call In Dialing

You can view the placed calls list when the phone is in the pre-dialing interface. You should enable the recent call in dialing feature in advance.

To enable the recent call in dialing feature via web user interface:

- Click on Directory->Setting.

- Select Enabled from the pull-down list of Recent Call In Dialing.

- Click Confirm to accept the change.

You can enable the recent call in dialing feature via web user interface only.

Note

To view place calls list when the phone is in the dialing interface:

      - Pick up the handset, press the speakerphone or press the line key.

- The phone LCD screen displays the placed calls list.

Auto Answer

You can use the auto answer feature to automatically answer an incoming call on a line. Auto answer is configurable on a per-line basis.

To configure auto answer via phone user interface:

      - Press Menu->Settings->Advanced Settings (password: admin) ->Accounts.

      - Select the desired account and then press the Enter soft key.

      - Press     or     , or the Switch soft key to select Enable from the Auto Answer field.

      - Press the Save soft key to accept the change or the Back soft key to cancel.

- The     icon appears on the LCD screen.

You can also configure auto answer via web user interface at the path Account->Basic.

The auto answer feature is only applicable when there is no other call in progress on the phone.

Note

Auto Redial

You can enable the auto redial feature to redial the phone number automatically when the called party is busy. You can also configure the times of the auto redial and the time to wait between redial attempts.

To configure auto redial via phone user interface:

      - Press Menu->Features->Auto Redial.

      - Press     or     , or the Switch soft key to select Enable from the Auto Redial field.

      - Enter the desired time in the Redial Interval field.

- The default time interval is 10s.

      - Enter the desired times in the Redial Times field.

- The default times are 10.

      - Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure auto redial via web user interface at the path Features->General Information.

To use auto redial:

When the called party is busy, the following prompt will appear on the LCD screen of the phone:

            - Press the OK soft key to activate auto redial. The following prompt will appear on the LCD screen of the phone:

            - Wait for a period of time or press the OK soft key to redial the phone number.

The phone will retry as many times as you configured until the called party is idle.

Call Completion

You can use the call completion feature to notify the caller who failed to reach a desired callee when the callee becomes available to receive a call.

To configure call completion via phone user interface:

            - Press Menu->Features->Call Completion.

            - Press     or     , or the Switch soft key to select Enable from the Call Completion field.

            - Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure call completion via web user interface at the path Features->General Information.

To use call completion:

When the called party is busy, the following prompt will appear on the LCD screen of the phone:

            - Press the OK soft key, the phone returns to the idle screen and call completion is activated.

- When the called party becomes idle, the following prompt appears on the LCD screen of the phone:

            - Press the OK soft key to redial the number.

The call completion feature is not available on all servers. For more information, contact your system administrator.

Note

Call Return

You can press a call return key to place a call back to the last incoming call.

To configure a call return key via phone user interface:

- Press Menu->Features->DSS Keys->Memory Keys (or Line Keys).

- Select a desired DSS key.

- Press     or     , or the Switch soft key to select Key Event from the Type field.

- Press     or     , or the Switch soft key to select Call Return from the Key Type field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure a call return key via web user interface at the path DSSKey.

Call Mute

You can mute the microphone of the active audio device during an active call, and then the other party cannot hear you.

To mute a call:

- Press      during an active call.

The phone LCD screen indicates that the call is on mute.

To un-mute a call:

- Press      again to un-mute the call.

Call Hold/Resume

You can place an active call on hold. At any time, at most one active call can be in progress on your phone, other calls can be received and made while placing the original call on hold. When placing a call on hold, your IP PBX might play a melody to the other party while waiting.

To place a call on hold:

            - Press      or the Hold soft key during a call.

- The phone LCD screen shows the call is on hold and the line key LED flashes green.

The phone will beep softly every 30 seconds to remind you that you still have a call on hold.

Note

To resume a held call:

- Press      or the Resume soft key.

Multiple Calls on Hold:

If multiple calls are placed on hold, press     or     to switch between the calls, and then press the Resume soft key to retrieve the desired call.

If more than two calls are on hold, an indication appears on the LCD screen, for example “2/3”, indicating that this is the second call out of three calls.

If multiple calls are on hold on more than one line key, you can view the details of the calls by pressing the corresponding line key, and then press the Resume soft key to retrieve the call.

Do Not Disturb (DND)

You can use the DND feature to reject the incoming calls automatically on the phone and the callers can hear a busy signal or a message depending on your server.

You can enable/disable the DND feature for the phone system, or you can customize the DND feature for each account or all accounts. The following describes the DND modes:

- Phone (default): DND in phone mode means that the DND feature is effective for the phone system.

- Custom: DND in custom mode means that you can configure the DND feature for each account or all accounts.

You can also configure the phone to receive incoming calls from authorized numbers when the DND feature is enabled.

To configure the DND mode via web user interface:

- Click on Features->Forward & DND.

- In the DND block, mark the desired radio box in the Mode field.

- Click Confirm to accept the change.

You can configure the DND mode via web user interface only.

Note

To activate DND in the phone mode:

            - Press the DND soft key when the phone is idle.

- The     icon on the idle screen indicates that the DND feature is enabled.

- Incoming calls will be rejected automatically and "n New Missed Call(s)" ("n" indicates the number of the missed calls) will prompt on the LCD screen.

To activate DND in the custom mode for a specific account:

- Press the DND soft key when the phone is idle.

- The LCD screen displays a list of the accounts on the phone.

- Press     or     to select the desired account.

- Press     or     soft key to select On to activate DND.

- Press the Save soft key to accept the change.

- The     icon appears on the idle screen and the associated line icon changes to    .

- Incoming calls on this line will be rejected automatically and "n New Missed Call(s)" ("n" indicates the number of the missed calls) will prompt on the LCD screen.

To activate DND in the custom mode for all accounts:

- Press the DND soft key when the phone is idle.

- The LCD screen displays a list of the accounts on the phone.

- Press the All On soft key to activate DND for all accounts.

- Press the Save soft key to accept the change.

- The     icon appears on the idle screen and all line icons change to    .

- Incoming calls will be rejected automatically and "n New Missed Call(s)" ("n" indicates the number of the missed calls) will prompt on the LCD screen.

The prompt message displays only if the Missed Call Log feature for the line is enabled. The Missed Call Log feature can be configured via web user interface at the path Account->Basic.

The Do Not Disturb feature is local to the phone, and may be overridden by the server settings. For more information, contact your system administrator.

Note

To configure the DND authorized numbers via web user interface:

      - Click on Features->Forward & DND.

      - Select Enabled from the pull-down list of DND Emergency.

      - Enter the numbers in the DND Authorized Numbers field.

- For multiple numbers, enter a "," between each number.

      - Click Confirm to accept the change.

- When the DND feature is enabled on the phone, the phone can still receive incoming calls from the numbers specified in the DND Authorized Numbers field.

You can configure the DND authorized numbers via web user interface only.

Note

Call Forward

You can configure your phone to forward incoming calls to another party by the static forwarding. You can also forward calls while your phone is ringing, refer to the dynamic forwarding.

Static Forwarding

You can configure three types of call forward:

- Always Forward: Incoming calls are immediately forwarded.

- Busy Forward: Incoming calls are immediately forwarded if the phone is busy.

- No Answer Forward: Incoming calls are forwarded if not answered after a period of time.

You can enable/disable the call forward feature for the phone system, or you can customize the call forward feature for each account or all accounts. The following describes the call forward modes:

- Phone (default): Call forward in phone mode means that the call forward feature is effective for the phone system.

- Custom: Call forward in custom mode means that you can configure the call forward feature for each account or all accounts.

To configure the call forward mode via web user interface:

- Click on Features->Forward & DND.

- In the Forward block, mark the desired radio box in the Mode field.

- Click Confirm to accept the change.

You can configure the call forward mode via web user interface only.

Note

To enable call forward in phone mode:

- Press Menu->Call Control->Call Forwarding.

- Press     or     to select the desired forwarding type, and then press the Enter soft key.

- Depending on your selection:

a.)  If you select Always Forward:

1)  Press     or     , or the Switch soft key to select Enable from the Always field.

- 2)  Enter the destination number you want to forward all incoming calls to in the Forward To field.

- 3)  (Optional.) Enter the always forward on code or off code respectively in the On Code or Off Code field.

- b.)  If you select Busy Forward:

- 1)  Press     or     , or the Switch soft key to select Enable from the Busy field.

- 2)  Enter the destination number you want to forward all incoming calls to when the phone is busy in the Forward To field.

- 3)  (Optional.) Enter the busy forward on code or off code respectively in the On Code or Off Code field.

- c.)  If you select No Answer Forward:

- 1)  Press     or     , or the Switch soft key to select Enable from the No Answer field.

- 2)  Enter the destination number you want to forward all unanswered incoming calls to in the Forward To field.

- 3)  Press     or     , or the Switch soft key to select the ring time to wait before forwarding in the After Ring Time field.

- The default ring time is 12 seconds.

- 4)  (Optional.) Enter the no answer forward on code or off code respectively in the On Code or Off Code field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

- The    icon on the idle screen indicates that call forward feature is enabled.

To enable call forward in custom mode:

            - Press Menu->Call Control->Call Forwarding.

            - Press     or     to select the desired account, and then press the Enter soft key.

            - Press     or     to select the desired forwarding type, and then press the Enter soft key.

            - Depending on your selection:

a.)  If you select Always Forward, you can enable it for a specific account.

1)  Press     or     , or the Switch soft key to select Enable from the Always field.

2)  Enter the destination number you want to forward all incoming calls to in the Forward To field.

3)  (Optional.) Enter the always forward on code or off code respectively in the On Code or Off Code field.

You can also enable the always forward feature for all accounts. After the always forward feature was enabled for a specific account, do as below:

1)  Press     or     to highlight the Always field.

2)  Press the All Lines soft key.

The LCD screen prompts “Copy to All Lines?”.

3)  Press the OK soft key to accept the change or the Cancel soft key to cancel.

b.)  If you select Busy Forward, you can enable it for a specific account.

1)  Press     or     , or the Switch soft key to select Enable from the Busy field.

2)  Enter the destination number you want to forward all incoming calls to when the phone is busy in the Forward To field.

3)  (Optional.) Enter the busy forward on code or off code respectively in the On Code or Off Code field.

You can also enable the busy forward feature for all accounts. After the busy forward feature was enabled for a specific account, do as below:

1)  Press     or     to highlight the Busy field.

2)  Press the All Lines soft key.

The LCD screen prompts “Copy to All Lines?”.

3)  Press the OK soft key to accept the change or the Cancel soft key to cancel.

c.)  If you select No Answer Forward, you can enable it for a specific account.

1)  Press     or     , or the Switch soft key to select Enable from the No Answer field.

2)  Enter the destination number you want to forward all unanswered incoming calls to in the Forward To field.

3)  Press     or     , or the Switch soft key to select the ring time to wait before forwarding in the After Ring Time field.

The default ring time is 12 seconds.

4)  (Optional.) Enter the no answer forward on code or off code respectively in the On Code or Off Code field.

You can also enable the no answer forward feature for all accounts. After the no answer forward feature was enabled for a specific account, do as below:

1)  Press     or     to highlight the No Answer field.

2)  Press the All Lines soft key.

The LCD screen prompts “Copy to All Lines?”.

3)  Press the OK soft key to accept the change or the Cancel soft key to cancel.

            - Press the Save soft key to accept the change or the Back soft key to cancel.

The    icon appears on the idle screen and the associated line icon changes to    .

You can also configure call forward feature via web user interface at the path Features->Forward & DND.

You can also enter the SIP URL or IP address in the Forward To field. For more information on using the SIP URL or IP address, refer to Placing Calls on page 65.

The call forward feature is local to the phone, and may be overridden by the server settings. Call forward on code or off code may be different between servers. For more information, contact your system administrator.

Note

To disable call forward in phone mode:

Do one of the following:

- Press      when the phone is idle.

- Press Menu->Call Control->Call Forwarding.

- Press     or     to select the desired forwarding type, then press the Enter soft key.

- Press     or     , or the Switch soft key to select Disable to disable the call forward.

- Press the Save soft key to accept the change.

To disable call forward in custom mode for a specific account:

            - Press Menu->Call Control->Call Forwarding or press       when the phone is idle.

            - Press     or     to select the desired account, then press the Enter soft key.

            - Press     or     to select the desired forwarding type, then press the Enter soft key.

            - Press     or     , or the Switch soft key to select Disable to disable the call forward.

            - Press the Save soft key to accept the change.

Dynamic Forwarding

To forward an incoming call to another party:

- When the phone is ringing, press the Fwd soft key.

- Enter the number you want to forward the incoming call to.

- Press     ,      , or the Send soft key.

- The incoming call is forwarded. The LCD screen prompts a call forward message.

Call Transfer

You can transfer a call to another party in one of the three ways:

- Blind Transfer: Transfer a call directly to another party without consulting.

- Semi-Attended Transfer: Transfer a call when receiving ringback.

- Attended Transfer: Transfer a call with prior consulting.

To perform a blind transfer:

- Press      or the Tran soft key during a call.

- Enter the number you want to transfer the call to.

- Press      or the Tran soft key to complete the transfer.

Then the call is connected to the number to which you are transferring.

To perform a semi-attended transfer:

- Press      or the Tran soft key during a call.

- Enter the number you want to transfer the call to.

- Press     or      to dial out.

- Press      or the Tran soft key to complete the transfer when receiving the ringback.

To perform an attended transfer:

- Press      or the Tran soft key during a call.

- Enter the number you want to transfer the call to.

- Press     or      to dial out.

- After the party answers the call, press      or the Tran soft key to complete the transfer.

If you are using a handset, the transfer can be completed by hanging up the handset.

You can cancel the transfer before the call is connected by pressing the Cancel soft key.

Call Waiting

You can enable or disable the call waiting feature on the phone. If the call waiting feature is enabled, you can receive another call when there is an active call on the phone. Otherwise, another incoming call is automatically rejected by the phone with a busy message when there is an active call on the phone. You can also enable or disable the phone to play a warning tone when receiving another call.

To configure call waiting via phone user interface:

            - Press Menu->Features->Call Waiting.

            - Press     or     , or the Switch soft key to select Enable from the Call Waiting field.

            - Press     or     , or the Switch soft key to select Enable from the Play Tone field.

            - (Optional.) Enter the call waiting on code or off code respectively in the CW On Code or CW Off Code field.

            - Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure the call waiting feature via web user interface at the path Features->General Information.

Conference

You can create a conference with other parties using the phone’s local conference feature. You can create a conference between an active call and a call on hold (on the same or another line) by pressing the CONF key or the Conf soft key. The network conference feature allows you to add and remove specific conference parties.

The network conference feature is not available on all servers. For more information, contact your system administrator.

Note

Local Conference

The SIP-T26P IP phone supports up to 3 parties (including yourself) in a conference call. This is the default method of conference called Local Conference.

To set up a local conference call:

- Place a call to the first party.

- When the first party answers the call, press      or the Conf soft key to place a new call.

The active call is placed on hold.

- Enter the number of the second party and press     ,      , or the Send soft key.

- When the second party answers the call, you can consult with him or her before adding to the conference.

- Press      or the Conf soft key again to join all parties in the conference.

To join two calls in a conference:

- Place two calls using two different accounts on the phone (for example, place the first call using account 1, and then place the second call using account 2).

- Press     or     to select the call for conference and make sure the call is active (for example, select the call on account 1).

- Press      or the Conf soft key to join the two calls in the conference on account 1.

You can press      or the Hold soft key to place the conference on hold. You can press the Split soft key to split the conference call into two individual calls. To drop the conference call, press the Cancel soft key.

Network Conference

You can use the network conference feature on the SIP-T26P IP phone to conduct a conference with multiple participants.

This feature allows you to perform the following:

- Join two calls together into a conference call.

- Invite another party into an active conference call.

To use this feature, contact your system administrator for the network conference URI in advance.

To configure the network conference feature via web user interface:

- Click on Account.

- Select the desired account from the pull-down list of Account.

- Click on Advanced.

- Select Network Conference from the pull-down list of Conference Type.

- Enter the conference URI (e.g., conference@example.com) in the Conference URI field.

- Click Confirm to accept the change.

You can configure the network conference via web user interface only.

Note

To set up a network conference call:

- Place a call to the first party.

- Press      or the Conf soft key to create a new call.

The active call is placed on hold.

- Enter the number of the second party and press     ,      , or the Send soft key.

- When the second party answers the call, press      or the Conf soft key to add the second party to the conference.

- Press the Conf soft key to create a new call.

The conference is placed on hold.

- Enter the number of the new party and then press     ,      , or the Send soft key.

- When the new party answers the call, press      or the Conf soft key to add the new party to the conference.

- Repeat steps 5 to 7 until you have added all intended parties.

The procedures to set up a network conference call for specific servers may be different from introduced above. Contact your system administrator for more information.

Call Park

You can use call park feature to place a call on hold, and then retrieve the call from another phone in the system (for example, a phone in another office or conference room). You can park an active call by pressing the call park key on the phone. If the call is parked successfully, the response may be voice prompts confirming that the call was parked, or a visible indication on the LCD screen.

The call park feature is not available on all servers. Contact your system administrator for more information.

Note

To configure a call park key via phone user interface:

- Press Menu->Features->DSS Keys->Memory Keys (or Line Keys).

- Select the desired DSS key.

- Press     or     , or the Switch soft key to select Key Event from the Type field.

- Press     or     , or the Switch soft key to select Call Park from the Key Type field.

- Press     or     , or the Switch soft key to select the desired line from the Account ID field.

- Enter the call park feature code in the Value field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure a call park key via web user interface at the path DSSKey.

To use the call park feature:

- User on phone A places a call to phone B.

- User on phone A wants to take the call in a conference room for privacy, then presses the call park key on phone A.

- User on phone A walks to an available conference room where the phone is designated as phone C. The user dials the call park retrieve feature code to retrieve the parked call.

The system establishes call between phone C and B.

The call park feature code and call park retrieve feature code are predefined on the system server. Contact your system administrator for more information.

If the parked call is not retrieved within a period of time assigned by the system, the phone performing call park feature will receive call back.

Note

Call Pickup

You can use call pickup feature to answer someone else’s incoming call on the phone. The SIP-T26P IP phone supports the directed call pickup and group call pickup features. Directed call pickup is used for picking up a call that is ringing at a specific phone number. Group call pickup is used for picking up a call that is ringing at any phone number in the group. The pickup group should be predefined, contact your system administrator for more information.

You can pick up an incoming call by using the DPickup/GPickup soft key. To use the call pickup feature, you need to configure the call pickup code beforehand on a global or per-line basis via web user interface.

If there are many incoming calls at the same time, pressing the GPickup soft key on the phone will pick up the call that rings first.

Note

Directed Call Pickup

To enable the directed call pickup via web user interface:

- Click on Features->Call Pickup.

- Select Enabled from the pull-down list of Directed Call Pickup.

- Click Confirm to accept the change.

To configure the directed call pickup code on a global basis via web user interface:

- Click on Features->Call Pickup.

- Enter the directed call pickup code in the Directed Call Pickup Code field.

- Click Confirm to accept the change.

To configure the directed call pickup code on a per-line basis via web user interface:

- Click on Account.

- Select the desired account from the pull-down list of Account.

- Click on Advanced.

- Enter the directed call pickup code in the Directed Call Pickup Code field.

- Click Confirm to accept the change.

To pick up a call directly:

- Pick up the handset.

- The DPickup soft key appears on the LCD screen.

- Press the DPickup soft key on your phone when a phone receives an incoming call.

- Enter the phone number which receives an incoming call.

- Press the DPickup soft key again.

- The call is answered on your phone.

You can also configure a DSS key as directed pickup key via phone user interface or web user interface. You can pick up a call by pressing the directed pickup key directly.

Group Call Pickup

To enable the group call pickup via web user interface:

- Click on Features->Call Pickup.

- Select Enabled from the pull-down list of Group Call Pickup.

- Click Confirm to accept the change.

To configure the group call pickup code on a global basis via web user interface:

            - Click on Features->Call Pickup.

            - Enter the group call pickup code in the Group Call Pickup Code field.

            - Click Confirm to accept the change.

To configure the group call pickup code on a per-line basis via web user interface:

      - Click on Account.

      - Select the desired account from the pull-down list of Account.

      - Click on Advanced.

      - Enter the group call pickup code in the Group Call Pickup Code field.

      - Click Confirm to accept the change.

To pick up a call in the group:

- Pick up the handset.

- The GPickup soft key appears on the LCD screen.

- Press the GPickup soft key on your phone when a phone in the group receives an incoming call.

The call is answered on your phone.

You can also configure a DSS key as a group pickup key via phone user interface or web user interface. You can pick up a call by pressing the group pickup key directly.

The directed call pickup code and group call pickup code are predefined on the system server. Contact your system administrator for more information.

The call pickup code configured on a per-line basis takes precedence over that configured on a global basis.

Note

Anonymous Call

You can use anonymous call feature to block the identity and phone number from showing up to the called party when you call someone. For example, you want to call to consult some of the services, but you don’t want to be harassed. Anonymous call is configurable on a per-line basis.

The anonymous call feature is not available on all servers. Contact your system administrator for the anonymous call on code and off code.

Note

To configure anonymous call via phone user interface:

- Press Menu->Features->Anonymous Call.

- Press     or     , or the Switch soft key to select the desired line from the Line ID field.

- Press     or     , or the Switch soft key to select Enable from the Anonymous Call field.

- (Optional.) Enter the anonymous call on code in the Call On Code field.

- (Optional.) Enter the anonymous call off code in the Call Off Code field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure anonymous call via web user interface at the path Account->Basic.

To place an anonymous call:

- Using the specific line on the phone to place a call to phone B.

The LCD screen of phone B prompts an incoming call from anonymity.

Anonymous Call Rejection

You can use anonymous call rejection feature to reject incoming calls from anonymous callers. Anonymous call rejection automatically rejects incoming calls from callers who deliberately block their identities and numbers from showing up. Anonymous call rejection is configurable on a per-line basis.

To configure anonymous call rejection via phone user interface:

- Press Menu->Features->Anonymous Call.

- Press     or     , or the Switch soft key to select the desired line from the Line ID field.

- Press     or     to scroll to the Rejection field.

- Press     or     , or the Switch soft key to select Enable from the Rejection field.

- (Optional.) Enter the anonymous call rejection on code in the Reject On Code field.

- (Optional.) Enter the anonymous call rejection off code in the Reject Off Code field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure anonymous call rejection via web user interface at the path Account->Basic.

Advanced Phone Features

This chapter provides operating instructions for the advanced features of the SIP-T26P IP phone. The topics include:

- Busy Lamp Field (BLF)

- Call Recording

- Hot Desking

- Intercom

- Multicast Paging

- Music on Hold

- Automatic Call Distribution (ACD)

- Messages

If you require additional information or assistance with your new phone, contact your system administrator.

Busy Lamp Field (BLF)

You can use the BLF feature to monitor a specific user for status changes on the phone. For example, you can configure a BLF key on the phone for monitoring the status of a friend’s phone (busy or idle). When the friend makes a call, the BLF key (memory key) LED illuminates solid red on your phone to indicate that the friend’s phone is in use and busy. For more BLF key LED indications, refer to LED Instructions on page 5.

You can press a BLF key to dial out the monitored phone number when the monitored phone is idle. You can receive a visual alert and an audio alert when the monitored phone number receives an incoming call. You can also pick up a call directly by pressing the BLF key when the monitored phone number receives an incoming call. For more information, contact your system administrator.

To configure a BLF key via phone user interface:

- Press Menu->Features->DSS Keys->Memory Keys (or Line Keys).

- Select the desired DSS key.

- Press     or     , or the Switch soft key to select BLF from the Type field.

- Press     or     , or the Switch soft key to select the desired line from the Account ID field.

- Enter the phone number or extension you want to monitor in the Value field.

- (Optional.) Enter the pickup code in the Extension field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

- You can also configure the BLF key via web user interface at the path DSSKey.

You can enable the audio alert for BLF pickup feature on the phone. This allows the monitoring phone to play a warning tone, when the monitored phone number receives an incoming call. You can also enable the visual alert for BLF pickup feature on the phone. This allows the LCD screen of the monitoring phone to display the caller ID, when the monitored phone number receives an incoming call.

To enable the visual and audio alert feature via web user interface:

- Click on Features->Call Pickup.

- Select Enabled from the pull-down list of Visual Alert for BLF Pickup.

- Select Enabled from the pull-down list of Audio Alert for BLF Pickup.

- Click Confirm to accept the change.

You can enable the visual and audio alert feature via web user interface only.

Note

When the monitored phone number receives an incoming call, the followings occur on the phone:

- The phone plays a warning tone (if enabled).

- The BLF key LED flashes.

- The caller ID appears on the LCD screen (if enabled).

In the following figure, the phone LCD screen shows an incoming call from 2508 on the monitored line.

If your phone is locked and the type of the keypad lock is set to be Function Keys or All keys, you cannot use the Pickup, Dial, NewCall and Cancel soft keys appear in the figure above until unlocked.

Note

When there is an active call on the IP phone, you can transfer the active call to the monitored phone number directly by pressing the BLF key. The phone transfers the active call differently depending on the transfer mode on DSS key. For more information on performing call transfer, refer to Call Transfer on page 83.

To configure the transfer mode on DSS key via web user interface:

- Click on Features->Transfer.

- Select the desired transfer mode from the pull-down list of Transfer Mode Via Dsskey. Depending on your selection:

- If you select Blind Transfer, press the BLF key to complete the blind transfer to the monitored phone number.

- If you select Attended Transfer, press the BLF key to dial out the monitored phone number and then you can complete the attended or semi-attended transfer.

- If you select New Call, press the BLF key to place a new call to the monitored user.

- Click Confirm to accept the change.

You can configure the transfer mode on DSS key via web user interface only.

Note

Call Recording

You can record calls by pressing a record key on the SIP-T26P IP phone. There are 2 different ways of configuring call recording and they even work differently:

- Record: The phone sends SIP INFO message containing a specific header “Record: on/off” to trigger a recording.

- URL Record: The phone sends HTTP URL request to trigger a recording. Contact your system administrator for the predefined URL.

The record feature is not available on all servers. Contact your system administrator for more information.

Note

To configure a record key via phone user interface:

- Press Menu->Features->DSS Keys->Memory Keys (or Line Keys).

- Select the desired DSS key.

- Press     or     , or the Switch soft key to select Key Event from the Type field.

- Press     or     , or the Switch soft key to select Record from the Key Type field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

To configure a URL record key via phone user interface:

- Press Menu->Features->DSS Keys->Memory Keys (or Line Keys).

- Select the desired DSS key.

- Press     or     , or the Switch soft key to select URL Record from the Type field.

- Enter the URL (e.g., http://10.1.2.224/phonerecording.cgi) in the Value field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure the record or URL record keys via web user interface at the path DSSKey.

The record/URL record keys control the recording and are available:

- During an active call

- When calls are on hold or mute

- During a blind or attended transfer

- During a conference call

- When the phone prompts you to answer an incoming call

The record/ URL record keys are not available:

- When there are no connected calls on your phone

- When you place a new call

To record a call:

- Press a record or a URL record key during a call.

- If the recording starts successfully, the recording icon appears on the LCD screen, and the record or URL record key LED flashes green.

- Press the record or URL record key again to stop recording.

- The recording icon disappears from the LCD screen, and the record or URL record key LED turns off.

Recording status indications you need to know:

| Circumstance | Icons on the LCD screen |
| --- | --- |
| A recording is started | appears on the LCD screen |
| A recording cannot be started | appears for 1 second |
| A recording cannot be stopped | appears for 1 second, then goes back |
| The recording box is full | appears for 1 second |
| The call cannot be recorded | appears for 1 second |

You can listen to the recordings stored on your server system. For example, you can dial an access code to listen to the recordings.

The way in which you listen to the recordings may be different on different servers. Contact your system administrator for more information.

Note

Hot Desking

Hot desking originates from the definition of being the temporary physical occupant of a work station or surface by a particular employee. A primary motivation for hot desking is cost reduction. This feature is regularly used in places where not all the employees are in the office at the same time, or not in the office for very long at all, which means actual personal offices would be often vacant, consuming valuable space and resources.

You can use hot desking on the SIP-T26P IP phone to log out the existing accounts and then log in a new account, that is, many users can share the phone resource in different time. To use this feature, you need to configure a hot desking key in advance.

The hot desking feature is not available on all servers. Contact your system administrator for more information.

Note

To configure a hot desking key via phone user interface:

- Press Menu->Features->DSS Keys->Memory Keys (or Line Keys).

- Select the desired DSS key.

- Press     or     , or the Switch soft key to select Key Event from the Type field.

- Press     or     , or the Switch soft key to select Hot Desking from the Key Type field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure a hot desking key via web user interface at the path DSSKey.

To use hot desking:

- Press the hot desking key when the phone is idle.

The LCD screen prompts “Clear all account config?”.

- Press the OK soft key, registration configurations of all accounts will be cleared immediately.

- The login wizard will be shown as below:

- Enter the login information in each field.

Intercom

Intercom is a useful feature in an office environment to quickly connect with the operator or the secretary. You can press the intercom key to automatically connect with a remote extension for outgoing intercom calls, and the remote extension will automatically answer the incoming intercom calls.

The intercom feature is not available on all servers. Contact your system administrator for more information.

Note

Outgoing Intercom Calls

To configure an intercom key via phone user interface:

- Press Menu->Features->DSS Keys->Memory Keys (or Line Keys).

- Select the desired DSS key.

- Press     or     , or the Switch soft key to select Intercom from the Type field.

- Select the desired line from the Account ID field.

- Enter the remote extension number in the Value field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure an intercom key via web user interface at the path DSSKey.

To place an intercom call:

- Press the intercom key when the phone is idle.

- The phone is automatically connected to the extension number specified in the Value field.

- Press the intercom key again or the Cancel soft key to end the intercom call.

Incoming Intercom Calls

By default, the SIP-T26P IP phone supports to answer an incoming intercom call automatically. The phone automatically plays a warning tone when it receives an incoming intercom call. In addition, you can enable the phone to mute the microphone when it automatically answers an incoming intercom call. You can also enable the phone to automatically answer an incoming intercom call while there is already an active call on the phone, the active call is placed on hold.

Intercom features you need to know:

| Intercom features | Description |
| --- | --- |
| Accept Intercom | Allows you to enable or disable the IP phone to automatically answer an incoming intercom call. |
| Intercom Mute | Allows you to enable or disable the microphone on the IP phone for Intercom calls. |
| Warning Tone | Allows you to enable or disable the IP phone to play a warning tone when the IP phone receives an incoming intercom call. |
| Intercom Barge | Allows you to enable or disable the IP phone to automatically answer an incoming intercom call while there is already an active call on the phone. |

To configure intercom features via phone user interface:

- Press Menu->Features->Intercom.

- Make the desired changes (Press     or     to switch between different intercom features).

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure these specific parameters via web user interface at the path Features->Intercom.

Accept Intercom

You can enable or disable the phone to automatically answer an incoming intercom call. If Accept Intercom is enabled, the phone automatically answers an incoming intercom call. If Accept Intercom is disabled, the phone rejects incoming intercom calls and sends a busy signal to the caller. Accept Intercom is enabled by default.

Your administrator can set a time period delay before the phone automatically answers. Contact your system administrator for more information.

Note

Intercom Mute

You can mute or un-mute the microphone on the phone for intercom calls automatically. If Intercom Mute is enabled, the microphone is muted for intercom calls. If Intercom Mute is disabled, the microphone is un-muted for incoming calls. Intercom Mute is disabled by default.

Warning Tone

You can enable or disable the phone to play a warning tone when receiving an intercom call. If Warning Tone is enabled, the phone plays a warning tone to alert you before answering the intercom call. If Warning Tone is disabled, the phone automatically answers the intercom call without warning. Warning Tone is enabled by default.

Intercom Barge

You can enable or disable the phone to automatically answer an incoming intercom call while there is already an active call on the phone. If Intercom Barge is enabled, the phone automatically answers the intercom call and places the active call on hold. If Intercom Barge is disabled, the phone handles an incoming intercom call like a waiting call. Intercom Barge is disabled by default.

Multicast Paging

You can use multicast paging to quickly and easily forward time sensitive announcements out to people within the multicast group. You can configure a multicast paging key on the phone, which allows you to send a Real Time Transport Protocol (RTP) stream to the pre-configured multicast address(es) without involving SIP signaling. You can configure the phone to allow it to receive an RTP stream from pre-configured multicast listening address(es) without involving SIP signaling. You can specify up to 10 multicast listening addresses.

Sending RTP Stream

To configure a multicast paging key via phone user interface:

- Press Menu->Features->DSS Keys->Memory Keys (or Line Keys).

- Select the desired DSS key.

- Press     or     , or the Switch soft key to select Key Event from the Type field.

- Press     or     , or the Switch soft key to select Multicast Paging from the Key Type field.

- Enter the multicast IP address and port number (e.g., 224.5.6.20:10008) in the Value field.

- The valid multicast IP addresses range from 224.0.0.0 to 239.255.255.255.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure a multicast paging key via web user interface at the path DSSKey.

You can also configure the phone to use a default codec for sending multicast RTP stream via web interface.

To configure a default codec for multicast paging via web user interface:

- Click on Features->General Information.

- Select the desired codec from the pull-down list of Multicast Codec.

- The default codec is G722.

- Click Confirm to accept the change.

If G722 codec is used for multicast paging, the phone LCD screen displays the     icon to indicate that the phone is providing high definition voice.

You can configure a default codec for multicast paging via web user interface only.

Note

To send RTP stream:

- Press the multicast paging key when the phone is idle.

- The phone sends RTP to a preconfigured multicast address (IP: Port). Any phone in the local network then listens to the RTP on the preconfigured multicast address (IP: Port). For both sending and receiving of the multicast RTP there is no SIP signaling involved.

The multicast paging key LED illuminates solid green.

The following figure shows a multicast RTP session on the phone:

- Press the Hold soft key to place the current multicast RTP session on hold.

- Press the Cancel soft key to cancel the multicast RTP session.

Multicast RTP is one way only- from sender to the multicast address(es) (receiver). For outgoing RTP multicasts, all other existing calls on the phone will be placed on hold.

Note

Receiving RTP Stream

You can configure the phone to receive a Real Time Transport Protocol (RTP) stream from the pre-configured multicast address(es) without involving SIP signaling. You can specify up to 10 multicast addresses that the phone listens to on the network.

You can also change the behavior of how the phone handles incoming multicast paging calls by configuring specific parameters via web user interface. The specific parameters are: Paging Barge and Paging Priority Active.

Paging Barge

You can use the paging barge feature to define the priority of the voice call in progress. If the priority of an incoming multicast paging call is lower than that of the active call, then it will be ignored automatically. If Disabled is selected from the pull-down list of Paging Barge, the voice call in progress shall take precedence over all incoming multicast paging calls. The valid values in the Paging Barge field are:

- 1 to 10: Define the priority of an active call, 1 is the highest priority, 10 is the lowest priority.

- Disabled: The voice call in progress shall take precedence over all incoming paging calls.

Paging Priority Active

You can enable or disable this feature to decide how the phone handles the incoming multicast paging calls, when there is already a multicast paging call on the phone. If enabled, the phone will ignore the incoming multicast paging call with a lower priority, otherwise, the phone will play the incoming multicast RTP and place the previous multicast paging call on hold. If disabled, the phone will automatically ignore all incoming multicast paging calls.

To configure multicast listening addresses via web user interface:

- Click on Directory->Multicast IP.

- Select the desired value from the pull-down list of Paging Barge.

- Select the desired value from the pull-down list of Paging Priority Active.

- Enter the multicast IP address(es) and port number (e.g., 224.5.6.20:10008) which the phone listens for incoming RTP multicast in the Listening Address field.

- The valid multicast IP addresses range from 224.0.0.0 to 239.255.255.255.

- Enter the label in the Label field. Label will appear on the LCD screen when receiving the RTP multicast.

- Click Confirm to accept the change.

The priorities of listening addresses are predefined: 1 with the highest priority, 10 with the lowest priority.

Both the multicast paging sender and the receiver will play a warning tone when pressing the multicast paging key.

You can configure multicast listening addresses via web user interface only.

Note

Music on Hold

Music on hold (MoH) is the business practice of playing recorded music to fill the silence that would be heard by the callers who have been placed on hold. To use this feature, you should specify a SIP URI pointing to a Music on Hold Server account, when placing a call on hold, the phone will invite this SIP URI to the Music on Hold Server account. The Music on Hold account automatically answers to the SIP INVITE messages and immediately plays audio from some source located anywhere (LAN, Internet) to the held party. Contact your system administrator for the SIP URI.

To configure music on hold server via web user interface:

- Click on Account.

- Select the desired account from the pull-down list of Account.

- Click on Advanced.

- Enter the SIP URI (e.g., sip:moh@sip.com) in the Music Server URI field.

- Click Confirm to accept the change.

- When you have placed a call on hold, the held party can hear the music.

All involved parties cannot use encrypted RTP (SRTP).

You can configure the music on hold server via web user interface only.

Note

Automatic Call Distribution (ACD)

ACD is often used in offices for customer service, such as call center. The ACD system handles large volumes of incoming calls from callers who have no need to talk to a specific person but who require assistance from any of multiple persons at the earliest opportunity. The ACD feature on the SIP-T26P IP phone allows the ACD system to distribute large volumes of incoming calls to the registered IP phone users. To use this feature, you should configure an ACD key in advance.

Make sure the ACD feature is enabled on your IP phone. For more information on enabling the ACD feature, contact your system administrator.

Note

To configure an ACD key via phone user interface:

- Press Menu->Features->DSS Keys->Memory Keys (or Line Keys).

- Select the desired DSS key.

- Press     or     , or the Switch soft key to select ACD from the Type field.

- Press the Save soft key to accept the change or the Back soft key to cancel.

You can also configure an ACD key via web user interface at the path DSSKey.

To log in the ACD system:

- Press the ACD key when the phone is idle.

The phone LCD screen prompts you the following information:

- User ID: the identity used to log in the queue.

- Password: the password used to log in the queue.

- Press the Login soft key to log in.

Contact your system administrator for the User ID and Password to access the ACD system.

Note

After configuring an ACD key, you can press the ACD key to log in the ACD system. After logging in, you are ready to receive calls from the ACD system. You can press the ACD key to show your current phone status. You can press the Available/Unavailable soft key to change your phone status. The system server monitors your phone status. When you set the phone status to available, the ACD key LED illuminates solid green, and then the server begins distributing calls to your phone. When you set the phone to unavailable, the ACD key LED illuminates flashing green, and then the server temporarily stops distributing calls to your phone. To log out of the ACD system, press the Logout soft key.

It is recommended you configure no more than one ACD key per phone. At any time, at most one ACD key can be in progress on your phone.

Note

Messages

Short Message Service (SMS)

You can send and receive text messages using the SIP-T26P IP phone. New text messages can be indicated both acoustically and visually. When receiving a new text message, the phone will play a warning tone, the phone LCD screen will prompt receiving new text messages with the number of waiting messages (example: 1 New Text Message(s)) and a flashing icon.

You can store text messages in your phone’s Inbox, Sentbox, Outbox or Draftbox. Each of the boxes can store up to 100 text messages. If the number of the text messages in one box is more than 100, the phone will directly delete the oldest text message in the box.

The SMS feature is not available on all servers. Contact your system administrator for more information.

Note

To read a text message:

- Press Menu->Messages->Text Message->Inbox.

- Select the desired message and press the View soft key.

If the phone prompts receiving new text messages, you can also press the View soft key to read the new messages directly.

Note

To send a text message:

- Press Menu->Messages->Text Message->New Message.

- Compose the new text message. You can press the abc soft key to change the input mode.

- Press the Send soft key after completing the content.

- (Optional.) Press     or     , or the Switch soft key to select the desired account from the From field.

- Enter the number to which you want to send the message in the To field.

- Press the Send soft key to send the message or the Back soft key to cancel.

You can also send a text message via web user interface at the path Features->SMS.

To reply a text message:

- Press Menu->Messages->Text Message->Inbox.

- Select the desired message and press the Reply soft key.

- Compose the new text message. You can press the abc soft key to change the input mode.

- Press the Send soft key after completing the content.

- Check the From and To fields, and then press the Send soft key.

To delete a text message:

- Press Menu->Messages->Text Message->Inbox (Sentbox, Outbox or Draftbox).

- Select the desired message and then press the Delete soft key.

- Select Delete to delete the desired message.

The LCD screen prompts “Delete Message?”.

- Press the OK soft key to delete this message or the Cancel soft key to cancel.

You can also delete all text messages by pressing the Delete soft key and then select Del All. For more information, refer to the above steps.

You can also delete a specific message after retrieving by pressing the Delete soft key.

Note

Voice Mail

You can leave voice mails for someone else on the SIP-T26P IP phone. You can also listen to the voice mails stored in a centralized location. When receiving a new voice mail, the phone will play a warning tone and the MESSAGE key LED will illuminate and the phone LCD screen will appear a flashing icon.

The voice mail feature is not available on all servers. Contact your system administrator for more information.

Note

To leave a voice mail:

You can leave a voice mail for someone else when he/she is busy or inconvenient to answer the call. Follow the voice prompt from the system server to leave the voice mail, and then hang up after completing the voice mail.

To configure voice mail access codes via phone user interface:

- Press Menu->Messages->Voice Mail->Set Voice Mail.

- Press     or     to highlight the account which you want to set, press the 123 soft key to select the proper input mode and then enter the voice mail access code.

- Press the Save soft key to accept the change or the Back soft key to cancel.

- Voice mail access code must be predefined on the system server. Contact your system administrator for the more information.

Note

To listen to the voice mails:

- When the phone user interface prompts receiving new voice mails, press  or  or the Connect soft key to dial out the access voice mail access code.

- Follow the voice prompt to listen to the voice mails.

Before listening to the voice mails, make sure the voice mail access code has been configured.

When all new voice mails are retrieved, the MESSAGE key LED will turn off.

Note

To view the voice mail via phone user interface:

- Press Menu->Messages->Voice Mail->View Voice Mail.

- The phone LCD screen displays the amount of the voice mails that includes new or old voice mails.

- Select an account and then press the Connect soft key to listen to the voice mails.

Message Waiting Indicator (MWI)

The SIP-T26P IP phone supports MWI feature when receiving a new voice message. If someone leaves you a voice mail, you will receive a message waiting indicator. MWI will be indicated in three ways: a warning tone, the MESSAGE key LED illuminates solid green and an indicator message (including a voice mail icon) appears on the LCD screen. This is cleared when you retrieve all voice mails or delete them.

For some particular servers, the MWI service is unsolicited. So the SIP-T26P IP phone just needs to handle the MWI messages sent from the server. But for some servers, the MWI service is solicited. In this case, the SIP-T26P IP phone must enable MWI Subscription for MWI messages.

MWI service is not available on all servers. Contact your system administrator for more information.

Note

The MWI Subscription parameters you need to know:

| Option | Description |
| --- | --- |
| Subscribe for MWI | Enable or disable a subscription for MWI service. |
| MWI Subscription Period | Period of MWI subscription. The IP phone re-sends a MWI subscription before expiring. |
| Subscribe MWI to Voice Mail | Enable or disable a subscription to the voice mail number for MWI service. To use this feature, you should configure the subscribe for MWI feature and the voice mail number in advance. |

Whether the phone subscribes the MWI messages to the account or the voice mail number depends on the server. Contact your system administrator for more information.

Note

To configure subscribe for MWI via web user interface:

- Click on Account.

- Select the desired account from the pull-down list of Account.

- Click on Advanced.

- Select Enabled from the pull-down list of Subscribe for MWI field.

- Enter the period time in the MWI Subscription Period (Seconds) field.

- Click Confirm to accept the change.

To enable the Subscribe MWI to Voice Mail feature via web user interface:

- Click on Account.

- Select the desired account from the pull-down list of Account.

- Click on Advanced.

- Select Enabled from the pull-down list of Subscribe MWI To Voice Mail.

- Enter the desired voice mail number in the Voice Mail field.

- Click Confirm to accept the change.

- The IP phone will subscribe to the voice mail number for MWI service using Subscribe MWI to WM feature.

You can configure the MWI subscription via web user interface only.

Note

Troubleshooting

This chapter provides general troubleshooting information to help to solve the problems you might encounter when using your SIP-T26P IP phone.

If you require additional information or assistance with your new phone, contact your system administrator.

Why is the phone LCD screen blank?

- Ensure that the phone is properly plugged into a functional AC outlet.

- Ensure that the phone isn’t plugged into a plug controlled by a switch that is off.

- If the phone is plugged into a power strip, try plugging it directly into a wall outlet instead.

- If the phone is powered from PoE, ensure you use a PoE compliant switch or hub.

Why does the phone display “Network Unavailable”?

- Ensure that the Ethernet cable is plugged into the Internet port o the phone and the Ethernet cable is not loose.

- Ensure that the switch or hub in your network is operational.

- Contact your system administrator for more information.

Why does the phone display “No Service”?

The phone LCD screen prompts “No Service” message when no SIP account registers successfully.

Why doesn’t the phone display time and date correctly?

Check if you have configured the phone to obtain the time and date from the SNTP server automatically. If the phone fails to connect the SNTP server, configure the time and date manually.

How do I find the basic information of the phone?

Press the OK key when the phone is idle to check the basic information of the phone, such as the IP address and firmware version. For more the basic information, refer to Phone Status on page 13.

How to obtain the MAC address of a phone when the phone is not powered on?

There are three ways to obtain the MAC address of a phone:

- You can ask your supplier for shipping information sheet which includes MAC addresses according to the corresponding PO.

- You can find the MAC address in the label of carton box.

- You can also find the MAC address from the phone’s bar code on the back of the phone.

Why can’t I get a dial tone?

- Check for any loose connections and that the phone has been installed properly. For the installation instructions, refer to Phone Installation on page 11.

- Switch between the Handset, Headset (if present) or Hands-Free Speakerphone to check whether the dial tone is present for one of the audio modes.

If the dial tone exists on another audio mode, connect a different handset or headset to isolate the problem.

Why doesn’t the phone ring?

Check that the ringer volume on the phone. To adjust the ringer volume setting, press the Volume key when the phone is idle. For more information, refer to Volume on page 25.

Why can’t I receive calls?

- Check that the SIP registration with your system administrator.

- Check that the DND (Do Not Disturb) mode is turned off on the phone. Refer to Do Not Disturb (DND) on page 75.

- Check that the call forward feature is disabled on the phone. Refer to Call Forward on page 77.

- Check whether the caller number is stored in the blacklist directory. Refer to Blacklist on page 39.

Why is my handset not working?

Check that the handset cord is fully connected to both the handset jack on the phone and handset. Refer to Phone Installation on page 11.

Why is my headset not working?

- Check that the headset cord is fully connected to the headset jack on the phone. Refer to Phone Installation on page 11.

- Check that the headset mode is activated. Refer to Headset Mode Activation/Deactivation on page 45.

- Check that the headset volume is adjusted to an appropriate level. Refer to Volume on page 25.

What will happen if I connect both PoE cable and power adapter? Which has the priority?

The phones manufactured before February 2010 use the power adapter preferentially, otherwise the phones use PoE preferentially.

What is the difference between user name, register name and display name?

Both user name and register name are defined by the server. A user name is used to identify the account while a register name matched with a password is used for authentication if the server requires. Display name is the caller ID that will be displayed on the callee’s phone LCD screen. Server configurations may override local configurations.

Why does the phone play a tone when there is a call on hold? How to disable it?

When there is a call is on hold, the phone will play a hold tone every 30 seconds. The call hold tone feature is enabled by default. You can disable it or change the interval to play a hold tone via web user interface only.

To configure the call hold tone and call hold tone delay via web user interface:

- Click on Features->General Information.

- Select the Enabled or Disabled from the pull-down list of Play Hold Tone.

- Enter the desired time (in seconds) in the Play Hold Tone Delay field.

- Click Confirm to accept the change.

Why can’t I send a SMS to any other phone?

The SMS feature depends on support from a SIP server. Contact your system administrator for more information.

How to change the user password?

To change the user password via web user interface:

- Click on Security->Password.

- Select user from the pull-down list of User Type.

- Enter the new user password in the New Password field and Confirm Password field.

- Click Confirm to accept the change.

If logging in the web user interface of the phone with the user credential, you need to enter the current user password in the Old Password field.

You can change the user password via web user interface only.

Note

How to make a call using SRTP?

You can enable SRTP to encrypt the audio stream(s) of phone calls. The parties participating in the call should enable SRTP. You can enable SRTP on a per-line basis.

To enable SRTP via web user interface:

- Click on Account.

- Select the desired account from the pull-down list of Account.

- Click on Advanced.

- Select the desired value from the pull-down list of RTP Encryption (SRTP).

- Click Confirm to accept the change.

The SRTP feature is not available on all servers. Contact your system administrator for more information.

You can configure the SRTP via web user interface only.

Note

How to reboot the phone?

To reboot the phone via web user interface:

- Click on Settings->Upgrade.

- Click Reboot to reboot the phone.

You can also long press the X key to reboot the phone directly. Any reboot of your phone may take a few minutes.

Note

How to export PCAP trace?

We may need you to provide a PCAP trace to help analyze your problem.

To export a PCAP trace via web user interface:

- Click on Settings->Upgrade.

- Click Start to begin recording signal traffic.

- Recreate the error to be documented in the trace.

- Click Stop to end recording.

- Click Export to open file download window, and then save the file to your local system.

How to export system log?

We may need you to provide a system log to help analyze your problem.

To export a system log via web user interface:

- Click on Settings->Configuration.

- Select 6 from the pull-down list of System Log Level.

- Click Confirm to accept the change.

- The web user interface prompts “Do you want to restart your machine?”. The configuration will take effect after reboot.

- Click OK to reboot the phone.

- After rebooting the phone, the system log level is set as 6, the debug level.

- Mark the Local radio box in the Export System Log field.

- Click Export to open file download window, and then save the file to your local system.

- You can also export the system log to a syslog server. Contact your system administrator for more information.

How to export/import phone configurations?

We may need you to provide the phone configurations to help analyze your problem. In some instance, you may need to import configurations to the phone.

To export the phone configurations via web user interface:

- Click on Settings->Configuration.

- Click Export to open file download window, and then save the file to your local system.

To import the phone configurations via web user interface:

- Click on Settings->Configuration.

- Click Browse to locate a configuration file from your local system.

- Click Import to import the configuration file.

The file format of configuration file must be “.bin”.

Note

How to upgrade firmware?

To upgrade firmware via web user interface:

            - Click on Settings->Upgrade.

            - Click Browse to locate the firmware from your local system.

            - Click Upgrade to upgrade the firmware.

- The web user interface prompts “Firmware of the SIP Phone will be updated. It will take 5 minutes to complete. Please don't power off!”.

            - Click OK to confirm upgrading.

How to reset the phone?

Reset the phone when other troubleshooting suggestions do not correct the problem. You need to note that all customized settings will be overwritten after resetting. We recommend asking your system administrator for advice before resetting the phone.

To reset the phone via phone user interface:

- Press Menu->Settings->Advanced Settings (password: admin).

- Press     or     to scroll to Reset to Factory, and then press the Enter soft key.

- The LCD screen prompts “Reset to factory settings?”.

- Press the OK soft key.

- The LCD screen prompts “Resetting…Please Waiting!”.

- The LCD screen prompts “Welcom Initializing…Please wait”.

- The phone will be reset to factory sucessfully after startup.

Reset of the phone may take a few minutes. Do not power off until the phone starts up successfully.

Note

Regulatory Notices

Service Agreements

Contact your Yealink Authorized Reseller for information about service agreements applicable to your product.

Limitations of Liability

TO THE FULL EXTENT ALLOWED BY LAW, YEALINK EXCLUDES FOR ITSELF AND ITS SUPPLIERS ANY LIABILITY, WHETHER BASED IN CONTRACT OR TORT (INCLUDING NEGLIGENCE), FOR INCIDENTAL, CONSEQUENTIAL, INDIRECT, SPECIAL, OR PUNITIVE DAMAGES OF ANY KIND, OR FOR LOSS OF REVENUE OR PROFITS, LOSS OF BUSINESS, LOSS OF INFORMATION OR DATA, OR OTHER FINANCIAL LOSS ARISING OUT OF OR IN CONNECTION WITH THE SALE, INSTALLATION, MAINTENANCE, USE, PERFORMANCE, FAILURE, OR INTERRUPTION OF ITS PRODUCTS, EVEN IF YEALINK OR ITS AUTHORIZED RESELLER HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH AMAGES, AND LIMITS ITS LIABILITY TO REPAIR, REPLACEMENT, OR REFUND OF THE PURCHASE RICE PAID, AT YEALINK’S OPTION. THIS DISCLAIMER OF LIABILITY FOR DAMAGES WILL NOT BE AFFECTED IF ANY REMEDY PROVIDED HEREIN SHALL FAIL OF ITS ESSENTIAL PURPOSE.

Safety Instructions

Save these instructions. Read these safety instructions before use!

The following basic safety precautions should always be followed to reduce risk of fire, electrical shock, and other personal injury.

General Requirements

Before you install and use the device, read the safety instructions carefully and observe the situation during operation.

During the process of storage, transportation, and operation, please always keep the device dry and clean.

During the process of storage, transportation, and operation, please avoid collision and crash of the device.

Please attempt not to dismantle the device by yourself. In case of any discrepancy, please contact the appointed maintenance center for repair.

Without prior written consent, no organization or individual is permitted to make any change to the structure or the safety design of the device. Yealink is under no circumstance liable to consequences or legal issues caused by such changes.

Please refer to the relevant laws and statutes while using the device. Legal rights of others should be respected as well.

Environmental Requirements

Place the device at a well-ventilated place. Do not expose the device under direct sunlight.

Keep the device dry and free of dusts.

Place the device on a stable and level platform.

Please place no heavy objects on the device in case of damage and deformation caused by the heavy load.

Keep at least 10 cm between the device and the closest object for heat dissipation.

Do not place the device on or near any inflammable or fire-vulnerable object, such as rubber-made materials.

Keep the device away from any heat source or bare fire, such as a candle or an electric heater.

Keep the device away from any household appliance with strong magnetic field or electromagnetic field, such as a microwave oven or a refrigerator.

Operating Requirements

Do not let a child operate the device without guidance.

Do not let a child play with the device or any accessory in case of accidental swallowing.

Please use the accessories provided or authorized by the manufacturer only.

The power supply of the device shall meet the requirements of the input voltage of the device. Please use the provided surge protection power socket only.

Before plugging or unplugging any cable, make sure that your hands are completely dry.

Do not spill liquid of any kind on the product or use the equipment near water, for example, near a bathtub, washbowl, kitchen sink, wet basement or near a swimming pool.

Do not tread on, pull, or over-bend any cable in case of malfunction of the device.

During a thunderstorm, stop using the device and disconnect it from the power supply. Unplug the power plug and the Asymmetric Digital Subscriber Line (ADSL) twisted pair (the radio frequency cable) to avoid lightning strike.

If the device is left unused for a rather long time, disconnect it from the power supply and unplug the power plug.

When there is smoke emitted from the device, or some abnormal noise or smell, disconnect the device from the power supply, and unplug the power plug immediately. Contact the specified maintenance center for repair.

Do not insert any object into equipment slots that is not part of the product or auxiliary product.

Before connecting a cable, connect the grounding cable of the device first. Do not disconnect the grounding cable until you disconnect all other cables.

Cleaning Requirements

Before cleaning the device, stop using it and disconnect it from the power supply.

Use a piece of soft, dry and anti-static cloth to clean the device.

Keep the power plug clean and dry. Using a dirty or wet power plug may lead to electric shock or other perils.

Appendix A - Time Zones

| Time Zone | Time Zone Name |
| --- | --- |
| −11:00 | Samoa |
| −10:00 | United States-Hawaii-Aleutian |
| −10:00 | United States-Alaska-Aleutian |
| −09:00 | United States-Alaska Time |
| −08:00 | Canada(Vancouver, Whitehorse) |
| −08:00 | Mexico(Tijuana, Mexicali) |
| −08:00 | United States-Pacific Time |
| −07:00 | Canada(Edmonton, Calgary) |
| −07:00 | Mexico(Mazatlan, Chihuahua) |
| −07:00 | United States-Mountain Time |
| −07:00 | United States-MST no DST |
| −06:00 | Canada-Manitoba(Winnipeg) |
| −06:00 | Chile(Easter Islands) |
| −06:00 | Mexico(Mexico City, Acapulco) |
| −06:00 | United States-Central Time |
| −05:00 | Bahamas(Nassau) |
| −05:00 | Canada(Montreal, Ottawa, Quebec) |
| −05:00 | Cuba(Havana) |
| −05:00 | United States-Eastern Time |
| −04:30 | Venezuela(Caracas) |
| −04:00 | Canada(Halifax, Saint John) |
| −04:00 | Chile(Santiago) |
| −04:00 | Paraguay(Asuncion) |
| −04:00 | United Kingdom-Bermuda(Bermuda) |
| −04:00 | United Kingdom(Falkland Islands) |
| −04:00 | Trinidad&Tobago |
| −03:30 | Canada- New Foundland(St.Johns) |
| −03:00 | Denmark-Greenland(Nuuk) |
| −03:00 | Argentina(Buenos Aires) |
| −03:00 | Brazil(no DST) |
| −03:00 | Brazil(DST) |
| −02:00 | Brazil(no DST) |
| −01:00 | Portugal(Azores) |
| 0 | GMT |
| 0 | Greenland |
| 0 | Denmark-Faroe Islands(Torshavn) |
| 0 | Ireland(Dublin) |
| 0 | Portugal(Lisboa, Porto, Funchal) |
| 0 | Spain-Canary Islands(Las Palmas) |
| 0 | United Kingdom(London) |
| 0 | Morocco |
| +01:00 | Albania(Tirane) |
| +01:00 | Austria(Vienna) |
| +01:00 | Belgium(Brussels) |
| +01:00 | Caicos |
| +01:00 | Chad |
| +01:00 | Croatia(Zagreb) |
| +01:00 | Czech Republic(Prague) |
| +01:00 | Denmark(Kopenhagen) |
| +01:00 | France(Paris) |
| +01:00 | Germany(Berlin) |
| +01:00 | Hungary(Budapest) |
| +01:00 | Italy(Rome) |
| +01:00 | Luxembourg(Luxembourg) |
| +01:00 | Macedonia(Skopje) |
| +01:00 | Netherlands(Amsterdam) |
| +01:00 | Namibia(Windhoek) |
| +02:00 | Estonia(Tallinn) |
| +02:00 | Finland(Helsinki) |
| +02:00 | Gaza Strip(Gaza) |
| +02:00 | Greece(Athens) |
| +02:00 | Israel(Tel Aviv) |
| +02:00 | Jordan(Amman) |
| +02:00 | Latvia(Riga) |
| +02:00 | Lebanon(Beirut) |
| +02:00 | Moldova(Kishinev) |
| +02:00 | Russia(Kaliningrad) |
| +02:00 | Romania(Bucharest) |
| +02:00 | Syria(Damascus) |
| +02:00 | Turkey(Ankara) |
| +02:00 | Ukraine(Kyiv, Odessa) |
| +02:00 | Syria(Damascus) |
| +03:00 | East Africa Time |
| +03:00 | Iraq(Baghdad) |
| +03:00 | Russia(Moscow) |
| +03:30 | Iran(Teheran) |
| +04:00 | Armenia(Yerevan) |
| +04:00 | Azerbaijan(Baku) |
| +04:00 | Georgia(Tbilisi) |
| +04:00 | Kazakhstan(Aktau) |
| +04:00 | Russia(Samara) |
| +04:30 | Afghanistan |
| +05:00 | Kazakhstan(Aqtobe) |
| +05:00 | Kyrgyzstan(Bishkek) |
| +05:00 | Pakistan(Islamabad) |
| +05:00 | Russia(Chelyabinsk) |
| +05:30 | India(Calcutta) |
| +06:00 | Kazakhstan(Astana, Almaty) |
| +06:00 | Russia(Novosibirsk, Omsk) |
| +07:00 | Russia(Krasnoyarsk) |
| +07:00 | Thailand(Bangkok) |
| +08:00 | China(Beijing) |
| +08:00 | Singapore(Singapore) |
| +08:00 | Australia(Perth) |
| +09:00 | Korea(Seoul) |
| +09:00 | Japan(Tokyo) |
| +09:30 | Australia(Adelaide) |
| +09:30 | Australia(Darwin) |
| +10:00 | Australia(Sydney, Melbourne, Canberra) |
| +10:00 | Australia(Brisbane) |
| +10:00 | Australia(Hobart) |
| +10:00 | Russia(Vladivostok) |
| +10:30 | Australia(Lord Howe Islands) |
| +11:00 | New Caledonia(Noumea) |
| +12:00 | New Zealand(Wellington, Auckland) |
| +12:45 | New Zealand(Chatham Islands) |
| +13:00 | Tonga(Nukualofa) |

Index

A

About This Guide   v

Account Management   55

Account Registration   55

Adding Contacts   30

Adding Groups 30

Administrator password   21

Anonymous Call   93

Anonymous Call Rejection   94

Answering Calls   67

Area Code   60

Attaching Stand   11

Audio Settings   25

Auto Answer   69

Auto Redial   71

Automatic Call Distribution (ACD)   108

B

Backlight   17

Basic Network Settings   14

Blacklist   39

Block Out   61

Busy Lamp Field (BLF)   95

C

Call Completion   72

Call Forward   77

Call History Management   42

Call Hold/Resume   74

Call Mute   73

Call Park   88

Call Pickup   89

Call Recording   98

Call Return   69

Call Transfer   83

Call Waiting   84

CE Mark Warning   ii

Conference   84

Contact Management   28

Copyright   i

Customizing Your Phone   17

D

Declaration of Conformity   ii

Deleting Contacts   34

Deleting Groups   31

Dial-now   58

Dial Plan   56

Directory   28

Do Not Disturb (DND)   75

Documentations   8

DSS Keys   46

Dual Headset   46

E

Editing Contacts   33

Editing Groups   30

Emergency Number   62

Ending Calls   68

G

General Settings   17

Getting Started   9

H

Hardware Component Instructions   1

Headset Mode Activation/Deactivation   45

Headset Prior   45

Headset Use   44

Hot Desking   100

Hot Line   64

I

Icon Instructions   3

Idle screen   16

Importing/Exporting Contact lists   36

In This Guide   v

Incoming intercom call features   102

Intercom   100

K

Key as Send   22

Keypad Lock   22

L

Language   18

LED Instructions   5

Limitation of Liability   127

Line Keys   54

Live Dialpad   63

Local Conference   85

Local Directory   29

Logo Customization   44

M

Memory Keys   47

Messages   110

Message Waiting Indicator   114

Multicast Paging   104

Music on Hold   107

N

Network Conference   86

Network Connection   11

No answer Forward   77

O

Outgoing intercom Calls   102

Overview   1

P

Packaging Contents   9

Phone Initialization   13

Phone Installation   11

Phone Status   13

Phone User Interface   6

Placing Calls   65

Placing Calls to Contacts   35

Programmable Keys   54

R

Receiving RTP Stream   106

Recent Call In Dialing   69

Redialing Numbers   69

Registration   15

Regulatory Notices   127

Remote Phone Book   39

Replace Rule   57

Ring Tones   26

S

Safety Instructions   127

Search Source List in Dialing   36

Searching for Contacts   36

Semi-attended Transfer   83

Sending RTP Stream   104

Service Agreements   127

Short Message Service (SMS)   110

System Customizations   44

T

Time and Date   19

Time Zones   129

Troubleshooting   117

V

Voice Mail   112

Volume   25

W

Warranty   ii

Web User Interface   6

WEEE Warning   iii